| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0033 |
| --- | --- |

| Title: | SCCE3: Summary report of CE on palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Wen Huang Patrice Onno Rajan Joshi Robert Cohen Xiaoyu Xiu Zhan Ma | Emails: | yuwen.huang@mediatek.com patrice.onno@crf.canon.fr rajanj@qti.qualcomm.com cohen@merl.com xiaoyu.xiu@interdigital.com Zhan.Ma@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the Screen Content Core Experiment 3 (SCCE3) on palette mode. There were 30 tests conducted in SCCE3. Evidences were provided that palette mode can significantly improve screen content coding efficiency (more than 20% and 26% BD-rate savings for 1080p text and graphics with motion under FF-IBC AI condition and 2CTU-IBC AI condition, respectively). In general, the palette mode complexity is affordable.

# Introduction

The palette mode was used in all the responses to the Screen Content Call for Proposals (CfP) at the 17th JCT-VC meeting. Most of the proposed palette techniques correspond to variations of previous contributions on palette mode submitted at the 16th JCT-VC meeting: JCTVC-P0108 [1] and JCTVC-P0303 [2]. Through the descriptions of the responses to the CfP and the related contributions submitted at the 17th JCT-VC meeting, these variants can be roughly categorized as follows: palette table coding, color index map coding, and single color modes. Then, the HEVC Screen Content Core Experiment 3 (SCCE3) [3] was established to further investigate each of these different technical parts of the palette mode and evaluate their individual performance according to the screen content test conditions [4]. Evaluation of combinations of some individual parts of the palette mode was also included into SCCE3. This contribution summarizes the test conditions and test results. Comparisons are also provided, and conclusions are given.

# Test conditions

Test C.1 and Test C.3 are built on top of SCM-1.0 (i.e., no palette).

The rest tests are to be built on top of a palette design: SCM-1.0 + JCTVC-Q0094 (i.e., a palette originating from JCTVC-P0303) + bug-fixes of JCTVC-Q0047 (i.e., bug-fixes of the JCTVC-Q0094 palette). In the rest of this document, for the sake of convenience, we name this palette design as Q0094BF palette.

All methods were tested under the following two test conditions that both enable intra block copy (IBC). One condition enables IBC with full frame search (FF-IBC), while the other enables IBC with 2-CTU search (2CTU-IBC). Each test condition has two mandatory anchors.

## Test condition 1: FF-IBC

- Anchor 1-1: SCM-1.0 common test condition with FF-IBC and Q0094BF palette

- Anchor 1-2: SCM-1.0 common test condition with FF-IBC

## Test condition 2: 2CTU-IBC

- Anchor 2-1: SCM-1.0 common test condition with 2CTU-IBC and Q0094BF palette

- Anchor 2-2: SCM-1.0 common test condition with 2CTU-IBC

# Test descriptions and results

## Q0094BF palette

- Results:

Left: BD-rates of Q0094BF palette (Anchor 1-1) versus palette-off under FF-IBC (Anchor 1-2)

Right: BD-rates of Q0094BF palette (Anchor 2-1) versus palette-off under 2CTU-IBC (Anchor 2-2)

## Category A – Palette table coding

### Test A.1 – Triplet palette representation and coding

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Nokia

- Description:

This test evaluates performance of the triplet palette representation technique. The information of palette composes component-wise palette and triplet palette. The palettes are predictive encoded by a palette copy mode or a palette modification mode. In the palette copy mode, the palette will be the same as the predictor. In the palette modification mode, the palette will be derived from the predictor according to the signaled modification information. The palette predictor will be updated according to modification and fully propagated to the next CU.

- Results:

Withdrawn.

### Test A.2 – Palette coding and palette predictor updates

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083, JCTVC-R0119

- Crosschecker: Canon, JCTVC-R0091

- Description:

This test evaluates performance of the palette predictive coding technique. The palette is predictive encoded by a palette copy mode and a palette modification mode. In the palette copy mode, the palette will be the same as the predictor. In the palette modification mode, the palette will be derived from the predictor according to the signaled modification information. The palette predictor will be updated according to modification and fully propagated to the next palette coded CU.

- Results:

Left: BD-rates of Test A.2 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.2 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

- Supplemental (non-CE) results:

Test A.2 includes a palette copy mode and a palette modification mode with full palette propagation. When only the palette copy mode is enabled, the results are as follows.

Left: BD-rates of Test A.2 supplemental versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.2 supplemental versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test A.3 – Palette stuffing

- Proponent: Canon, JCTVC-Q0063, JCTVC-R0082

- Crosschecker: InterDigital, JCTVC-R0205

- Description:

It is proposed to combine the last coded palette and the last predictor palette by stuffing unused elements of the last predictor at the end of the last coded palette, using the resulting palette as predictor for the next palette-coded CU, and signalling when there are no predicted elements. This predictor and flags can then be used to determine the current CU palette.

- Results:

Left: BD-rates of Test A.3 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.3 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

Runtime percentages are inaccurate.

### Test A.4 – Color table processing

- Proponent: Huawei, JCTVC-Q0034/JCTVC-Q0176, JCTVC-R0142

- Crosschecker: MediaTek, JCTVC-R0297

- Description:

It is proposed to derive color table using packed (interleaved) components together. It includes color table merge from neighboring blocks as implicit signaling method. It further contains the inter table color sharing and intra table color DPCM as explicit signaling mechanism.

- Results:

Left: BD-rates of Test A.4 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.4 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test A.5 – Palette table prediction

- Proponent: InterDigital, JCTVC-Q0037, JCTVC-R0166

- Crosschecker: Canon, JCTVC-R0092

- Description:

A representative color dictionary is used to store the representative colors from previously coded palette tables in previous CUs. When coding the current CU, the representative color dictionary is used to predict the representative colors in the current palette table. After the current CU is coded, the representative color dictionary is updated by: 1) adding the representative colors in the current CU into the dictionary, and 2) removing old representative colors from the dictionary adaptively. Additionally, a palette table skip mode is applied to directly re-use the representative color dictionary for the current CU. The signaling of the position of the last predictable palette table entry will be tested.

- Results:

Left: BD-rates of Test A.5 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.5 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

Additional (CE) results:

In Test A.5, the palette table skip mode (i.e., palette share or palette copy) is disabled.

Left: BD-rates of Test A.5 without palette share versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.5 without palette share versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test A.6 – Palette table generation

- Proponent: InterDigital, JCTVC-Q0037, JCTVC-R0167

- Crosschecker: Huawei, JCTVC-R0250

- Description:

At the encoder side, an improved palette table generation method is applied to generate the representative colors in the palette table. The proposed palette table generation consists of three main steps: 1) representative color initialization, 2) representative color refinement, and 3) RDO based palette table entry selection. The histogram based method is used to calculate an initial set of M representative colors in the current CU. In the second step, for each quantization zone, the refined representative color is calculated as the weighted average of all the colors within that quantization zone.

- Results:

Left: BD-rates of Test A.6 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.6 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test A.7 – Encoder-improved palette generation

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047, JCTVC-R0064

- Crosschecker: InterDigital, JCTVC-R0172

- Description:

In this test, improvement of palette generation is investigated. The main steps are the use of histogram of the pixel values in a CU to derive the palette, use of rate-distortion checking to prune the palette (decide whether pixels should be coded as escape pixel or as index), use of centroids in deriving the final palette. Results for subsets of these steps will also be provided.

- Results:

Left: BD-rates of Test A.7 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.7 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test A.8 – Improvements on palette prediction vector signaling

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0225, JCTVC-R0063

- Crosschecker: InterDigital, JCTVC-R0207

- Description:

In this test, methods of coding the binary predictor vector are investigated. A hierarchical binary tree data structure is used to code the binary palette predictor vector. Use of contexts for encoding the higher levels of the tree, starting and stopping the coding of the binary tree at various levels in the hierarchy will be evaluated. Sending the number of ones in the binary predictor vector or the total number of palette entries before sending the bits will also be tested. Additionally, these methods will be tested on top of palette stuffing.

- Results:

Left: BD-rates of Test A.8 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.8 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test A.9 – Long-term palette prediction

- Proponent: Nokia, JCTVC-Q0071, JCTVC-R0047

- Crosschecker: Canon, JCTVC-R0093

- Description:

This test evaluates performance of the long-term palette prediction technique. The proposed method signals a long-term palette at the slice header to be used in the coding of the palette of a CU that is coded with palette mode within that slice. Long term palette information signaled at the slice header consists of the size and entries of the long term palette. At CU level, while coding the palette of a CU, for each palette entry that is not predicted from a previous slice, one bit is signaled to indicate whether this entry is copied from the long term palette or explicitly signaled. If the bit indicates the entry is copied from the long term palette, then the index of the entry within the long term palette is signaled.

- Results:

Left: BD-rates of Test A.9 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test A.9 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

## Category B – Color index map coding

### Test B.1 – Line-based color index map coding

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083, JCTVC-R0121

- Crosschecker: Huawei, JCTVC-R0251

- Description:

The color indices of all samples in the CU are coded line by line. For each sample line, an “index line mode” is signaled to indicate the coding mode. The index line mode includes vertical mode, horizontal mode, and normal mode:

- If the vertical mode is selected, the color indices of the current line are copied from the above line.

- Else if the horizontal mode is selected, one color index is signaled and the color indices of all rest samples in the current line are set to the signaled color index.

- Else (the normal mode is selected), the color indices of the line are signaled sample by sample. For each sample, a color prediction index is signaled to specify how to predict the color index, and a color index of the sample is transmitted directly when the color prediction index indicates no prediction.

The index coding in normal mode includes the methods described in Test B.2, Test B3, and Test B.4 in line-based color index map coding, as shown in JCTVC-Q0083, while Test B.2, Test B3, and Test B4 are in run-based color index map coding.

- Results:

Left: BD-rates of Test B.1 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.1 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.2 – Transition copy mode

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083, JCTVC-R0120

- Crosschecker: Huawei, JCTVC-R0252

- Description:

Transition copy (TC) candidates are considered as one of the prediction modes in the color index map coding. In each CU, a TC table inherits the TC table of the previous CU. The TC table is maintained as the samples in the CU are coded. The input of the TC table is called a pilot color index and is set to the color index of the left sample, and the outputs of the TC table are TC color indices, where the indices are likely to occur right after an occurrence of the pilot color index. The color indices in TC table are updated according to coded samples.

- Results:

Left: BD-rates of Test B.2 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.2 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.3 – Candidate-based color index map coding

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083

- Crosschecker: Qualcomm

- Description:

A candidate list which includes copy-above, copy-left, copy-above-left, copy-above-right, copy-above-above (indices from neighboring samples), and transition-copy, is first generated. The redundant candidates in the list are removed. A candidate index is signaled to specify which candidate is used. If a new color index is signaled, the redundant index values from predictors are removed.

- Results:

Withdrawn

### Test B.4 – Adaptive color index map scan

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083, JCTVC-R0057

- Crosschecker: Microsoft, JCTVC-R0298

- Description:

This test evaluates performance of the adaptive color index map scan technique. For a palette coded CU, if the proposed tool is signaled to be enabled, the sample position axes for the color index map will be swapped before being encoded.

- Results:

Left: BD-rates of Test B.4 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.4 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

- Supplemental (non-CE) results:

The test sequences are all clockwise rotated by 90 degrees.

Left: BD-rates of Test B.4 supplemental versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.4 supplemental versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.5 – Palette index coding

- Proponent: Canon, JCTVC-Q0064, JCTVC-R0083

- Crosschecker: Qualcomm, JCTVC-R0069

- Description:

It is proposed to use two code lengths depending on the index, as well as removing impossible values from the set instead of using FLC as in AHG10 software.

- Results:

Left: BD-rates of Test B.5 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.5 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.6 – Transition copy mode

- Proponent: Canon, JCTVC-Q0065, JCTVC-R0084

- Crosschecker: ITRI, JCTVC-R0275

- Description:

A new index prediction method is proposed. It uses the position of so-called transitions to determine a position from which the prediction copies indexes. This test describes a simpler prediction method that encodes a run of values being the expected transition.

- Results:

Left: BD-rates of Test B.6 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.6 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

Runtime percentages are inaccurate.

### Test B.7 – Run coding

- Proponent: Canon, JCTVC-Q0066, JCTVC-R0085

- Crosschecker: InterDigital, JCTVC-R0210

- Description:

It is proposed to limit the number of runs coded after a palette index for the run mode. A syntax element is transmitted for each palette CU to determine when not to decode the run.

- Results:

Left: BD-rates of Test B.7 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.7 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

Runtime percentages are inaccurate.

### Test B.8 – Color index map processing

- Proponent: Huawei, JCTVC-Q0034/JCTVC-Q0176, JCTVC-R0143

- Crosschecker: Qualcomm, JCTVC-R0302

- Description:

This test evaluates the color index map coding using 1D and 2D string match based index compression.

- Results:

Left: BD-rates of Test B.8 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.8 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.9 – BWT-based index grouping

- Proponent: InterDigital, JCTVC-Q0037, JCTVC-R0168

- Crosschecker: Canon, JCTVC-R0094

- Description:

The Burrows–Wheeler transform (BWT) is used to increase the correlation between adjacent positions in the palette index map. The BWT is applied before encoding the palette index map. After scanning each position as a 1-D index sequence, this sequence is fed into the BWT process and the end position is also coded as BWT side-information (overhead) for decoder side reconstruction. Due to the side information needed for BWT reconstruction, a 1-bit flag for each CU is used to indicate if the BWT is performed or not. The BWT with adaptive scanning methods will be tested to get best performance.

- Results:

Left: BD-rates of Test B.9 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.9 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.10 – Palette index value mapping

- Proponent: InterDigital, JCTVC-Q0037, JCTVC-R0169

- Crosschecker: Qualcomm, JCTVC-R0070

- Description:

To reduce the magnitude of the coded palette index value, the palette index value is mapped before being coded. The proposed palette index value mapping consists of two main steps: 1) reduce palette index value by comparing to the last compared palette index (LCPI), and 2) mapping of escape color index. In the first step, the palette index value is reduced by 1 if the palette index value is larger than its LCPI. In the second step, the index value of escape color pixels is set to a smaller index value, and the other entries in the palette table are shifted backward by 1 position. In addition, different escape color representation method will be tested.

- Results:

Left: BD-rates of Test B.10 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.10 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.11 – Escape color prediction

- Proponent: InterDigital, JCTVC-Q0037, JCTVC-R0170

- Crosschecker: Canon, JCTVC-R0247

- Description:

Escape color prediction method is used to code the escape colors efficiently. A flag is signaled to indicate if every color component of the current escape color pixel is the same as the previous escape color pixel or not. And if not (flag is equal to 0), the current escape color value is predicted from an entry in the palette table. The index value of the table entry used for prediction, along with the prediction difference (including absolute difference and the sign bit), are signaled to the decoder. This test is applied for lossless coding only.

- Results:

Left: Bit savings of Test B.11 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: Bit savings of Test B.11 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.12 – Binarization of escape sample and palette index

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047, JCTVC-R0065

- Crosschecker: Canon, JCTVC-R0127

- Description:

Alternatives to fixed length binarization of escape pixel and index coding are tested in this experiment. Variable length binarization (e.g. truncated binary code) will be applied for coding of escape pixel and palette index coding. Index adjustment as described in JCTVC-Q0047 will also be tested.

- Results:

Left: BD-rates of Test B.12 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.12 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.13 – Flag to indicate escape and index coding

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047, JCTVC-R0066

- Crosschecker: Microsoft, JCTVC-R0299

- Description:

Instead of assigning an index to indicate escape pixel, a flag is used to indicate whether the current pixel is an escape pixel or index coded. Modifications to the signaling scheme to accommodate this flag will also be tested.

- Results:

Left: BD-rates of Test B.13 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.13 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.14 – Restricted run coding

- Proponent: Nokia, JCTVC-Q0071, JCTVC-R0045

- Crosschecker: KDDI, missing

- Description:

This test evaluates performance of the run mode restriction technique. The proposed method signals which palette indexes have “palette_run” associated with them in “run mode” and which don’t. For the palette indexes for which the palette_run is not signaled those are assumed to be 0. This avoids signaling unnecessary run-length information for “stand-alone” samples that are not repeating consequently within the block. A single CU level syntax element is included in the bitstream to indicate the maximum palette index for which the “palette_run” is indicated.

- Results:

Left: BD-rates of Test B.14 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.14 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test B.15 – Scanning indicator for the palette coded CUs

- Proponent: Nokia, JCTVC-Q0071, JCTVC-R0048

- Crosschecker: KDDI, missing

- Description:

This test evaluates performance of the palette CU scanning technique. The proposed method signals whether the palette coded CU is scanned in horizontal or vertical scan order. Proposed method does not change the coding/decoding of the remaining syntax elements of the palette coded CU.

- Results:

Left: BD-rates of Test B.15 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test B.15 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

## Category C – Combinations of palette table coding and color index map coding

### Test C.1 – Combination of Test A.2 and Test B.1

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0083, JCTVC-R0122

- Crosschecker: Canon, JCTVC-R0095

- Description:

This test combines Test A.2 and Test B.1, which cover the triplet palette representation and coding and the line-based color index map coding.

- Results:

Left: BD-rates of Test C.1 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test C.1 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

- Supplemental (non-CE) results:

The encoder-only RDO process in Test A.6 is integrated with Test C.1.

Left: BD-rates of Test C.1 supplemental versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test C.1 supplemental versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

To get a rough idea on the overall SCCE3 coding gain, Test C.1 supplemental is chosen here as an example of comparison against palette-off. The results are as follows.

Left: BD-rates of Test C.1 supplemental versus palette-off under FF-IBC (Anchor 1-2)

Right: BD-rates of Test C.1 supplemental versus palette-off under 2CTU-IBC (Anchor 2-2)

### Test C.2 – Combination of palette coding tools

- Proponent: Canon, JCTVC-Q0069, JCTVC-R0086

- Crosschecker: MediaTek, JCTVC-R0294

- Description:

This test includes JCTVC-Q0063, JCTVC-Q0064, JCTVC-Q0065, JCTVC-Q0066, and encoding choices. It is related to the combinations of the aforementioned tools. Encoding choices will incorporate further improvements related to said combinations, as well as palette derivation and palettization process improvements.

- Results:

Left: BD-rates of Test C.2 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test C.2 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

Runtime percentages are inaccurate.

### Test C.3 – Combination of Test A.4 and Test B.8

- Proponent: Huawei, JCTVC-Q0034/JCTVC-Q0176, JCTVC-R0144

- Crosschecker: MediaTek, JCTVC-R0295

- Description:

This test combines aforementioned tools in Test A.4 and Test B.8, which covers color table processing using packed components and string match based index map coding.

- Results:

Left: BD-rates of Test C.3 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test C.3 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

### Test C.4 – Combination of Tests A.3, Test A.7, Test A.8, Test B.12, and Test B.13

- Proponent: Qualcomm, JCTVC-Q0031/JCTVC-Q0047/JCTVC-Q0225, JCTVC-R0067

- Crosschecker: MediaTek, JCTVC-R0296

- Description:

A combination of palette stuffing (A.3), encoder palette improvements (Test A.7), binary prediction vector signaling (Test A.8), truncated binary coding of indices (Test B.12), and a flag to indicate escape pixel or index value (Test B.13) will be tested. Combinations of subsets of these methods will also be tested.

- Results:

Left: BD-rates of Test C.4 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test C.4 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

- Supplemental (non-CE) results:

The encoder-only RDO process in Test C.2 with some modifications is integrated with Test C.4.

Proposal: Qualcomm, JCTVC-R0175

Crosscheck report: MediaTek, JCTVC-R0296, only the encoder-only non-normative part is crosschecked.

Left: BD-rates of Test C.4 supplemental versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test C.4 supplemental versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

The encoder-only RDO process in Test C.2 with some modifications is integrated with Test C.4 and two tools are disabled. The escape is coded as an index instead of a flag and the redundancy reduction part of subtest B.12 is disabled.

Proposal: Qualcomm, JCTVC-R0175 (C4 with improved encoder and subset of tools)

Crosscheck report: Sharp, JCTVC-R0308

Left: BD-rates of Test C.4 supplemental versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test C.4 supplemental versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

## Category D – Single color modes

### Test D.1 – Single color mode

- Proponent: MediaTek, JCTVC-Q0033/JCTVC-Q0093, JCTVC-R0058

- Crosschecker: MERL, JCTVC-R0160

- Description:

In the proposed scheme, one flag is signaled in the slice header to enable the single color mode for the entire slice. When the slice-level flag is enabled, one CU-level flag is further signaled to indicate the enabling of the single color mode for coding each CU.

To reconstruct a CU coded as single color mode, a sample color candidate list is first constructed by using the spatial neighboring samples of current CU in a predefined order into the candidate list. A redundancy pruning process is applied to the construction of sample candidate list. After the pruning process, if empty entry exists, a default candidate (e.g. value equal to 1<<(g_bitDepth-1)) will be used to fill in those empty entries. After the sample candidate list is constructed, a sample candidate index is signaled in the bitstream to indicate which sample candidate is used to fill the current CU.

- Results:

Left: BD-rates of Test D.1 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test D.1 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

- Supplemental (non-CE) results:

In Test D.1, the five spatial neighboring samples of the current CU are reduced to two.

Proposal: MediaTek, JCTVC-R0198

Crosscheck report: MERL, JCTVC-R0222

BD-rates of Test D.1 supplemental versus Q0094BF palette under FF-IBC (Anchor 1-1) are as follows.

### Test D.2 – Independent uniform prediction

- Proponent: MERL, JCTVC-Q0036, JCTVC-R0200

- Crosschecker: MediaTek, JCTVC-R0265

- Description:

In Independent Uniform Prediction (IUP), a small set of color triplets is available to each CU for generating a uniform prediction block for each color component, where all pixels in the predictor block have the same value as the corresponding element in the color triplet selected from the set. This predictor block is then treated as in other prediction methods, where the predictor is added to the reconstructed residual block to form the reconstructed block in the decoder.

- Results:

Left: BD-rates of Test D.2 versus Q0094BF palette under FF-IBC (Anchor 1-1)

Right: BD-rates of Test D.2 versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

# Comparisons

To compare different tests, for each coding structures (AI, RA, and LB), the average BD-rate of all sequences from the first four RGB or YUV classes is calculated color component-wise, and the obtained average BD-rates of the last two color components are further averaged. The reason of skipping the last two RGB or YUV classes is that all the tests show close-to-zero BD-rates for these classes.

## Comparison of Category A proposals – Palette table coding

- BD-rate savings of Category A tests versus Q0094BF palette under FF-IBC (Anchor 1-1)

- BD-rate savings of Category A tests versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

## Comparison of Category B proposals – Color index map coding

- BD-rate savings of Category B tests versus Q0094BF palette under FF-IBC (Anchor 1-1)

- BD-rate savings of Category B tests versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

## Comparison of Category C proposals – Combinations of palette table coding and color index map coding

- BD-rate savings of Category C tests versus Q0094BF palette under FF-IBC (Anchor 1-1)

- BD-rate savings of Category C tests versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

## Comparison of Category D proposals – Single color modes

- BD-rate savings of Category D tests versus Q0094BF palette under FF-IBC (Anchor 1-1)

- BD-rate savings of Category D tests versus Q0094BF palette under 2CTU-IBC (Anchor 2-1)

# Conclusions

Quite a few tests were conducted in SCCE3. Evidences were provided that palette mode can significantly improve screen content coding efficiency. In general, the palette mode complexity is affordable.

To be further discussed in the Sapporo meeting.

# References

- X. Guo, Y. Lu, S. Li, “RCE4: Test1. Major-color-based screen content coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P0108, 16th Meeting: San José, US, January 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, J. Xu, “Suggested software for the AHG on investigation of palette mode coding tools,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-P0303, 16th Meeting: San José, US, January 2014.

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1123, 17th Meeting: Valencia, ES, March-April 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1015, 17th Meeting: Valencia, ES, March-April 2014.