| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0034 |
| --- | --- |

| Title: | SCCE4: Summary Report on string matching for sample coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ying Chen Jizheng Xu | Tel: Email: | cheny@qti.qualcomm.com jzxu@microsoft.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCCE4 on String matching for sample coding. The evaluated tools and the test conditions are defined in document JCTVC-Q1124.

# Introduction

Techniques related to string matching for sample coding, a.k.a. 1D dictionary coding, were proposed in JCTVC-Q0035, JCTVC-Q0035/JCTVC-Q0149, JCTVC-Q0124 and JCTVC-Q0034/JCTVC-Q0176 in the previous JCT-VC meeting. To further investigate these techniques, this core experiment, SCCE 4 was established.

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| Bin Li Jizheng Xu | libin@microsoft.com jzxu@microsoft.com | Microsoft | X | X |
| Ying Chen Feng Zou Chao Pang Joel Sole | cheny@qti.qualcomm.com fzou@qti.qualcomm.com cpang@qti.qualcomm.com joels@qti.qualcomm.com | Qualcomm | X | X |
| Jing Ye Shan Liu Yu-Wen Huang Tzu-Der Chuang Shih-Ta Hsiang | jing.ye@mediatek.com shan.liu@mediatek.com yuwen.huang@mediatek.com peter.chuang@mediatek.com shih-ta.hsiang@mediatek.com | MediaTek | X | X |
| Tao Lin | lintao@tongji.edu.cn | Tongji University | X |  |
| Zhan Ma Wei Wang Meng Xu Haoping Yu | zhan.ma@huawei.com wei.wang.sc@huawei.com m.xu@huawei.com haoping.yu@huawei.com | Huawei | X | X |
| Xiaoyu Xiu Yan Ye | Xiaoyu.Xiu@InterDigital.com Yan.Ye@InterDigital.com | InterDigital |  |  |
| Wen-Hsiao Peng Chun-Chi Chen Tsui-Shan Chang | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.nctu.edu.tw abaibai1122.cs01g@nctu.edu.tw | NCTU |  |  |
| Jung-Yang Kao Jih-Sheng Tu Chao-Hsiung Hung Yao-Jen Chang Ching-Chieh Lin | Yang_Kao@itri.org.tw sunriseJSTu@itri.org.tw chhung@itri.org.tw britpablo@itri.org.tw JackLin.itri@gmail.com | ITRI |  |  |
| Ankur Saxena | a.saxena1@samsung.com (saxena.ankur0@gmail.com) | Samsung |  |  |
| Robert Cohen Akira Minezawa Kazuyuki Miyazawa | cohen@merl.com Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp | MERL |  |  |
| Patrice Onno Guillaume Laroche Chris Rosewarne | patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr Chris.Rosewarne@cisra.canon.com.au | Canon |  |  |
| Seung-Hwan Kim | kimse@sharplabs.com | Sharp |  |  |
| Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com | Fujitsu |  |  |
| Sehoon Yea | sehoon.yea@lge.com | LGE |  |  |

# List of documents

| Participants | Doc No. | Title | Type |
| --- | --- | --- | --- |
| CE contributions |  |  |  |
| Microsoft | JCTVC-R0098 | SCCE4: Result of Test 3.1 | Proposal |
| Qualcomm | JCTVC-R0140 | SCCE4: Test 3.2 Pixel based 1D dictionary coding | Proposal |
| Tongji/MediaTek | JCTVC-R0060 | SCCE4: Results of Test 3.3 | Proposal |
| Huawei | JCTVC-R0145 | SCCE4: Result of Test 4.3.4 for string search for sample coding | Proposal |
| NCTU/ITRI | JCTVC-R0130 | Crosscheck of SCCE4 Test 3.1 on Combined Normal 1-D Dictionary Mode and Reconstruction Based 1-D Dictionary Mode | Crosscheck |
| MediaTek | JCTVC-R0197 | SCCE4: Crosscheck results of Test 3.2 | Crosscheck |
| Microsoft | JCTVC-R0109 | SCCE4: Cross-check of test 3.3 (JCTVC-R0060) | Crosscheck |
| Microsoft | JCTVC-R0110 | SCCE4: Cross-check of test 3.4 (JCTVC-R0145) | Crosscheck |
| Non-CE contributions |  |  |  |
| MediaTek | JCTVC-R0137 | Non-SCCE4:Unified method for coding string matching syntax using a universal entropy coding scheme | Proposal |
| Qualcomm | JCTVC-R0225 | Non-SCCE4: Constrained run for 1D dictionary coding | Proposal |
| Qualcomm | JCTVC-R0303 | Non-SCCE4: Cross-check of JCTVC-R0137 | Crosscheck |

# Tools under investigation

## JCTVC-R0098 from Microsoft

The 1-D dictionary is enabled at the CU level with both horizontal scanning and vertical scans. Per-pixel matching is used for string matching. There are two types of 1-D dictionary mode, line based 1-D matching and shape based 2-D matching.

In the first 1D matching mode, the previous coded pixels using 1-D dictionary mode are kept in the dictionary (which is dynamically updated). In this mode, prediction mode and direct mode are allowed. If a matching is found, an offset (the offset relative to the position of the current pixel in the dictionary) and a matching length are signaled, otherwise the pixel value is signaled directly.  The above two cases are called prediction mode and escape mode respectively.

In the second 2D matching mode, all the previously reconstructed pixels can be used for prediction. Similar to 1D matching mode, 2D matching mode has prediction mode and escape mode. If a matching is found, a 2D MV and a matching length are signaled, otherwise, the pixel value is also signaled directly. Note that in 2D matching, when the current region starts a new row or column, the pixel used for prediction also starts a new row or column, as shown in the following figure. There is no additional memory requirement at the decoder side.

## JCTVC-R0140 from Qualcomm

1D dictionary is enabled at the CU level but with a search range constrained to the current CTU and left CTU. The proposed method utilizes horizontal scan only and per-pixel matching. The proposed method utilizes 2D matching mode thus can share the same reconstruction buffer as in intra BC, with up to 2CTU search range. It supports prediction mode and escape mode. The previously coded pixels within the current CU are allowed to be chosen as reference pixels.

## JCTVC-R0060 from Tongji/MediaTek

1D dictionary is enabled at the CU level, but based on sample matching: instead of matching three components of a full pixel, sample matching can take one sample of a pixel and then either stop or go to the next sample of the same pixel. The proposed method utilizes horizontal scan only and is based on 1D matching mode. It supports also prediction and escape mode. In the 1D matching mode, 1-D distance (pos) is used to represent 2-D displacement with the basic unit of sample instead of pixel. The maximum matching string length is extended from 273 to 12288. It is found that the matching string length has unusual characteristic that many special and large values such as multiples of 3 and multiples of the current CU width occur much more frequently than many other small values. Based on the characteristics, a mapping of the string length values to two matching parameters (pos and len) is introduced to improve the coding efficiency for length values. A histogram-based qp-dependent color quantization scheme is used as a pre-processing step of the input pixels to enable lossy match at the encoder side.

## JCTVC-R0145 from Huawei

1D dictionary is enabled at the CU level but with a search range constrained to the current CTU and three left CTUs. The proposed method utilizes horizontal scan and per-pixel matching. The 1D string matching can be based on either 1D matching mode or 2D matching mode. Compared to other CE proposals, different approaches for both prediction mode and escape mode are used. In the prediction mode, residual can be signaled with the normal HEVC residue coding. In the escape mode, if the color value can be found in an adaptive color table, the index to it rather than the color value itself is coded, otherwise, the color value is coded and the adaptive color table is updated based on the color value. The adaptive table is derived at the decoder unless if it contains only one entry. In that case, the adaptive table is signaled.

# Comparison of the CE proposals

## Summary of the coding results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM1.0, and SCC common test conditions [1] are used. The BD-rates are provided for the lossy configuration.

Table 1 Lossy results in full frame intra BC

## Summary of the CE proposals

The CE proposals are compared in Table 2 and Table 3. As shown in these tables, commonalities may be identified among the CE proposals. The major difference of the proposed method, in terms of coding efficiency, encoder, decoder memory and complexity may be mainly due to different encoder optimizations.

Table 2 Comparison of the key technical aspects of the CE proposals (1/2)

| Proposal | Scan | Min. run | Matching modes | Search Range | Res. Coding | Additional dec. Buffer | Lossy enc. Match |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R0098/3.1 | V&H | 1 pixel | 1D/2D | Full frame | No | 1M pixels | No |
| R0140/3.2 | H | 1 pixel | 2D | 2 CTU | No | None | No |
| R0060/3.3 | H | 1 sample | 1D | Full frame | No | 4K/3~4M/3 pixels | Yes |
| R0145/3.4 | H | 1 pixel | 1D/2D | 3 or 4?CTU | Yes | 3x64x64 pixels | No |

Table 3 Comparison of the key technical aspects of the CE proposals (2/2)

| Proposal | Enc. Hash Table Size (Bytes) | Eff. Highlight | Enc. Time AI, Other | Dec. time | Miscellaneous |
| --- | --- | --- | --- | --- | --- |
| R0098/3.1 | 160 M B (1080p) | -21.3% | 150%, 110% | 100% | 2D matching from pixels outside the current CU |
| R0140/3.2 | 64 K (2 CTUs) | -11.7% | 190%, 115% | 100% | 2D matching from any decoded pixels of the current CU |
| R0060/3.3 | 16K~ 16M (configurable) | -14.1% | 140%, 110% | 105% | Mapped table for offset/length coding |
| R0145/3.4 | 144 K (3 CTUs) | -11.4% | 105%, 105% | 120%~180% | Adaptive color table for escape pixel |

# Description of the non-CE proposals

## JCTVC-R0137 from MediaTek

This contribution develops a method to code new syntax elements employed by string matching coding. The proposed method utilizes a universal entropy coding scheme representing a syntax value by sending its most significant bit (MSB) index and refinement bits. It is reported to achieve 4.8%, 2.9%, and 2.2% average Luma BD-rate savings compared with the results of the SCCE4 Test3.1 for YUV, text & graphics with motion, 1080p sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively, of the screen content coding (SCC) CTCs.

## JCTVC-R0255 from Qualcomm

This proposal presents preliminary results by constraining the current 1D dictionary (string match) methods investigated under the current SCCE4. It is reported that the current 1D dictionary methods may require much more frequent memory accesses from different locations of the reference area compared to e.g., Intra BC. Therefore, it could be beneficial to address more pixels for each string match. In this proposal, specifically, the matching length is constrained to be equal or larger than a predefined threshold T, e.g., 4. The simulation results show that the constraint as implemented based on SCCE4 proposals JCTVC-R0098 and JCTVC-R0140 lead to noticeable bitrate increase.

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-Q1015, Valencia, ES, March 2014.