| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0035 |
| --- | --- |

| Title: | SCCE5: Summary report of HEVC Screen Content Coding Core Experiment 5 on Inter-component prediction and adaptive color transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu Jianle Chen | Email: | xiaoyu.xiu@interdigital.com cjianle@qti.qualcomm.com |
| Source: | SCCE coordinators |  |  |

_____________________________

# Introduction

A summary of HEVC Screen Content Coding Core Experiment (SCCE) 5 on inter-component prediction and adaptive color transforms is reported. Five methods have been evaluated based on the screen content coding common test conditions according to the SCCE5 description in JCTVC-Q1125.

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| Z. Ma W. Wang M. Xu H. Yu | zhan.ma@huawei.com Wei.Wang.sc@huawei.com m.xu@huawei.com haoping.yu@huawei.com | Huawei Technologies (USA) | X | X |
| A. Minezawa K. Miyazawa S. Sekiguchi | Minezawa.Akira@ds.MitsubishiElectric.co.jp Miyazawa.Kazuyuki@cw.MitsubishiElectric.co.jp Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp | Mitsubishi Electric Corporation | X | X |
| P. Lai X. Zhang Y.-W. Huang S. Liu T.-D. Chuang | polin.lai@mediatek.com xianguo.zhang@mediatek.com yuwen.huang@mediatek.com Shan.Liu@mediatek.com peter.chuang@mediatek.com | MediaTek Inc. | X | X |
| J. Chen L. Zhang J. Sole M. Karczewicz | cjianle@qti.qualcomm.com lizhang@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com | Qualcomm Incorporated | X | X |
| J. Xu | jzxu@microsoft.com | Microsoft Corp. | X | X |
| X. Xiu Y.-W. He Y. Yan | xiaoyu.xiu@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com | InterDigital Communication Inc. | X | X |
| W.-H. Peng C.-C. Chen T.-S. Chang | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.nctu.edu.tw abaibai1122.cs01g@nctu.edu.tw | NCTU |  |  |
| J.-Y. Kao J.-S. Tu C.-H. Hung C.-C. Lin | Yang_Kao@itri.org.tw sunriseJSTu@itri.org.tw chhung@itri.org.tw britpablo@itri.org.tw JackLin.itri@gmail.com | ITRI |  |  |
| K. Kawamura | ki-kawamura@kddi.com | KDDI |  | X |
| A. Saxena | a.saxena1@samsung.com | Samsung |  |  |
| S.-H. Kim K.Misra | kimse@sharplabs.com misrak@sharplabs.com | Sharp |  |  |
| C. Rosewarne C. Gisquet G. Laroche P. Onno | Chris.Rosewarne@cisra.canon.com.au christophe.gisquet@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr | Cannon |  |  |
| J. Xu | Jun.Xu@am.sony.com | Sony |  |  |

# Document list

## SCCE contributions and corresponding cross-checks

### Test 3.1.1: LM Chroma for intra mode

- JCTVC-R0146, “SCCE5: Result of Test 3.1.1 for modified LMChroma”, M. Xu, Z. Ma, W. Wang, H. Yu (Huawei)

  - Cross-check: (Mediatek)

### Test 3.1.2: LM Chroma for intra mode and intra block copy mode

- JCTVC-R0072, “SCCE5 3.1.2: Extended inter-component prediction”, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

  - Cross-check: JCTVC-R0177 (Qualcomm)

### Test 3.1.3: Alpha parameter signaling for inter-component prediction

- JCTVC-R0115, “SCCE5 Test 3.1.3: Adaptive alpha-paramter coding in cross-component prediction”, X. Zhang, K. Zhang, J. An, H. Huang, S. Lei  (MediaTek)

  - Cross-check: (KDDI)

### Test 3.2.1: Adaptive color transform in residual domain

- JCTVC-R0147, “SCCE5 Test 3.2.1: In-loop color-space transform”, L. Zhang, J. Chen, J. Sole, M. Karczewicz (Qualcomm), X. Xiu, Y. He, Y. Ye  (InterDigital)

  - Cross-check: JCTVC-R0111 (Microsoft)

### Test 3.2.2: Adaptive color transform in pixel domain

- JCTVC-R0099, “SCCE5: Result of Test 3.2.2”, B. Li, J. Xu (Microsoft)

  - Cross-check: JCTVC-R0254 (Huawei)

## Non-SCCE contributions

- JCTVC-R0178, “Non-SCCE5: On index signaling of cross component prediction”, L. Zhang, M. Karczewicz, J. Chen, J. Sole (Qualcomm)

- JCTVC-R0180, “Non-SCCE5: Deblocking filter for in-loop color-space transform”, L. Zhang, J. Chen, J. Sole, M. Karczewicz (Qualcomm)

- JCTVC-R0219, “Improvement of cross-component prediction”, X. Zhang, R. Cohen (MERL)

# Tested methods

Based on the screen content coding common test conditions, five methods, as described as follows, were evaluated.

### Test 3.1.1: LM Chroma for intra mode (JCTVC-R0146)

- LM Chroma prediction mode which predicts chroma samples using linear combination of luma samples

- Essentially same to LM Chroma model as proposed in HEVC version 1, removing the luma sub-sampling.

- LM Chroma is only used in intra coded CU as one addition chroma intra mode.

- LM Chroma is enabled/disabled on PU basis.

- LM calculation is performed on TU basis.

- Averageperformance (full IntraBC lossy)

- AI

- G: -1.7%, B: -2.0%, R: -2.0%

- Y: -0.3%, U: -1.6%, V: -1.5%

- Avg enc time: 104%, avg dec time: 101%

- RA

- G: -1.0%, B: -1.2%, R: -1.3%

- Y: -0.2%, U: -1.4%, V: -1.2%

- Avg enc time: 101%, avg dec time: 101%

- LD

- G: -0.5%, B: -0.6%, R: -0.6%

- Y: 0.0%, U: -0.8%, V: -0.7%

- Avg enc time: 101%, avg dec time: 101%

### Test 3.1.2: LM Chroma for intra mode and intra block copy mode (JCTVC-R0072)

- LM Chroma prediction mode which predicts chroma samples using linear combination of luma samples.

- R (Cr) can be predicted from either G (Y) or B (Cb).

- LM Chroma is not jointly applied with CCP.

- LM Chroma is applied to intra mode and intra block copy mode.

- LM Chroma in intra mode

- LM Chroma is enabled/disabled on PU basis.

- LM calculation is performed on TU basis.

- LM Chroma in intra block copy mode

- LM Chroma is enabled/disabled on PU basis.

- LM calculation is performed on CU basis.

- Additional encoder only change

- A modified lambda for YCbCr sequences.

- Average performance (full IntraBC lossy)

- AI

- G: -3.9%, B: -4.5%, R: -4.6%

- Y: -1.9%, U: -1.9%, V: -1.9%

- Avg enc time: 114%, avg dec time: 101%

- RA

- G: -2.4%, B: -2.9%, R: -3.1%

- Y: -1.5%, U: -1.4%, V: -1.6%

- Avg enc time: 101%, avg dec time: 101%

- LD

- G: -1.2%, B: -1.5%, R: -1.6%

- Y: -1.1%, U: 0.2%, V: 0.3%

- Avg enc time: 101%, avg dec time: 100%

### Test 3.1.3: Alpha parameter signaling for inter-component prediction (JCTVC-R0115)

- Two signaling methods are used to signal the alpha parameter in CCP.

- A PPS flag is added to indicate which signaling method is applied.

- Additional binarization method for the alpha parameter.

- Additional 10 contexts are used for the newly introduced syntax element log2_res_scale_pred_4minus and bypass coding is applied to log2_res_scale_neg.

| Alpha | YCbCr |  | GBR |  |
| --- | --- | --- | --- | --- |
|  | log2_res_scale_abs_plus1 | res_scale_sign_flag | log2_res_scale_pred_4minus | log2_res_scale_neg |
| 0 | 0 | 0 | 0 | N/A |
| 8 | 1111 | 0 | 10 | N/A |
| 4 | 1110 | 0 | 110 | N/A |
| 2 | 110 | 0 | 1110 | N/A |
| 1 | 10 | 0 | 11110 | N/A |
| −1 | 10 | 1 | 11111 | 00 |
| −2 | 110 | 1 | 11111 | 01 |
| −4 | 1110 | 1 | 11111 | 10 |
| −8 | 1111 | 1 | 11111 | 11 |

- Average performance (full IntraBC lossy)

- AI

- G: -0.1%, B: -0.1%, R: -0.1%

- Avg enc time: 100%, avg dec time: 101%

- RA

- G: -0.2%, B: -0.1%, R: -0.2%

- Avg enc time: 99%, avg dec time: 99%

- LD

- G: -0.4%, B: -0.2%, R: -0.3%

- Avg enc time: 100%, avg dec time: 102%

### Test 3.2.1: Adaptive color transform in residual domain (JCTVC-R0147)

- The method adaptively transforms prediction residuals of one color space into another at CU level.

- The method can be jointly applied with CCP.

- The method is applied to intra mode, intra block copy mode and inter mode.

- Two color space transforms are applied to lossy coding and lossless coding.

- Lossy coding

- The YCoCg transform is applied.

Forward: C'0C'1C'2=12120−2−12−1C0C1C24 	Inverse:	C0C1C2=11−11011−1−1C'0C'1C'2

- QP − 5 is applied to code C'0 and C'2 components and QP − 3 is applied to code C'1component, as the forward color transform is not normalized.

- Lossless coding

- The YCoCg-R transform is applied.

- The CU level color space transform flag is only signaled when there is at least one non-zero coefficient in the current CU.

- Additional encoder only change for RGB coding

- Modified weighting parameter wchroma for chroma related RDO processes.

- When color space transform is used, IntraBC BV is determined in transformed color space for intra block copy mode.

- Average performance (full IntraBC lossy)

- AI

- G: -19.2%, B: -7.2%, R: -8.3%

- Y: -0.6%, U: -1.8%, V: -1.6%

- Avg enc time: 141%, avg dec time: 100%

- RA

- G: -21.8%, B: -7.7%, R: -9.3%

- Y: -0.4%, U: -1.9%, V: -1.5%

- Avg enc time: 110%, avg dec time: 96%

- LD

- G: -21.3%, B: -4.7%, R: -6.1%

- Y: -0.6%, U: -2.2%, V: -1.9%

- Avg enc time: 109%, avg dec time: 97%

### Test 3.2.2: Adaptive color transform in pixel domain (JCTVC-R0099)

- Adaptive color space transform of GBR into YCoCg, RGB and BGR

- The method adaptively transforms prediction pixels into one of those color spaces at CU level.

- The method is only applied to intra mode. At most, 4*N + 1 neighboring pixels are converted into a specific color space. And the reconstructed pixels are needed to be converted back to GBR domain.

- Lossy coding

- Color space transforms from GBR to YCoCg, RGB and BGR are enabled.

- When YCoCg is used, QP − 5 is applied to code Y and Cg components and QP − 3 is applied to code Co component, as the color transform GBR->YCoCg is not normalized.

- Lossless coding

- Color space transforms from GBR to RGB and BGR are enabled.

- Average performance (full IntraBC lossy)

- AI

- G: -11.9%, B: -7.0%, R: -7.9%

- Avg enc time: 179%, avg dec time: 103%

- RA

- G: -10.2%, B: -6.2%, R: -6.8%

- Avg enc time: 107%, avg dec time: 100%

- LD

- G: -5.9%, B: -3.8%, R: -3.8%

- Avg enc time: 106%, avg dec time: 99%

# Summary of results

Results are obtained by following the the screen content coding common test conditions [1]. All the methods were tested against two mandatory anchors: SCM-1.0 with full frame IBC search and SCM-1.0 with 2-CTU IBC search. Cross-checkers confirmed the results.

## Lossy coding results

Table 1 Summarized lossy coding results for full frame IBC

| AI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 YCoCg |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | -0.3% | -0.5% | -1.2% | -1.9% | -1.2% | -1.9% | 0.0% | 0.0% | -7.4% | -3.4% | -10.7% | -4.7% | -3.6% | -2.3% | -2.0% | -1.9% | -4.6% | -3.3% |
| RGB, text & graphics,720p | -1.5% | -1.7% | -4.0% | -4.5% | -4.0% | -4.5% | 0.0% | -0.1% | -14.4% | -7.1% | -17.2% | -6.3% | -8.3% | -4.3% | -2.1% | -2.6% | -8.4% | -4.8% |
| RGB, mixed content, 1440p | -1.6% | -1.8% | -3.4% | -4.0% | -3.4% | -4.0% | -0.2% | -0.1% | -18.5% | -9.1% | -20.9% | -7.7% | -13.1% | -7.4% | -1.1% | -1.6% | -13.0% | -7.7% |
| RGB, mixed content, 1080p | -1.1% | -1.4% | -3.0% | -3.6% | -3.0% | -3.6% | -0.2% | -0.1% | -13.5% | -5.6% | -15.9% | -4.2% | -7.8% | -4.3% | -0.9% | -1.4% | -7.8% | -4.5% |
| RGB, Animation, 720p | -2.4% | -3.4% | -7.6% | -8.5% | -7.6% | -8.5% | -0.2% | -0.1% | -22.5% | -18.6% | -25.2% | -16.3% | -20.6% | -15.5% | -3.8% | -8.7% | -20.0% | -16.8% |
| RGB, camera, 1080p | -3.1% | -3.1% | -4.5% | -4.8% | -4.5% | -4.8% | -0.1% | -0.1% | -21.2% | -9.0% | -25.3% | -7.5% | -18.4% | -7.1% | 0.1% | -2.3% | -17.6% | -7.8% |
| YUV, text & graphics, 1080p | -0.2% | -0.8% | -1.0% | -2.5% | -1.7% | -2.2% | - | - | -2.0% | -1.6% | -2.0% | -1.6% | - | - | - | - | - | - |
| YUV, text & graphics,720p | -0.3% | -1.6% | -1.0% | -3.6% | -1.9% | -2.6% | - | - | -1.3% | -1.7% | -1.3% | -1.7% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | -0.2% | -1.4% | -0.7% | -3.0% | -1.5% | -1.6% | - | - | -0.1% | -1.1% | -0.1% | -1.1% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | -0.1% | -0.8% | -0.6% | -2.6% | -1.3% | -1.5% | - | - | -0.2% | -0.6% | -0.2% | -0.6% | - | - | - | - | - | - |
| YUV, Animation, 720p | -0.4% | -2.6% | -1.4% | -5.1% | -2.7% | -2.4% | - | - | 0.2% | -5.2% | 0.2% | -5.2% | - | - | - | - | - | - |
| YUV, camera, 1080p | -0.5% | -2.3% | -0.7% | -3.3% | -2.2% | -1.2% | - | - | 0.1% | 0.0% | 0.1% | 0.0% | - | - | - | - | - | - |
| Enc Time(%) | 104% |  | 114% |  | - |  | 100% |  | 146% |  | 141% |  | 135% |  | 138% |  | 179% |  |
| Dec Time(%) | 101% |  | 101% |  | - |  | 101% |  | 101% |  | 100% |  | 104% |  | 101% |  | 103% |  |
| RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | 0.0% | -0.2% | -0.5% | -0.9% | -0.5% | -0.9% | -0.2% | -0.1% | -6.3% | -2.8% | -10.1% | -3.5% | -1.9% | -1.3% | -1.4% | -1.2% | -2.5% | -1.9% |
| RGB, text & graphics,720p | -1.4% | -1.5% | -4.2% | -4.9% | -4.2% | -4.9% | -0.1% | -0.2% | -15.8% | -7.6% | -19.9% | -7.8% | -9.8% | -5.1% | -2.4% | -2.7% | -9.9% | -5.8% |
| RGB, mixed content, 1440p | -1.0% | -1.3% | -2.2% | -2.8% | -2.2% | -2.8% | -0.2% | -0.2% | -23.8% | -11.0% | -27.6% | -10.7% | -16.5% | -9.7% | -1.0% | -1.3% | -16.4% | -10.0% |
| RGB, mixed content, 1080p | -0.7% | -1.0% | -2.2% | -2.7% | -2.2% | -2.7% | -0.1% | 0.0% | -16.1% | -6.2% | -19.9% | -5.8% | -10.4% | -6.1% | -0.7% | -1.2% | -10.3% | -6.2% |
| RGB, Animation, 720p | -1.4% | -2.0% | -3.5% | -4.4% | -3.5% | -4.4% | -0.3% | -0.1% | -21.6% | -15.9% | -25.6% | -13.8% | -14.2% | -11.4% | -1.5% | -4.1% | -14.0% | -11.9% |
| RGB, camera, 1080p | -1.6% | -1.6% | -2.1% | -2.3% | -2.1% | -2.3% | -0.3% | -0.2% | -25.9% | -12.4% | -27.9% | -9.7% | -8.5% | -2.9% | 0.1% | -1.0% | -8.0% | -3.2% |
| YUV, text & graphics, 1080p | 0.0% | -0.4% | -0.4% | -1.4% | -1.0% | -0.9% | - | - | -1.3% | -1.3% | -1.3% | -1.3% | - | - | - | - | - | - |
| YUV, text & graphics,720p | -0.3% | -1.3% | -1.4% | -4.1% | -2.4% | -3.2% | - | - | -1.9% | -2.2% | -1.9% | -2.2% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | -0.1% | -1.5% | -0.5% | -3.1% | -1.0% | -1.5% | - | - | 0.2% | -1.0% | 0.2% | -1.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | -0.3% | -0.8% | -0.4% | -2.2% | -1.0% | -1.1% | - | - | 0.1% | -0.5% | 0.1% | -0.5% | - | - | - | - | - | - |
| YUV, Animation, 720p | -0.1% | -2.2% | -0.7% | -4.1% | -1.6% | -1.8% | - | - | -0.1% | -5.2% | -0.1% | -5.2% | - | - | - | - | - | - |
| YUV, camera, 1080p | -0.2% | -1.9% | -0.3% | -2.6% | -1.7% | -0.4% | - | - | 0.4% | 0.1% | 0.4% | 0.1% | - | - | - | - | - | - |
| Enc Time(%) | 101% |  | 101% |  | - |  | 99% |  | 112% |  | 110% |  | 104% |  | 101% |  | 107% |  |
| Dec Time(%) | 101% |  | 101% |  | - |  | 99% |  | 98% |  | 96% |  | 101% |  | 99% |  | 100% |  |
| LD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | 0.0% | -0.1% | -0.2% | -0.5% | -0.2% | -0.5% | -0.1% | -0.1% | -5.5% | -2.2% | -8.7% | -1.8% | -1.0% | -0.7% | -0.8% | -0.7% | -1.4% | -1.2% |
| RGB, text & graphics,720p | -1.0% | -0.9% | -3.4% | -3.7% | -3.4% | -3.7% | -0.2% | -0.1% | -16.5% | -7.6% | -20.2% | -6.2% | -6.5% | -3.9% | -2.2% | -2.1% | -6.3% | -4.0% |
| RGB, mixed content, 1440p | -0.7% | -0.9% | -1.1% | -1.7% | -1.1% | -1.7% | -0.3% | -0.3% | -24.6% | -10.1% | -28.2% | -7.8% | -12.8% | -7.2% | -0.7% | -1.0% | -12.6% | -7.4% |
| RGB, mixed content, 1080p | -0.2% | -0.5% | -0.6% | -1.1% | -0.6% | -1.1% | -0.7% | -0.4% | -16.4% | -4.4% | -20.1% | -1.6% | -8.1% | -4.8% | -0.5% | -0.8% | -8.2% | -4.9% |
| RGB, Animation, 720p | -0.6% | -0.9% | -1.2% | -1.6% | -1.2% | -1.6% | -0.3% | -0.1% | -20.1% | -12.8% | -24.3% | -8.1% | -5.5% | -4.9% | -0.4% | -1.2% | -5.5% | -5.0% |
| RGB, camera, 1080p | -0.5% | -0.4% | -0.6% | -0.6% | -0.6% | -0.6% | -0.4% | -0.2% | -23.5% | -10.7% | -26.2% | -6.9% | -1.6% | -0.1% | 0.0% | -0.4% | -1.5% | -0.3% |
| YUV, text & graphics, 1080p | 0.0% | -0.3% | -0.3% | -1.0% | -0.9% | -0.4% | - | - | -1.7% | -1.8% | -1.7% | -1.8% | - | - | - | - | - | - |
| YUV, text & graphics,720p | -0.1% | -0.9% | -1.3% | -3.2% | -2.1% | -1.9% | - | - | -2.1% | -2.8% | -2.1% | -2.8% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | -0.1% | -1.2% | -0.2% | -2.5% | -0.8% | 0.2% | - | - | 0.2% | -1.0% | 0.2% | -1.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.3% | -0.3% | 0.1% | -0.9% | -0.4% | 0.3% | - | - | 0.1% | -1.3% | 0.1% | -1.3% | - | - | - | - | - | - |
| YUV, Animation, 720p | 0.0% | -1.2% | 0.0% | -2.0% | -1.1% | 1.0% | - | - | -0.1% | -5.3% | -0.1% | -5.3% | - | - | - | - | - | - |
| YUV, camera, 1080p | -0.1% | -0.6% | 0.0% | -0.7% | -1.4% | 2.3% | - | - | 0.3% | -0.1% | 0.3% | -0.1% | - | - | - | - | - | - |
| Enc Time(%) | 101% |  | 101% |  | - |  | 100% |  | 110% |  | 109% |  | 102% |  | 101% |  | 106% |  |
| Dec Time(%) | 101% |  | 100% |  | - |  | 102% |  | 97% |  | 97% |  | 101% |  | 99% |  | 99% |  |

Table 2 Summarized lossy coding results for 2 CTU IBC

| AI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 YCoCg |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | -0.3% | -0.5% | -1.2% | -1.8% | -1.2% | -1.8% | 0.0% | 0.0% | -7.2% | -3.4% | -10.5% | -4.3% | -3.9% | -2.5% | -1.9% | -1.9% | -4.8% | -3.6% |
| RGB, text & graphics,720p | -1.0% | -1.1% | -4.1% | -4.6% | -4.1% | -4.6% | -0.1% | -0.1% | -14.3% | -7.3% | -17.2% | -6.2% | -9.0% | -4.9% | -2.1% | -2.6% | -9.0% | -5.4% |
| RGB, mixed content, 1440p | -1.1% | -1.3% | -3.2% | -3.8% | -3.2% | -3.8% | -0.2% | -0.1% | -17.4% | -8.4% | -19.9% | -7.0% | -12.8% | -7.0% | -1.1% | -1.6% | -12.7% | -7.2% |
| RGB, mixed content, 1080p | -0.8% | -1.0% | -2.8% | -3.4% | -2.8% | -3.4% | -0.1% | -0.1% | -12.9% | -5.4% | -15.6% | -4.0% | -8.1% | -4.4% | -0.8% | -1.3% | -8.0% | -4.6% |
| RGB, Animation, 720p | -2.5% | -3.5% | -7.6% | -8.5% | -7.6% | -8.5% | -0.2% | -0.1% | -22.5% | -18.6% | -25.2% | -16.3% | -20.6% | -15.5% | -3.9% | -8.7% | -20.0% | -16.8% |
| RGB, camera, 1080p | -3.2% | -3.2% | -4.6% | -4.8% | -4.6% | -4.8% | -0.1% | -0.1% | -21.2% | -9.0% | -25.3% | -7.5% | -18.4% | -7.1% | 0.1% | -2.3% | -17.7% | -7.8% |
| YUV, text & graphics, 1080p | -0.2% | -0.8% | -1.0% | -2.5% | -1.7% | -2.2% | - | - | -1.6% | -1.4% | -1.6% | -1.4% | - | - | - | - | - | - |
| YUV, text & graphics,720p | -0.1% | -1.2% | -1.0% | -3.4% | -1.9% | -2.2% | - | - | -0.9% | -1.4% | -0.9% | -1.4% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | -0.1% | -0.7% | -0.7% | -2.9% | -1.5% | -1.5% | - | - | 0.0% | -1.4% | 0.0% | -1.4% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | -0.1% | -0.4% | -0.6% | -2.2% | -1.4% | -1.1% | - | - | -0.1% | -0.6% | -0.1% | -0.6% | - | - | - | - | - | - |
| YUV, Animation, 720p | -0.4% | -2.6% | -1.4% | -5.2% | -2.7% | -2.4% | - | - | 0.2% | -5.2% | 0.2% | -5.2% | - | - | - | - | - | - |
| YUV, camera, 1080p | -0.4% | -2.3% | -0.7% | -3.3% | -2.3% | -1.2% | - | - | 0.1% | -0.1% | 0.1% | -0.1% | - | - | - | - | - | - |
| Enc Time(%) | 103% |  | 115% |  | - |  | 101% |  | 145% |  | 142% |  | 141% |  | 143% |  | 186% |  |
| Dec Time(%) | 99% |  | 103% |  | - |  | 102% |  | 98% |  | 99% |  | 97% |  | 94% |  | 97% |  |
| RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | -0.1% | -0.2% | -0.5% | -0.9% | -0.5% | -0.9% | -0.1% | 0.0% | -6.3% | -2.8% | -10.2% | -3.2% | -2.0% | -1.4% | -1.3% | -1.2% | -2.7% | -2.1% |
| RGB, text & graphics,720p | -1.3% | -1.5% | -4.3% | -4.9% | -4.3% | -4.9% | -0.1% | -0.2% | -15.5% | -7.6% | -19.6% | -7.5% | -10.3% | -5.6% | -2.5% | -2.7% | -10.2% | -6.0% |
| RGB, mixed content, 1440p | -1.2% | -1.4% | -2.3% | -2.9% | -2.3% | -2.9% | -0.3% | -0.2% | -23.1% | -10.8% | -26.8% | -10.4% | -16.3% | -9.6% | -1.0% | -1.4% | -16.3% | -9.8% |
| RGB, mixed content, 1080p | -0.7% | -1.1% | -2.0% | -2.7% | -2.0% | -2.7% | -0.1% | 0.0% | -15.5% | -6.1% | -19.3% | -5.7% | -10.5% | -6.1% | -0.6% | -1.2% | -10.4% | -6.2% |
| RGB, Animation, 720p | -1.4% | -2.0% | -3.6% | -4.4% | -3.6% | -4.4% | -0.2% | -0.2% | -21.7% | -15.9% | -25.6% | -13.8% | -14.3% | -11.4% | -1.4% | -4.1% | -13.9% | -11.9% |
| RGB, camera, 1080p | -1.5% | -1.6% | -2.2% | -2.3% | -2.2% | -2.3% | -0.2% | -0.2% | -25.8% | -12.4% | -27.9% | -9.7% | -8.5% | -2.8% | 0.1% | -1.0% | -8.0% | -3.1% |
| YUV, text & graphics, 1080p | -0.2% | -0.5% | -0.6% | -1.5% | -1.1% | -0.9% | - | - | -1.4% | -1.4% | -1.4% | -1.4% | - | - | - | - | - | - |
| YUV, text & graphics,720p | -0.3% | -1.5% | -1.4% | -4.1% | -2.2% | -3.1% | - | - | -1.8% | -2.3% | -1.8% | -2.3% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | 0.0% | -1.2% | -0.5% | -2.8% | -1.2% | -1.5% | - | - | 0.1% | -1.0% | 0.1% | -1.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | -0.1% | -0.8% | -0.6% | -2.1% | -1.1% | -1.0% | - | - | 0.0% | -0.7% | 0.0% | -0.7% | - | - | - | - | - | - |
| YUV, Animation, 720p | -0.2% | -2.1% | -0.6% | -4.0% | -1.5% | -1.7% | - | - | 0.0% | -5.1% | 0.0% | -5.1% | - | - | - | - | - | - |
| YUV, camera, 1080p | -0.2% | -2.0% | -0.3% | -2.6% | -1.6% | -0.4% | - | - | 0.5% | 0.1% | 0.5% | 0.1% | - | - | - | - | - | - |
| Enc Time(%) | 99% |  | 101% |  | - |  | 97% |  | 112% |  | 111% |  | 98% |  | 99% |  | 100% |  |
| Dec Time(%) | 102% |  | 100% |  | - |  | 98% |  | 99% |  | 97% |  | 99% |  | 101% |  | 103% |  |
| LD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | -0.1% | -0.2% | -0.4% | -0.5% | -0.4% | -0.5% | -0.2% | -0.2% | -5.8% | -2.2% | -8.9% | -1.5% | -1.1% | -0.9% | -0.9% | -0.9% | -1.7% | -1.4% |
| RGB, text & graphics,720p | -1.1% | -1.1% | -3.4% | -3.8% | -3.4% | -3.8% | -0.3% | -0.3% | -16.7% | -8.0% | -20.5% | -6.4% | -6.8% | -4.2% | -2.4% | -2.3% | -6.7% | -4.2% |
| RGB, mixed content, 1440p | -0.7% | -0.9% | -1.2% | -1.6% | -1.2% | -1.6% | -0.4% | -0.3% | -24.7% | -10.3% | -28.3% | -7.8% | -13.0% | -7.3% | -0.6% | -1.0% | -12.7% | -7.3% |
| RGB, mixed content, 1080p | -0.4% | -0.6% | -0.6% | -1.1% | -0.6% | -1.1% | -0.3% | -0.4% | -16.6% | -4.9% | -20.4% | -1.8% | -8.6% | -5.1% | -0.3% | -0.8% | -8.2% | -5.0% |
| RGB, Animation, 720p | -0.6% | -0.8% | -1.2% | -1.7% | -1.2% | -1.7% | -0.3% | -0.2% | -20.1% | -12.9% | -24.3% | -8.2% | -5.7% | -5.0% | -0.5% | -1.2% | -5.5% | -5.0% |
| RGB, camera, 1080p | -0.5% | -0.4% | -0.5% | -0.6% | -0.5% | -0.6% | -0.3% | -0.2% | -23.5% | -10.7% | -26.2% | -6.9% | -1.7% | -0.2% | 0.1% | -0.4% | -1.4% | -0.2% |
| YUV, text & graphics, 1080p | 0.0% | -0.1% | -0.3% | -0.9% | -0.9% | -0.1% | - | - | -1.8% | -1.8% | -1.8% | -1.8% | - | - | - | - | - | - |
| YUV, text & graphics,720p | -0.2% | -0.9% | -1.3% | -3.4% | -2.3% | -1.7% | - | - | -2.3% | -3.1% | -2.3% | -3.1% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | -0.1% | -1.1% | -0.3% | -1.9% | -1.0% | 0.4% | - | - | 0.2% | -1.0% | 0.2% | -1.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.1% | -0.3% | -0.1% | -0.8% | -0.7% | 1.1% | - | - | -0.2% | -1.0% | -0.2% | -1.0% | - | - | - | - | - | - |
| YUV, Animation, 720p | -0.2% | -1.2% | -0.1% | -2.0% | -1.0% | 1.3% | - | - | -0.1% | -4.9% | -0.1% | -4.9% | - | - | - | - | - | - |
| YUV, camera, 1080p | -0.1% | -0.6% | 0.0% | -0.8% | -1.3% | 2.4% | - | - | 0.3% | -0.2% | 0.3% | -0.2% | - | - | - | - | - | - |
| Enc Time(%) | 100% |  | 101% |  | - |  | 96% |  | 112% |  | 112% |  | 102% |  | 104% |  | 105% |  |
| Dec Time(%) | 101% |  | 102% |  | - |  | 98% |  | 98% |  | 98% |  | 99% |  | 101% |  | 99% |  |

## Lossless coding results

Table 3 Summarized lossless coding results for full frame IBC

| AI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 YCoCg |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average |
| RGB, text & graphics, 1080p | 0.1% | 0.1% | 0.6% | 0.6% | 0.6% | 0.6% | 0.0% | 0.0% | 0.6% | 0.5% | 1.7% | 1.7% | 0.0% | 0.0% | 0.8% | 0.8% | 0.8% | 0.8% |
| RGB, text & graphics,720p | 0.2% | 0.2% | 0.9% | 0.9% | 0.9% | 0.9% | 0.0% | 0.0% | 0.8% | 0.9% | 0.9% | 1.0% | 0.0% | 0.0% | 0.7% | 0.8% | 0.7% | 0.8% |
| RGB, mixed content, 1440p | 0.3% | 0.2% | 0.8% | 0.8% | 0.8% | 0.8% | 0.0% | 0.0% | 2.0% | 2.0% | 2.1% | 2.1% | 0.0% | 0.0% | 0.6% | 0.5% | 0.6% | 0.5% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.7% | 0.7% | 0.7% | 0.7% | 0.0% | 0.0% | 1.6% | 1.6% | 1.7% | 1.7% | 0.0% | 0.0% | 0.7% | 0.7% | 0.7% | 0.7% |
| RGB, Animation, 720p | 0.2% | 0.2% | 3.5% | 3.5% | 3.5% | 3.5% | 0.0% | 0.0% | 4.3% | 4.3% | 4.3% | 4.3% | 0.0% | 0.0% | 3.2% | 3.2% | 3.2% | 3.2% |
| RGB, camera, 1080p | 0.1% | 0.1% | 0.6% | 0.5% | 0.6% | 0.5% | 0.0% | 0.0% | 0.4% | 0.2% | 0.4% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, text & graphics, 1080p | 0.0% | 0.0% | 0.7% | 0.7% | 0.7% | 0.7% | - | - | 0.4% | 0.4% | 0.4% | 0.4% | - | - | - | - | - | - |
| YUV, text & graphics,720p | 0.1% | 0.1% | 0.7% | 0.6% | 0.7% | 0.6% | - | - | 0.2% | 0.1% | 0.2% | 0.1% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.3% | 0.3% | 0.3% | 0.3% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, Animation, 720p | 0.1% | 0.1% | 2.1% | 2.1% | 2.1% | 2.1% | - | - | 1.6% | 1.6% | 1.6% | 1.6% | - | - | - | - | - | - |
| YUV, camera, 1080p | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| Enc Time(%) | 104% |  | 112% |  | 112% |  | 99% |  | 146% |  | 147% |  | 98% |  | 141% |  | 139% |  |
| Dec Time(%) | 101% |  | 100% |  | 100% |  | 98% |  | 101% |  | 102% |  | 102% |  | 102% |  | 102% |  |
| RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | 0.0% | 0.0% | 0.4% | 0.3% | 0.4% | 0.3% | -0.1% | 0.1% | 0.8% | 0.3% | 1.3% | 1.1% | 0.0% | 0.0% | 0.6% | 0.4% | 0.6% | 0.4% |
| RGB, text & graphics,720p | 0.2% | 0.2% | 1.0% | 0.9% | 1.0% | 0.9% | 0.0% | 0.0% | 0.7% | 1.0% | 0.7% | 1.1% | 0.0% | 0.0% | 0.6% | 0.6% | 0.6% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% | 1.1% | 1.1% | 1.1% | 1.1% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% | 0.4% | 0.4% | 0.4% | 0.4% | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% | 0.0% | 0.0% | 1.5% | 1.5% | 1.5% | 1.5% | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, camera, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% | -0.2% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics, 1080p | 0.0% | 0.0% | 0.3% | 0.4% | 0.3% | 0.4% | - | - | 0.4% | 0.3% | 0.4% | 0.3% | - | - | - | - | - | - |
| YUV, text & graphics,720p | 0.1% | 0.1% | 0.8% | 0.5% | 0.8% | 0.5% | - | - | 0.5% | 0.2% | 0.5% | 0.2% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% | - | - | 0.3% | 0.3% | 0.3% | 0.3% | - | - | - | - | - | - |
| YUV, camera, 1080p | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| Enc Time(%) | 100% |  | 101% |  | 101% |  | 99% |  | 119% |  | 122% |  | 100% |  | 103% |  | 102% |  |
| Dec Time(%) | 101% |  | 99% |  | 99% |  | 97% |  | 98% |  | 101% |  | 103% |  | 102% |  | 100% |  |
| LD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | 0.0% | 0.0% | 0.4% | 0.2% | 0.4% | 0.2% | -0.1% | 0.1% | 0.7% | 0.2% | 1.2% | 0.9% | 0.0% | 0.0% | 0.5% | 0.4% | 0.5% | 0.4% |
| RGB, text & graphics,720p | 0.2% | 0.1% | 0.9% | 0.8% | 0.9% | 0.8% | 0.0% | 0.0% | 0.7% | 1.1% | 0.7% | 1.1% | 0.0% | 0.0% | 0.5% | 0.6% | 0.5% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 1.1% | 1.1% | 1.1% | 1.1% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 1.5% | 1.5% | 1.5% | 1.5% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics, 1080p | 0.0% | 0.0% | 0.2% | 0.3% | 0.2% | 0.3% | - | - | 0.5% | 0.3% | 0.5% | 0.3% | - | - | - | - | - | - |
| YUV, text & graphics,720p | 0.0% | 0.0% | 0.8% | 0.4% | 0.8% | 0.4% | - | - | 0.6% | 0.2% | 0.6% | 0.2% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.3% | 0.3% | 0.3% | 0.3% | - | - | - | - | - | - |
| YUV, camera, 1080p | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| Enc Time(%) | 100% |  | 101% |  | 101% |  | 99% |  | 109% |  | 108% |  | 100% |  | 104% |  | 102% |  |
| Dec Time(%) | 101% |  | 99% |  | 99% |  | 100% |  | 101% |  | 99% |  | 101% |  | 102% |  | 101% |  |

Table 4 Summarized lossless coding results for 2 CTU IBC

| AI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 YCoCg |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average | Total | Average |
| RGB, text & graphics, 1080p | 0.1% | 0.1% | 0.6% | 0.6% | 0.6% | 0.6% | 0.0% | 0.0% | 0.5% | 0.3% | 1.3% | 1.4% | 0.0% | 0.0% | 0.7% | 0.8% | 0.7% | 0.8% |
| RGB, text & graphics,720p | 0.2% | 0.2% | 0.9% | 0.9% | 0.9% | 0.9% | 0.0% | 0.0% | 0.9% | 0.9% | 1.0% | 1.0% | 0.0% | 0.0% | 0.7% | 0.8% | 0.7% | 0.8% |
| RGB, mixed content, 1440p | 0.5% | 0.4% | 0.9% | 0.9% | 0.9% | 0.9% | 0.0% | 0.0% | 1.7% | 1.6% | 1.7% | 1.7% | 0.0% | 0.0% | 0.5% | 0.5% | 0.5% | 0.5% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.7% | 0.7% | 0.7% | 0.7% | 0.0% | 0.0% | 1.4% | 1.4% | 1.5% | 1.5% | 0.0% | 0.0% | 0.7% | 0.7% | 0.7% | 0.7% |
| RGB, Animation, 720p | 0.2% | 0.2% | 3.5% | 3.5% | 3.5% | 3.5% | 0.0% | 0.0% | 4.3% | 4.3% | 4.3% | 4.3% | 0.0% | 0.0% | 3.2% | 3.2% | 3.2% | 3.2% |
| RGB, camera, 1080p | 0.1% | 0.1% | 0.6% | 0.5% | 0.6% | 0.5% | 0.0% | 0.0% | 0.4% | 0.2% | 0.4% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, text & graphics, 1080p | 0.1% | 0.1% | 0.8% | 0.9% | 0.8% | 0.9% | - | - | 0.4% | 0.5% | 0.4% | 0.5% | - | - | - | - | - | - |
| YUV, text & graphics,720p | 0.1% | 0.1% | 0.8% | 0.6% | 0.8% | 0.6% | - | - | 0.2% | 0.1% | 0.2% | 0.1% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.3% | 0.4% | 0.3% | 0.4% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, Animation, 720p | 0.1% | 0.1% | 2.1% | 2.1% | 2.1% | 2.1% | - | - | 1.6% | 1.6% | 1.6% | 1.6% | - | - | - | - | - | - |
| YUV, camera, 1080p | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| Enc Time(%) | 104% |  | 113% |  | 113% |  | 103% |  | 146% |  | 148% |  | 99% |  | 145% |  | 143% |  |
| Dec Time(%) | 101% |  | 99% |  | 99% |  | 103% |  | 101% |  | 102% |  | 102% |  | 104% |  | 106% |  |
| RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | 0.0% | 0.0% | 0.4% | 0.3% | 0.4% | 0.3% | -0.1% | 0.0% | 0.6% | 0.1% | 0.9% | 0.9% | 0.0% | 0.0% | 0.5% | 0.4% | 0.5% | 0.4% |
| RGB, text & graphics,720p | 0.2% | 0.1% | 1.0% | 0.9% | 1.0% | 0.9% | 0.0% | 0.1% | 0.8% | 1.0% | 0.8% | 1.1% | 0.0% | 0.0% | 0.6% | 0.6% | 0.6% | 0.6% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% | 1.0% | 1.0% | 1.0% | 1.0% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% | 0.0% | 0.0% | 0.4% | 0.4% | 0.4% | 0.4% | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% | 0.0% | 0.0% | 1.5% | 1.5% | 1.5% | 1.5% | 0.0% | 0.0% | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, camera, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% | -0.2% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics, 1080p | 0.0% | 0.0% | 0.3% | 0.5% | 0.3% | 0.5% | - | - | 0.4% | 0.3% | 0.4% | 0.3% | - | - | - | - | - | - |
| YUV, text & graphics,720p | 0.0% | 0.0% | 0.8% | 0.5% | 0.8% | 0.5% | - | - | 0.5% | 0.2% | 0.5% | 0.2% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% | - | - | 0.3% | 0.3% | 0.3% | 0.3% | - | - | - | - | - | - |
| YUV, camera, 1080p | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| Enc Time(%) | 99% |  | 101% |  | 101% |  | 98% |  | 107% |  | 105% |  | 100% |  | 103% |  | 103% |  |
| Dec Time(%) | 102% |  | 103% |  | 103% |  | 98% |  | 95% |  | 94% |  | 101% |  | 98% |  | 97% |  |
| LD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Test 3.1.1 |  | Test 3.1.2 |  | Test 3.1.2 Enc |  | Test 3.1.3 |  | Test 3.2.1 |  | Test 3.2.1 Enc |  | Test 3.2.2 |  | Test 3.2.2 Reorder |  | Test 3.2.2 Both |  |
|  | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V | Y | U&V |
| RGB, text & graphics, 1080p | 0.0% | 0.0% | 0.4% | 0.2% | 0.4% | 0.2% | 0.0% | 0.0% | 0.6% | 0.1% | 0.9% | 0.7% | 0.0% | 0.0% | 0.4% | 0.3% | 0.4% | 0.3% |
| RGB, text & graphics,720p | 0.2% | 0.1% | 0.9% | 0.8% | 0.9% | 0.8% | 0.0% | 0.0% | 0.7% | 1.1% | 0.7% | 1.1% | 0.0% | 0.0% | 0.5% | 0.6% | 0.5% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 1.1% | 1.1% | 1.1% | 1.1% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 1.5% | 1.5% | 1.5% | 1.5% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics, 1080p | 0.0% | 0.0% | 0.3% | 0.4% | 0.3% | 0.4% | - | - | 0.4% | 0.3% | 0.4% | 0.3% | - | - | - | - | - | - |
| YUV, text & graphics,720p | 0.0% | 0.0% | 0.8% | 0.5% | 0.8% | 0.5% | - | - | 0.6% | 0.2% | 0.6% | 0.2% | - | - | - | - | - | - |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.4% | 0.4% | 0.4% | 0.4% | - | - | - | - | - | - |
| YUV, camera, 1080p | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | - | - | 0.0% | 0.0% | 0.0% | 0.0% | - | - | - | - | - | - |
| Enc Time(%) | 98% |  | 101% |  | 101% |  | 99% |  | 104% |  | 107% |  | 100% |  | 104% |  | 103% |  |
| Dec Time(%) | 102% |  | 102% |  | 102% |  | 101% |  | 96% |  | 99% |  | 101% |  | 104% |  | 97% |  |

# References

- H. Yu, R. Cohen, K. Rapaka and J. Xu, “Common conditions for screen content coding tests”, JCTVC-Q1015, Apr. 2014.

- X. Xiu and J. Chen, “HEVC Screen Content Coding Core Experiment 5 (SCCE5): Inter-component prediction and adaptive color transforms”, JCTVC-Q1125, Apr. 2014.

- M. Xu, Z. Ma, W. Wang, H. Yu, “SCCE5: Result of Test 3.3.1 for modified LMChroma”, JCTVC-R0146, June 2014.

- A. Minezawa, K. Miyazawa, S. Sekiguchi, “SCCE5 3.1.2: Extended inter-component prediction”, JCTVC-R0072, June 2014.

- X. Zhang, K. Zhang, J. An, H. Huang, S. Lei, “SCCE5 Test 3.1.3: Adaptive alpha-paramter coding in cross-component prediction”, JCTVC-R0115, June 2014.

- L. Zhang, J. Chen, J. Sole, M. Karczewicz, X. Xiu, Y. He, Y. Ye, “SCCE5 Test 3.2.1: In-loop color-space transform”, JCTVC-R0147, June 2014.

- B. Li, J. Xu, “SCCE5: Result of Test 3.2.2”, JCTVC-R0099, June 2014.

- X. Zhang, R. Cohen, “Improvement of cross-component prediction”, JCTVC-R0219, June 2014.

- L. Zhang, M. Karczewicz, J. Chen, J. Sole, “Non-SCCE5: On index signaling of cross component prediction”, JCTVC-R0178, June 2014.

- L. Zhang, J. Chen, J. Sole, M. Karczewicz, “Non-SCCE5: Deblocking filter for in-loop color-space transform”, JCTVC-R0180, June 2014.