| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014 | Document: JCTVC-R0041 |
| --- | --- |

| Title: | Use of general_profile_space for auxiliary picture profile indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave Hackensack, NJ 07601 USA | Email: | jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

It is proposed to use an unused value of the general_profile_space syntax element to indicate that a layer with a non-zero value of nuh_layer_id in a multi-layer bitstream is otherwise conforming to a profile.  It is asserted that this proposal could enable signaling of conformance points for auxiliary pictures without the need for the changes adopted in JCTVC-Q0078, which include a normative rewriting process, VPS rewriting SEI message and output layer set nesting SEI message.   Conceptually, an additional “Extension” profile would be defined for an existing profile, such that, for example, an Extension Main profile would be defined which allows general_profile_space to be equal to 1, without modifying the existing Main profile.  The “Extension” profiles could also be used to indicate conformance for simulcast enhancement layers.

# Introduction

In HEVC version 1, and in the current specification, it is required that the value of the general_profile_space syntax element in the profile_tier_level( ) syntax structure shall be equal to 0, and that decoder should ignore a bitstream with general_profile_space not equal to 0.  It is proposed to use a value of the general_profile_space syntax element to indicate that a layer with a non-zero value of nuh_layer_id in a multi-layer bitstream is otherwise conforming to a profile.  The intended use of this proposal is to enable conformance signaling of auxiliary pictures, including the case when multiple auxiliary picture layers are present within a bitstream.

At the Valencia meeting, JCTVC-Q0078 was adopted to address conformance signaling of auxiliary pictures.  Several components were included as part of this adoption, including the following, as expressed in the HLS BoG Report JCTVC-Q0223:

- JCTVC-Q0078: mechanism for signaling a profile/tier/level conformance point for auxiliary pictures, including the aspects listed below. […]

  - signaling of additional layer sets

  - description of how profile_tier_level applies

  - define normative rewriting process

  - VPS rewriting SEI message

  - output layer set nesting SEI message

During a teleconference meeting of the AHG on layered coding constraint specifications and capability indications, it was discussed that the current design, including the adoptions from JCTVC-Q0078, does not provide a way to signal conformance of multiple auxiliary picture layers within the same bitstream which use inter-layer prediction.  Especially for multi-view coding, it may be desirable to allow inter-view prediction between auxiliary pictures in the same access unit which are associated with different views, e.g. for depth, alpha, or overlays.  Inter-layer scalable coding of auxiliary picture may also be desirable, e.g. to code alpha at a lower resolution for a base layer and at a higher resolution for a spatial enhancement layer.

It is asserted that the proposed solution is simpler to specify, while accomplishing the main goal of indicating auxiliary picture conformance provided by JCTVC-Q0078, and additionally provides for indicating conformance of multiple auxiliary picture layers, which may be multi-view or scalably coded. It is proposed to remove from the specification the normative rewriting process, the VPS rewriting SEI message, and the output layer set nesting SEI message.

The “Extension” profiles could also be used to indicate conformance for simulcast spatial enhancement layers or view layers.

# Proposal

It is proposed to use a value of the general_profile_space syntax element to indicate that a layer with a non-zero value of nuh_layer_id in a multi-layer bitstream is otherwise conforming to a profile.  Conceptually, an additional “Extension” profile would be defined for an existing profile, such that, for example, an Extension Main profile would be defined which allows general_profile_space to be equal to 1, without modifying the existing Main profile.

Version 1 decoders conforming to Main profile would not be required to decode an Extension Main profile bitstream.  However, it would be straightforward for future decoder implementations to be expanded to support Extension Main profile, in addition to support for Main profile.  Similarly, “Extension” profiles could be defined for other profiles, such as Main 10, Stereo Main, Scalable Main, and the RExt profiles.

For decoders implementing the “Extension” profiles, when general_profile_space equal to 1 parsing of the SPS and PPS would include some syntax parsing modifications which occur when nuh_layer_id of the parameter set NAL unit is greater than 0.  Note, however, that the current design already includes a base_layer_parameter_set_compatibility_flag[ i ] in the VPS VUI which may be used to indicate a constraint that the nuh_layer_id value of the SPS and PPS NAL units must be equal to 0, in which case the parsing of the SPS and PPS is identical to those in the single layer profiles.  Parsing of the VPS and VPS extension would also be required for decoders conforming to the “Extension” profiles.

For bitstreams conforming to the “Extension” profiles corresponding to single layer profiles, i.e. Main, Main 10, constraints would be imposed such that decoding of the slice layer and below would be identical to the associated single layer profile. In the layered extensions specifications, profile, tier, and level constraints are defined for output layer sets, rather than for individual layers, as is the case for the single layer HEVC version 1 profiles.  To indicate conformance with a single layer “Extension” profile, i.e. Extension Main, an output layer set should contain only a single auxiliary picture layer.  With j being the nuh_layer_id value of the auxiliary picture layer, the value of NumDirectRefLayers[ j ] is restricted to be equal to 0 and the value of poc_lsb_not_present_flag[ j]  is restricted to be 0 when general_profile_space is equal to 1.  This restriction is expressed as a constraint on the value of general_profile_space for the layers contained in the output layer set to which the profile_tier_level( ) syntax structure applies.  Without such a restriction, the slice segment header syntax differs for layers with nuh_layer_id > 0 from those with layers with nuh_layer_id equal to 0. The restriction on NumDirectRefLayers requires that the layer not use inter-layer prediction from any other layers.  The restriction on poc_lsb_not_present_flag necessitates the presence in the slice segment header of the slice_pic_order_cnt_lsb, to correspond to HEVC version 1.

When multiple auxiliary picture layers are present in the bitstream, more than one output layer set may be defined.  For example, consider a bitstream containing two views, and overlay content auxiliary pictures associated with each view.  In addition to the output layer sets which contain the primary pictures, a first additional output layer set could contain an overlay content auxiliary picture layer associated with the base view layer with an Extension Main profile indication, and a second additional output layer set could contain both the overlay content auxiliary picture layer associated with the base view layer and the overlay content auxiliary picture associated with an enhancement view layer with an Extension Stereo Main profile indication.

The “Extension” profiles could also be used to indicate conformance for simulcast spatial enhancement layers or view layers.  For example, consider a bitstream containing two simulcast spatial layers.  The lower resolution base layer could conform to Main profile.  An output layer set containing only the higher resolution simulcast enhancement layer could conform to Extension Main profile.  If desired, an additional output layer set containing both layers could be described which would conform to Scalable Main profile.

# Proposed semantics changes

## profile_tier_level( ) syntax structure

general_profile_space specifies the context for the interpretation of general_profile_idc and general_profile_combatibility_flag[ i ] for all values of i in the range of 0 to 31, inclusive. The value of general_profile_space shall be in the range of 0 .. 1 equal to 0 in bitstreams conforming to this version of this Specification. Other values for general_profile_space are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the CVS when general_profile_space is not in the range of 0 .. 1 equal to 0.

For each layer j in the output layer set for which the profile_tier_level( ) syntax structure applies, when (NumDirectRefLayers[ j ] > 0 | | poc_lsb_not_present_flag[ j ] = = 1), the value of general_profile_space shall be equal to 0.

## Annex A

X.X.X Main profile

Bitstreams conforming to the Main profile shall obey the following constraints:

- SPSs shall have general_profile_space equal to 0 only.

- NUHs shall have nuh_layer_id equal to 0 only.

- SPSs shall have chroma_format_idc equal to 1 only.

…

X.X.X Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- SPSs shall have general_profile_space equal to 0 only.

- NUHs shall have nuh_layer_id equal to 0 only.

- SPSs shall have chroma_format_idc equal to 1 only.

…

X.X.X Scalable Main and Scalable Main 10 profiles

The derived sub-bitstream for an output layer set conforming to the Scalable Main or Scalable Main 10 profiles shall obey the following constraints:

- VPSs shall have general_profile_space equal to 0 only.

…

X.X.X Extension Main, Extension Main 10 profiles

Bitstreams conforming to the Extension Main and Extension Main 10 profiles shall obey all constraints of the Main and Main 10 profiles, respectively, with the following modifications

- SPSs shall have general_profile_space in the range of 0 .. 1 only.

- NUHs shall have nuh_layer_id in the range of 0 .. 62 only.

X.X.X Extension Stereo Main profile

The base layer bitstream derived from bitstreams conforming to the Extension Stereo Main shall obey the constraints of the Stereo Main profile with the following exceptions:

–	The base layer bitstream shall obey all constraints of the Extension Main profile specified in subclause X.X.X.

–	SPSs shall have general_profile_space in the range of 0 ..1 only

The derived sub-bitstream for an output layer set conforming to the Extension Stereo Main profile shall obey the constraints of the Stereo Main profile with the following exceptions:

–	VPSs shall have general_profile_space in the range of 0 ..1 only

X.X.X Extension Scalable Main and Extension Scalable Main 10 profiles

The base layer bitstream derived from bitstreams conforming to the Extension Scalable Main or Extension Scalable Main 10 profiles shall obey the constraints of the Scalable Main profile or Scalable Main 10 profile, respectively, with the following exceptions:

–	The base layer bitstream shall obey all constraints of the Extension Main or Extension Main 10 profile, respectively,  specified in subclause X.X.X or X.X.X.

–	SPSs shall have general_profile_space in the range of 0 ..1 only

The derived sub-bitstream for an output layer set conforming to the Extension Scalable Main or Extension Scalable Main 10 profiles shall obey the constraints of the Scalable Main or Scalable Main 10 profile, respectively, with the following exceptions:

–	VPSs shall have general_profile_space in the range of 0 ..1 only

# Relevant syntax in existing MV-HEVC/SHVC drafts

## VPS VUI

base_layer_parameter_set_compatibility_flag[ i ] equal to 1 specifies that the following constraints apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. base_layer_parameter_set_compatibility_flag[ i ] equal to 0 specifies that the following constraints may or may not apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ].

- Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a PPS with nuh_layer_id value equal to 0.

- Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a SPS with nuh_layer_id value equal to 0.

- The values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the active SPS for the layer with nuh_layer_id equal to layer_id_in_nuh[ i ] shall be the same as the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the vps_rep_format_idx[ i ]-th rep_format( ) syntax structure in the active VPS.

## Slice segment header

The sections highlighted in yellow indicate differences in the slice segment header syntax table which can be different for layers with nuh_layer_id > 0 vs layers with nuh_layer_id equal to 0.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| i = 0 |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| discardable_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| cross_layer_bla_flag | u(1) |
| } |  |
| for( i = 1; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( ( nuh_layer_id > 0 && !poc_lsb_not_present_flag[ LayerIdxInVPS[ nuh_layer_id ] ] ) ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && !all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| if( poc_reset_info_present_flag ) |  |
| poc_reset_idc | u(2) |
| if( poc_reset_idc != 0 ) |  |
| poc_reset_period_id | u(6) |
| if( poc_reset_idc = = 3 ) { |  |
| full_poc_reset_flag | u(1) |
| poc_lsb_val | u(v) |
| } |  |
| if( !PocMsbValRequiredFlag && vps_poc_lsb_aligned_flag ) |  |
| poc_msb_val_present_flag | u(1) |
| if( poc_msb_val_present_flag ) |  |
| poc_msb_val | ue(v) |
| while( more_data_in_slice_segment_header_extension( ) ) |  |
| slice_segment_header_extension_data_bit | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).