| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0048 |
| --- | --- |

| Title: | SCCE3:Test B.15 – Scanning indicator for the palette coded CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Draft |  |  |
| Author(s) or Contact(s): | Döne Bugdayci Sansli Jani Lainema Kemal Ugur | Email: | ext-done.bugdayci@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results for the SCCE3 Test B.1 on scanning indicator for the palette coded CUs. It is reported that the proposed method provides objective bitrate reductions of -0.4% and -0.3% against palette anchor, for lossy coding of 1080p text & graphics content with 2 CTU and full Intra Block Copy search ranges, in all intra configuration, respectively. The gains for lossless coding in these categories are -0.3% and -0.3%.

# Introduction

Proposed method signals whether the palette coded CU is scanned in either horizontal or vertical scan order. Proposed method does not change the coding/decoding of the remaining syntax elements of the palette coded CU. However, the reconstruction process is changed in the decoder.

## Syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ){ |  |
| palette_scan_direction_flag[ x0 ][ y0 ] |  |
| palette_coding( x0, y0, nCbS, nCbS ) |  |
| } |  |
| if(( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY)) && ( !palette_mode_flag[ x0 ][ y0 ] ) |  |
| part_mode | ae(v) |
| … |  |

## Semantics

palette_scan_direction_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using vertical scan order whereas equal to 0 means that the current coding unit is coded using the horizontal scan order.

# Coding efficiency results

Full results are provided in the attached Excel sheets. The tables below summarize the Intra coding results with respect to the SCCE3 palette anchor for 2 CTU and full Intra Block Copy search ranges.

Table 1: Intra coding results with respect to the SCCE3 palette anchor for 2 CTU Intra Block Copy search

Table 2: Intra coding results with respect to the SCCE3 palette anchor for full frame Intra Block Copy

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).