| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0049 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0027 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: VPSs with nuh_layer_id greater than 0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes allowing a VPS with a non-zero nuh_layer_id to be delivered to the decoder in addition to a "base" VPS with nuh_layer_id equal to 0, both with the same vps_video_parameter_set_id value. The VPS with a non-zero nuh_layer_id is required to be a semantic superset of the base VPS and may for example include information about additional layers. The VPS with the highest nuh_layer_id value among the VPSs with the same vps_video_parameter_set_id value is activated.

# Introduction

## Related HLS BoG notes

The notes within JCTVC-Q0223 ("BoG report on High Level Syntax") on JCTVC-Q0110 are copied below for convenience:

JCTVC-Q0110 Support for out-of-band signaling in VPS to enable future layer additions [S. Narasimhan, A. Luthra (Arris)]

The current VPS structure in HEVC requires a change to the parameters (VPS and SPS when they are conveyed within the video bitstream) to signal layer specific parameters when a new scalable layer is added to pre-compressed SHVC content. Even though removal of layers without a change to the VPS conveyed within the video bitstream is supported (with an SEI and appropriate text in the specification), addition of new layers without a change to the VPS does not seem to be supported. Earlier contributions (JCTVC-N0048, JCTVC-K0206 and ISO/IEC WG 11/N 12956) highlighted adding new layers at re-distribution points without a change to the parameters (VPS and SPS conveyed within the bitstream) as a required feature to be included in SHVC specification.

Even though it is possible to add new layers at re-distribution points when parameters (VPS and SPS) are conveyed by other means, this would not cover all the use cases in broadcast and streaming. During the initial content coding, parameters may be included within the video bitstream and applications that add new layers may want to convey only the additional parameters by external means while retaining the parameters conveyed within the video bitstream.

An earlier contribution (JCTVC-P0262) proposed adding a flag (VPS external means flag) to the VPS syntax to indicate that parameters for only the new layers (that are added later) are transmitted via external means. MPEG-2 transport streams provide the capability to signal these parameters through a ‘descriptor’ mechanism associated with the new layer-specific video stream carried in a separate PID. This proposal used one of the ‘reserved’ bits in the VPS for this flag.  With this signaling, VPS and SPS conveyed within the video bitstream do not have to be altered at re-multiplexing or re-distribution points when a new layer is added. In the last meeting, there was a suggestion to change the specification text and allow video parameters (VPS and SPS) to be conveyed within the video bitstream that apply to layers that are present in the bitstream and use external means to signal parameters for new layers only.

This contribution presents both the in-band signaling option and a change to section 7.4.2.4.2 to specify that parameters can be conveyed both within the bitstream as well as other means.

Suggestion to allow a VPS with a non-zero layer_id to be sent in addition to a base layer VPS, with the same vps_id as base layer VPS, which is required to include all of the same info as in the base VPS, but may also include info about additional layers.  This suggestion might have impact on SEI messages, which should be checked, especially layers not present.  The decoding process would use the info from the highest layer VPS present in the bitstream. Implications of multiple additional enhan layer VPSs should also be considered.

Further study in the HLS AHG (chaired by M. Hannuksela) encouraged using this suggested method, with distribution of draft versions of the text well in advance of the next meeting.  Additional work outside of JCT (i.e., in MPEG-2 systems) would also be needed, to convey the additional info in the non-base layer VPS.  The BoG recommends adding a mandate to the HLS AHG to study this.

## Purpose of this contribution

This contribution proposes a solution along the outline drafted in the BoG notes above, i.e. to allow a VPS with a non-zero layer_id to be sent in addition to a base layer VPS, with the same vps_id as base layer VPS, which is required to include all of the same info as in the base VPS, but may also include info about additional layers.

# Overview of the proposal

The proposal can be summarized as follows:

- As already required in HEVC version 1, a VPS with nuh_layer_id equal to 0 and with a vps_video_parameter_set_id that is the subject of activation shall be included in the bitstream or provided by external means.

- VPSs with nuh_layer_id greater than 0 may be included in the bitstream or provided by external means.

- If there are two VPSs with different values of nuh_layer_id and with the same value of vps_video_parameter_set_id, the VPS with the greater value of nuh_layer_id is activated.

- It is remarked that the VPS activation was assumed to take place prior to the selection of the target output layer set and therefore the nuh_layer_id values included in the target output layer set cannot be used to choose which VPS out of those having the same vps_video_parameter_set_id value is activated. However, constraints are proposed such that a conforming bitstream shall not require the presence of a VPS NAL unit with a nuh_layer_id value other than those included in the bitstream.

- It is observed that when a layer is added and described in a VPS NAL unit with nuh_layer_id greater than 0, also at least one layer set and at least one output layer set is added. Layer set indices and output layer set indices are referred to in various syntax structures, such as some SEI messages, and decoding processes. To keep them independent of the VPS RBSP that is activated, the following approach was taken in this contribution:

- num_layer_sets_minus1 shall have the same value in all VPS NAL units with the same vps_video_parameter_set_id value. Similarly, num_add_layer_sets shall have the same value in all VPS NAL units with the same vps_video_parameter_set_id value.

- The VPS NAL unit with nuh_layer_id equal to 0 may contain an indicated number of unused layer set indices, specified by the proposed delta_num_effective_layer_sets syntax element in vps_extension( ). Similarly, the proposed delta_num_effective_add_layer_sets syntax element in vps_extension( ) specifies the number of unused indices of additional layer sets.

Consequently, if the feature of VPS NAL units with nuh_layer_id greater than 0 is not going to be exercised in a bitstream, the encoder can set delta_num_effective_layer_sets and delta_num_effective_add_layer_sets (when present) to 0. Consequently, the cost of having the possibility VPSs with nuh_layer_id greater than 0 in bitstreams that are not using the feature is 1 bit or 2 bits (when additional layer sets are present) per VPS.

- Thanks to a) and b) above, the indices of the effective layer sets, additional layer sets, and output layer sets stay unchanged regardless of which VPS NAL unit is activated out of those VPS NAL units that have the same vps_video_parameter_set_id. Consequently, all syntax structures and decoding processes referring to layer set indices or output layer set indices need not be changed much.

- The variable EffectiveLayerSetFlag[ lsIdx ] is derived based on the active VPS and has value 0 if lsIdx is unused in the active VPS and value 1 otherwise. EffectiveLayerSetFlag[ lsIdx ] is used in vps_extension( ) and its child syntax structures to gate the presence layer set related syntax elements. Moreover, EffectiveLayerSetFlag[ lsIdx ] is used to constrain the semantics of syntax structures other than vps_extension( ) so that the syntax may refer to layer sets with EffectiveLayerSetFlag[ lsIdx ] equal to 0 but the semantics are unspecified. This allows e.g. SEI messages that refer to layer sets that are unused if the VPS NAL unit with nuh_layer_id is activated but are used if a VPS NAL unit with nuh_layer_id greater than 0 is activated. Specifically, nesting_op_idx[ i ] of the scalable nesting SEI message and ols_idx_to_vps[ i ] in the sub-bitstream property SEI message are handled this way.

- The target output layer set is selected (by the HRD or by external means) so that EffectiveLayerSetFlag[ lsIdx ] is equal to 1 for the layer set index lsIdx corresponding to the target output layer set.

The proposed layer set and output layer set indexing is illustrated in Figure 1.

Figure 1. Example of two VPSs with the same vps_video_parameter_set_id, where the EL VPS specifies a greater number of effective layer sets and effective additional layer sets.

- A VPS NAL unit with nuh_layer_id layerIdB is required to be a semantic superset of a VPS NAL unit with the same vps_video_parameter_set_id value and with nuh_layer_id layerIdA (such that layerIdA is less than layerIdB). For example, a VPS NAL unit with nuh_layer_id layerIdB may document a greater number of layers, but all layer-specific information in the two VPS NAL units that relates to the same layers shall be identical.

Two alternatives to phrase the constraints are provided in the accompanying specification text (see subclause F.7.4.2.4.2.1):

Option 1: Require the output of the decoding process be identical when the output layer sets of the same index are decoded regardless of which one of the two VPSs is activated.

Option 2: Specify constraints on syntax element and variable values such that the VPS RBSP of layerIdB is a semantic superset of the VPS RBSP of layerIdA. The exact constraints are (unfortunately) relatively complex to phrase, please see the accompanying specification text document for details.

Option 1 has two pages shorter specification text but is less specific and it should be carefully considered whether the constraints implied this way are sufficient. Option 2 is complex to specify and hence there is a risk that some constraints might be incorrectly specified or missing. Option 2 requires careful review for correctness and completeness.

It is remarked that constraints on vps_vui( ) are currently not included in the accompanying specification text. As vps_vui( ) does not affect the decoding process, the same additional constraints on vps_vui( ) are needed in both Option 1 and 2.

- As already required in HEVC version 1, the content of a VPS NAL unit must be identical to that of the active VPS, when the VPS NAL unit is present within a CVS and the active VPS has the same vps_video_parameter_set_id and the same nuh_layer_id. When a VPS NAL unit is present within a CVS and the active VPS has the same vps_video_parameter_set_id and but a different nuh_layer_id, the content of the VPS NAL unit shall be a semantic superset of that in the active VPS (when the active VPS has a smaller nuh_layer_id) or a semantic subset of that in the active VPS (when the active VPS has a greater nuh_layer_id).

- The number of layers (lnp_sei_num_layers_minus1) is proposed to be indicated in the layers not present SEI message rather than the ID of the active VPS. Consequently, the message becomes independent of the fact which VPS NAL unit (of the same vps_video_parameter_set_id) is activated. More than one layers not present SEI message with a different value of lnp_sei_num_layers_minus1 is allowed in the same access unit but it is required that information given for the respective layers in different messages shall be identical. This design allows for example to add a layer at a re-distribution point, add a respective VPS NAL unit additionally documenting the added layer, and also add layers not present SEI messages documenting also the added layer.

- num_views_minus1 is no longer required to be equal to NumViews − 1 in the multiview acquisition information SEI message and in the multiview view position SEI message. This design enables addition of views at a re-distribution point and making the SEI message independent of the VPS NAL unit that is activated out of the VPS NAL units having the same vps_video_parameter_set_id value.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).