| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0050 |
| --- | --- |

| Title: | Non-SCCE1: Intra block copy masking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur Miska Hannuksela | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes a possibility for “masking” areas in blocks predicted with intra block copy mode and substituting all the sample values in a masked area with a single sample value (as proposed earlier in JCTVC-O0351). It is argued this kind of operation helps coding efficiency by being able to provide finer prediction granularity for intra block copy with no significant complexity implications. It is reported that the proposed approach provides objective bitrate reductions of -1.4 %, -1.6 % for All Intra 1080p text and graphics RGB and YUV categories, respectively.

# Introduction

The proposed technique allows to signal a mask for the PUs coded with IntraBC mode. The mask is covering a square area extending from one of the borders of the PU and having a width varying from 1 to 3 sample rows or columns. All the sample values in the masked area are substituted with a single sample value obtained from the middle sample of the inner boundary of the mask. The black squares in Figure 1 illustrate locations of the samples used in the substitution process for different masking modes. Figure 2 shows the masking areas and source sample location for the three different mask sizes in the case of mode 2 (mask on the left border of the block) and block size of 8x8.

Figure 1: Examples of intra block copied CUs with different masking modes 1 through 4.

Figure 2: Examples of intra block copied CUs with different masking alternatives for masking mode 2 (mask on the left border of the block). The mask can extend 1, 2 or three sample rows from the border. The black sample location illustrates the sample which value is used to substitute all other sample values in the masked area (gray).

Motivation for the proposed technique was an observation that the current intra block copy operates occasionally inefficiently due to its fixed size. E.g. it can copy significant residue together with the actually desired content. This kind of behaviour is illustrated in Figure 3 below where part of the letter “a” gets copied together with the letter “c” to the predicted block.

Figure 3: Example of intra block copy operation without intra masking.

# Results

Results below are obtained with a simple encoder search algorithm where a SAD based cost of the luminance intra block copy mode is evaluated for all the available masking modes after the intra block search is performed for a PU. Full results can be found in the attached spreadsheets.

Table 1: Results for lossy All Intra configuration

Table 2: Results for lossless All Intra configuration

Figure 4: Bitrate reductions for different RGB sequences in lossy All Intra configuration.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).