Yellow ground color is the modification of JCTVC-Q0094 + bug-fixes of JCTVC-Q0047.

Pink ground color is the modification of the proposed.

7.3.8.5 Coding unit syntax

| … |  |
| --- | --- |
| palette_coding_component( x0, y0, CbWidth, CbHeight, NumComp ) { | Descriptor |
| compOffset = ( NumComp = = 3 ) ? 0 : ( NumComp – 1 ) |  |
| nCbS = ( 1 << log2CbSize ) |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteSize; i++ ) { |  |
| previous_palette_entry_flag[ i ] | ae(v) |
| if ( previous_palette_entry_flag[ i ] ) { |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ i ] |  |
| numPredPreviousPalette++ |  |
| } |  |
| } |  |
| if( numPredPreviousPalette < max_palette_size) |  |
| palette_num_signalled_entries | ae(v) |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| for( i = 0; i < palette_num_signalled_entries; i++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + palette_num_signalled_entries |  |
| palette_adaptive_escape_flag | ae(v) |
| previous_run_type_flag = INDEX_MODE |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| if ( yC != 0 && previous_run_type_flag != COPY_ABOVE_MODE ) |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| else |  |
| palette_run_type_flag[ xC ][ yC ] = INDEX_MODE |  |
| previous_run_type_flag = palette_run_type_flag[ xC ][ yC ] |  |
| if( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE ) { |  |
| palette_index | ae(v) |
| if( palette_index = = palette_size ) { /* ESCAPE_PIXEL */ |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) { |  |
| palette_escape_val | ae(v) |
| samples_array[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| } |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_ABOVE_MODE ¦ ¦ ( palette_run_type_flag[ xC ][ yC ] = = INDEX_MODE && palette_index < palette_size ) ) { |  |
| palette_run | ae(v) |
| runPos = 0 |  |
| while ( runPos <= palette_run ) { |  |
| paletteMap[ xC ][ yC ] = palette_index |  |
| else |  |
| paletteMap[ xC ][ yC ] = paletteMap[ xC ][ yC − 1 ] |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| samples_array[ cIdx ][ xC ][ yC ] = palette_entries[ cIdx ][ paletteMap[ xC ][ yC ]] |  |
| runPos++ |  |
| scanPos++ |  |
| xC = scanPos % nCbS |  |
| yC = scanPos / nCbS |  |
| } |  |
| } |  |
| } |  |
| previousPaletteSize = palette_size |  |
| for( i = 0; i < palette_size; i++ ) |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| previousPaletteEntries[ cIdx ][ i ] = palette_entries[ cIdx ][ i ] |  |

7.4.3.2 Sequence parameter set RBSP semantics

## Semantics

palette_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using the palette mode. palette_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded using the palette mode.

When palette_mode_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

previous_palette_entry_flag[ i ] equal to 1 specifies that the palette entry from the previous used palette is copied. previous_palette_entry_flag [ i ] equals to 0 specifies that the palette entry from the previous used palette is not copied.

The previousPaletteSize is the size of the palette of the previous CU decoded in palette mode. The variable previousPaletteSize is set equal to 0 for the first decoded palette CU of each CTB line as well as slice.

palette_num_signalled_entries specifies the number of entries in the palette that are explicitly signalled for the current coding unit.

When palette_num_signalled_entries is not present, it is inferred to be equal to 0.

palette_adaptive_escape_flag specifies whether coding of each index reserves a bit for escape index and escape pixels. If equal to 1, it reserves a bit for escape pixel. If equal to 0, it does not reserve a bit for escape pixel.

palette_entries[ cIdx ][ i ] specifies the i-th element in the palette for the color component cIdx.

The array previousPaletteEntries is equal to the array palette_entries of the last CU decoded in palette mode.

palette_escape_val specifies the escape pixel value.

palette_run_type_flag equal to 1 specifies that the decoding process is COPY_ABOVE_MODE where the decoded pixel value is equal to the pixel at the same location in the above row. palette_run_type_flag equal to 0 specifies that the decoding process mode is INDEX_MODE where the pixel’s palette index is coded in the bitstream. .

When palette_run_type_flag is not present, it is inferred to be equal to INDEX_MODE.

palette_index is an index to the palette entries. If palette_index is equal to palette_size, the pixel is coded as ESCAPE_PIXEL.

palette_run the number of consecutive locations minus 1 with the same palette index as the position in the above row when palette_run_type_flag is equal to COPY_ABOVE_MODE or represents the number of consecutive locations minus 1 with the same palette index when palette_run_type_flag is equal to INDEX_MODE.