| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0056 |
| --- | --- |

| Title: | Restriction of CU sizes for Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonghyun Ma, Hyunho Jo, Ismail Marzuki, Yongjo Ahn, Woong Lim, Donggyu Sim 507 Bima-gwan, 20 Gwagwun-ro Nowon-gu, Seoul, South Korea | Tel: Email: | +82-2-940-5470 {mday519, idjhh, ismail, madein1st, limwoong, dgsim}@kw.ac.kr |
| Source: | KWU (Kwangwoon University) |  |  |

_____________________________

# Abstract

This document presents IBC syntaxes to reduce amount of on/off flags for intra block copy (Intra BC) coded on each CU-level. In the proposed algorithm, two syntax elements are added into the sequence parameter set (SPS) syntax to indicate the minimum and maximum block sizes of Intra BC. Based on these two syntax elements, the intra block copying mode is performed only in the specified block sizes, and an on/off Intra BC flag is also coded for the CU blocks. Experimental results show that the average BD-rates of the proposed algorithm are -0.1%, 0.0%, and 0.0% for All intra (AI), Random access (RA), and Low-delay B (LB) under the common test conditions for screen content coding, respectively.

# Introduction

In the current HEVC screen content coding (SCC) extension, intra block copy (Intra BC) is employed to efficiently encode screen content videos. Intra BC is a block matching technique that predicts repetitive patterns in a picture and this technique is performed on each coding unit (CU)-level.

For indicating whether a CU is coded by intra block copying mode or not, an on/off flag for Intra BC is coded on each CU-level, as shown in Fig. 1. If this flag is 1 for a CU, it is coded by the intra block copying mode. If the flag is 0, the CU is coded by the intra mode or inter mode.

Figure 1: Example of on/off flags of Intra BC coded for a CU partition blocks

Coding efficiency with intra block copying mode could vary depending on properties of sequences. In addition, selected dominant block sizes for Intra BC may vary depending on different sequences. Based on the fact, the intra block copying mode can be selectively performed on limited CU-levels, depending on the sequences. However, for current SCM-1.0, the on/off flags for Intra BC are coded on all the CU-levels. Thus, the amount of the on/off flags for Intra BC by restricting the block size of Intra BC can be reduced.

# Proposed method

In the proposed method, two syntax elements are added in the sequence parameter set (SPS) syntax for restricting the CU block sizes for Intra BC. One element is for specifying the minimum block size of Intra BC and the other one is for specifying the maximum block size. With these two syntax elements, on/off flags for Intra BC are coded only for the restricted block sizes.

Figure 2: Syntax flow and decoding process of the proposed algorithm

The decoding syntax flow and decoding process of Intra BC for the proposed algorithm is as shown in Fig. 2. CU size means the current decoding CU size, and the min Intra BC size and max Intra BC size indicate the min and max restricted sizes for intra block copying mode, respectively. The decoding flow of the proposed algorithm starts from comparing the CU size with the min/max Intra BC size. If the current CU size is in the range of min/max Intra BC sizes, the Intra BC flag is decoded. Otherwise, the process of parsing Intra BC flag is skipped. Based on this method, the on/off flag for Intra BC can be adaptively coded by modifying the min/max Intra BC block size.

# Syntax and semantics description

## Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_intra_bc_size_minus3 | ue(v) |
| log2_diff_max_intra_bc_size | ue(v) |
| … |  |
| } |  |

log2_min_intra_bc_size_minus3 plus 3 specifies the minimum block size of Intra BC.

log2_diff_max_intra_bc_size specifies the difference between the maximum and minimum block size of Intra BC.

## Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag && log2CbSize <= log2MaxIntraBCSize && log2CbSize >= log2MinIntraBCSize) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| … |  |
| } |  |

The variable log2MinIntraBCSize and log2MaxIntraBCSize is derived as follows:

log2MinIntraBCSize = log2_min_intra_bc_size_minus3 + 3
	log2MaxIntraBCSize = log2MinIntraBCSize + log2_diff_max_intra_bc_size

# Experimental results

The proposed method was evaluated under the common test conditions for screen content coding [1], and it is integrated on HM-14.0+RExt-7.0+SCM-1.0 [2]. For this evaluation, the minimum block size and maximum block size of Intra BC are set to 8x8 and 16x16, respectively.

Table 1. Bitrate saving for the lossless coding mode

Table 2. BD rate of the lossy coding mode

# Conclusion

Restriction of CU sizes for Intra BC is proposed to reduce amount of on/off flags for Intra BC. Minor coding gain is observed in the experimental results when the min and max Intra BC block sizes are given with 8x8 and 16x16, respectively. However, coding performance of the proposed method can be improved by adaptively modifying the min/max Intra BC block sizes, depending on sequences.

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests, “ JCTVC-Q1015-v2, 17th JCT-VC meeting, Valencia, May., 2014.

- HM-14.0+RExt-7.0+SCM-1.0 (SCM-1.0), https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-1.0

# Patent rights declaration(s)

KWU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).