| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0057 |
| --- | --- |

| Title: | SCCE3 Test B.4: Adaptive color index map scan |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun1, Tzu-Der Chuang1, Yi-Wen Chen1, Xianguo Zhang2, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yc.sun, peter.chuang, yiwen.chen, xianguo.zhang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a new flag for a palette coded CU to indicate horizontal or vertical color index scan. Experiments show that compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047, BD-rates are -0.4%, -0.2%, and -0.1% for “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and are also -0.4%, -0.2%, and -0.1% for “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively. As supplemental information, when the test sequences are rotated by 90 degree to represent cases of rotating smartphone or tablet screens or cases of text that may be written column by column (e.g., Chinese, Japanese, …), the corresponding BD-rates become -3.4% and –3.5% for RGB and YUV under AI, respectively.

# Introduction

In the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 [1], the color indices in a CU are coded in fixed horizontal scan. However, horizontal color index scan is not suitable for all content. Therefore, the contribution proposes to add vertical scan in addition to horizontal scan. A new vertical_scan_flag is added in a palette coded CU to indicate horizontal and vertical scan.

# Proposed methods

For a palette coded CU, a vertical_scan_flag is signaled. If the flag is equal to 1, the color indices in a CU are coded in vertical scan. Otherwise, the indices are coded in horizontal scan.

In practical, a simple implementation of the proposed CU-adaptive color index scan method can be: if vertical scan is used, the decoder can simply swap the sample position axes for the color index map. In other words, samples_array[ x0 ][ y0 ] originally means x position is x0 and y position is y0. If the flag is equal to 1, it means y position is x0 and x position is y0, and the color index becomes samples_array[ y0 ][ x0 ].

# Experimental results

The proposed method is implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 [1]. The tests were conducted under the common test conditions in SCCE3 [1].

## SCCE3 test condition 1

For test condition 1, experiments are conducted with intraBC using full frame search. Table 1 shows the lossy results of the proposed method using Anchor 1-1 in [1]. Table 2 shows lossless results of the proposed method using Anchor 1-1.

Experiments show that, compared with Anchor 1-1, -0.3%, -0.1%, and -0.1% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and -0.4%, -0.2%, and -0.1% bit-rate saving is observed on “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

The results using Anchor 1-2 in [1] are shown in the attached excel files, JCTVC-R0057_TestB.4_FF_ref=scm.xlsm and JCTVC-R0057_TestB.4_FF_ref=scm_qp0.xlsm.

## SCCE3 test condition 2

For test condition 2, experiments are conducted with intraBC using 2-CTU search. Table 3 shows the lossy results of the proposed method using Anchor 2-1 in [1]. Table 4 shows the lossless results of the proposed method using Anchor 2-1

Experiments show that, compared with Anchor 2-1, -0.4%, -0.2%, and -0.1% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and -0.4%, -0.2%, and -0.1% bit-rate saving is observed on “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

The results using Anchor 2-2 in [1] are shown in the attached excel files, JCTVC-R0057_TestB.4_2CTU_ref=scm.xlsm and JCTVC-R0057_TestB.4_2CTU_ref=scm_qp0.xlsm.

## Test condition with sequences rotated by 90 degree

We found that horizontal scan order is suitable for most contents in the sequences in current test condition. Therefore, current test condition sequences cannot demonstrate the importance of the proposed method.

We conduct simple experiments: the test sequences are rotated by 90 degree to represent cases of rotating smartphone or tablet screens or cases of text that may be written column by column (e.g., Chinese, Japanese, …). The setting of frame width and height are switched accordingly. All of the other settings of test conditions are all the same to [1]. Table 5 shows the results of the proposed method with intraBC using 2-CTU search using Anchor 2.1.

Experiments show that, compared with Anchor 2-1, -3.4% and –3.5% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” and “YUV, text and graphics with motion, 1080p” sequences under AI, respectively.

# Conclusion

In this contribution, an adaptive scan order method is proposed and implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. Experiments show that compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047, BD-rates are -0.4%, -0.2%, and -0.1% for “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and are also -0.4%, -0.2%, and -0.1% for “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1123, April, 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, April, 2014.

Table 1  Lossy results of the proposed method under SCCE3 test condition 1. Anchor is Anchor 1-1.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.3% |
| YUV, text & graphics with motion,720p | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.2% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.1% | -0.2% | -0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | 0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | -0.4% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | -0.2% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | -0.5% | -1.5% | -0.4% |
| YUV, Animation, 720p | -0.1% | 0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 95% |  |  |

Table 2  Lossless results of the proposed method under SCCE3 test condition 1. Anchor is Anchor 1-1.

Table 3  Lossy results of the proposed method under SCCE3 test condition 2. Anchor is Anchor 2-1.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.2% | 0.0% | -0.2% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.5% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | 0.4% |
| YUV, mixed content, 1440p | -0.2% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.6% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 97% |  |  |

Table 4  Lossless results of the proposed method under SCCE3 test condition 2. Anchor is Anchor 2-1.

Table 5  Lossy results of the proposed method under SCCE3 test condition 2 with rotated test sequences. Anchor is Anchor 2-1.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -3.4% | -3.4% | -3.4% |
| RGB, text & graphics with motion,720p | -3.0% | -2.8% | -2.9% |
| RGB, mixed content, 1440p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p |  |  |  |
| YUV, text & graphics with motion, 1080p | -3.5% | -4.0% | -3.9% |
| YUV, text & graphics with motion,720p | -2.5% | -3.1% | -3.4% |
| YUV, mixed content, 1440p | -0.6% | -1.6% | -1.6% |
| YUV, mixed content, 1080p | -0.7% | -1.5% | -1.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p |  |  |  |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |