| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, Japan, 30 June – 09 July 2014 | Document: JCTVC-R0060 |
| --- | --- |

| Title: | SCCE4 : Results of subtest 3.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liping Zhao, Xianyi Chen, Tao Lin Jing Ye, Shan Liu, Shawmin Lei 1239 Siping Ave, Shanghai 200092, China 2860 Junction Ave, San Jose, CA 95134, USA | Email: | lintao@tongji.edu.cn jing.ye@mediatek.com shan.liu@mediatek.com |
| Source: | Tongji University, MediaTek Inc |  |  |

_____________________________

# Abstract

This document reports the experimental results of subtest 3.3 of HEVC Screen Content Core Experiment 4 String matching for sample coding, as described in document JCTVC-Q1124. The proposed implementation example using HM-14.0+RExt-7.0+SCM-1.0 as anchor reports -14.1%, -9.1%, and -5.9% BD-rate savings for SC RGB sequences under AI, RA, and LB, respectively, -14.5%, -9.2%, and -4.9% BD-rate savings for YUV TGM 1080p sequences under AI, RA, and LB, respectively, 26.1%, 17.1%, and 13.6% lossless bit-rate savings (average) for YUV TGM 1080p sequences under AI, RA, and LB, respectively.

# Introduction

String matching is first officially proposed by Tongji University [1] in the 11th JCT-VC meeting. In the 17th JCT-VC meeting, in April 2014 Valencia, a Screen Content Core Experiment 4(SCCE4) is defined to further study string matching. “Improvements on 1D Dictionary coding [3] is Test 3.3 in SCCE4.

The prior string matching is implemented in LCU level, and it is observed that string matching implemented in CU level could bring some benefits, such as better coding performance, flexibility and compatibility.

The existing LCU level string matching [1],[2] and CU level string matching [3] limit the maximum length (len) of a matching string to 273, which hurts the performance of string matching.

String matching uses 1-D distance (pos) to represent 2-D displacement between the current string and the reference string with the basic unit of sample instead of pixel. This technique has many benefits but also makes the matching string distance and the matching string length to have the unusual characteristic that many special and large values such as multiples of 3 and multiples of the current CU width occur much more frequently than many other small values. On the other hand, the existing HEVC entropy coding binarization usually has short binary code for small values and long binary code for large values. Thus, directly applying binarization to the two matching parameters results in poor entropy coding performance.

The existing string matching technique uses exact matching, so lossy coding performance improvement is much less than lossless coding.

To address these issues, in this document, a few enhancements to the current string matching technique are proposed: CU level string matching, extending maximum matching string length (len) to 12288, a bijective mapping for the whole value range of the two matching parameters (pos and len), color quantization pre-processing.

# Proposed enhancements

## CU level string matching

For a CU coded by string matching mode (P2M mode), a flag “cu_coded_by_p2m” is used to indicate the P2M mode. The searching for matched distance and length uses CU as the basic unit. The searching range (reference pixel sample range) is determined by an encoding parameter shown in Table 1.

| Level | posRange (byte) |
| --- | --- |
| 1 | 4096 |
| 2 | 8192 |
| 3 | 16384 |
| 4 | 65536 |
| 5 | 262144 |
| 6 | 1048576 |
| 7 | 4194304 |

Table1. Level and size (posRange) of reference pixel sample range.

Experimental results show that level 4 (64KB reference range) is a good tradeoff between coding performance and coding time/complexity.

The SPS syntax is modified as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| <snip ……> |  |
| intra_p2m_enabled_flag | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| <snip ……> |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The cu syntax is modified as follows:

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if(intra_p2m_enabled_flag ) |  |
| intra_p2m_flag[ x0 ][ y0 ] | ae(v) |
| if(intra_p2m_flag[ x0 ][ y0 ] ) |  |
| p2m_coder( x0, y0, log2CbSize ) |  |
| } |  |
| if( !intra_p2m_flag [ x0 ][ y0 ] ){ |  |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ]) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

p2m_coder( x0, y0, log2CbSize ) {

decoded_sample_count = 0

NumSamplesInCU = (3 << (log2CbSize << 1))

while(decoded_sample_count < NumSamplesInCU) {

matching_string_flag

if(matching_string_flag = = 1) {

unmatchable_sample

decoded_sample_count += 1

}

else {

matching_string_distance_use_recent_8_flag

if(matching_string_distance_use_recent_8_flag = = 1) {

matching_string_distance_recent_8_idx

matching_string_distance_minus1 = Recent8Distances[matching_string_distance_recent_8_idx]

for( i = matching_string_distance_recent_8_idx; i > 0; i-- )

Recent8Distances[i] = Recent8Distances[i-1]

Recent8Distances[0] = matching_string_distance_minus1

}

else {

matching_string_distance_minus1

for( i = 7; i > 0; i-- )

Recent8Distances[i] = Recent8Distances[i-1]

Recent8Distances[0] = matching_string_distance_minus1

}

matching_string_length_minus2

decoded_sample_count += (matching_string_length_minus2 + 2)

}

}

}

## Extending maximum matching string length

The maximum matching string length is extended from 273 to 12288, which is the total number of sample in an LCU.

## Bijective mapping for the whole value range of pos and len

A bijective mapping for the whole value range of len and a bijective mapping for the whole value range of pos are applied to len and pos, respectively. Then binarization is applied to the mapped len and pos. The mapping maps more frequently occurred values to smaller values. Thus, more frequently occurred values will be assigned shorter binary code to improve the entropy coding performance and the overall performance of string matching tool. The mappings are given in the following tables.

| Mapping of pos for depth = 3 |  |
| --- | --- |
| Pre-map value | Post-map value |
| 3 | 0 |
| 21, 24, 27 | 1, 2, 3 |
| 48 | 4 |
| 12, 15, 18 | 5, 6, 7 |
| 72, 96 | 8, 9 |
| 6, 9 | 10, 11 |
| 30, 33, 36, 39 | 12, 13, 14, 15 |
| 120, 144, ……, 216, 240 | 16, 17, ……, 20, 21 |
| 42, 45, 51, 54, ……, 66, 69, 75 | 22, 23, 24, 25, ……, 29, 30, 31 |
| 264,288,……,360,384 | 32,33,……,36,37 |
| 78,81,……,90,93, | 38,39,……,42,43 |
| 99,102,……,114,117 | 44,45,……,49,50 |
| 123,126,……,138,141 | 51,52,……,56,57 |
| 147,150,……,159,162 | 58,59,……,62,63 |
| 408,432,……,960,984 | 64,65,……,87,88 |
| 165,171,174,……,186,189 | 89,90,91,……,95,96 |
| 195,198,……,210,213 | 97,98,……,102,103 |
| 219,222,……,237,243 | 104,105,……,110,111 |
| 246,249,……,258,261 | 112,113,……,116,117 |
| 267,270,……,285,291,294,297 | 118,119,……,124,125,126,127 |
| 300,303,……,978,981(except multiple of 24) | 128,129,……,326,327 |
| 987,990,……,posRange-3, posRange | 328,329,……, posRange/3-2, posRange/3-1 |
| 1,4,……, posRange-5, posRange-2 | posRange/3, posRange/3+1, ……,posRange*2/3-2, posRange*2/3-1 |
| 2,5,……, posRange-4, posRange-1 | posRange*2/3, posRange*2/3+1, ……posRange, posRange-1 |

| Mapping of pos for depth = 2 |  |
| --- | --- |
| Pre-map value | Post-map value |
| 3 | 0 |
| 45, 48, 51 | 1, 2, 3 |
| 96 | 4 |
| 12, 15, 18 | 5, 6, 7 |
| 144, 54 | 8, 9 |
| 6, 9 | 10, 11 |
| 21, 24, 27, 30 | 12, 13, 14, 15 |
| 192,240 | 16,17 |
| 87,90,93 | 18,19,20 |
| 33,36,39,42 | 21,22,23,24 |
| 57,60,……,72,75 | 25,26,……,30,31 |
| 288,336,384 | 32,33,34 |
| 78,81,84 | 35,36,37 |
| 99,102,……,138,141 | 38,39,……,51,52 |
| 147,150,……,174,177 | 53,54,……,62,63 |
| 432,480,……,1448,1536 | 64,65,……,86,87 |
| 180,183,186,189 | 88,89,90,91 |
| 195,198,…..,234,237 | 92,93,……,105,106 |
| 243,246,……,282,285 | 107,108,……,120,121 |
| 291,294,……,303,306 | 122,123,……,126,127 |
| 309,312,……,1530,1533(except multiple of 48) | 128,129,……,510,511 |
| 1539,1542,……,posRange-3, posRange | 512,513,……, posRange/3-2, posRange/3-1 |
| 1,4,……, posRange-5, posRange-2 | posRange/3, posRange/3+1, ……,posRange*2/3-2, posRange*2/3-1 |
| 2,5,……, posRange-4, posRange-1 | posRange*2/3, posRange*2/3+1, ……posRange, posRange-1 |

| Mapping of pos for depth = 1 |  |
| --- | --- |
| Pre-map value | Post-map value |
| 3 | 0 |
| 93, 96, 99 | 1, 2, 3 |
| 192 | 4 |
| 90, 18, 21 | 5, 6, 7 |
| 288,384 | 8, 9 |
| 6, 9 | 10, 11 |
| 87, 12, 15, 24 | 12, 13, 14, 15 |
| 480 | 16 |
| 27,30,……,66,69 | 17,18,……,30,31 |
| 576,672,768 | 32,33,34 |
| 72,75,……,81,84 | 35,36,……,38,39 |
| 102,105,……,168,171 | 40,41,……,62,63 |
| 864,960,……,3072,3168 | 64,65,……,87,88 |
| 174,177,……,186,189 | 89,90,……,93,94 |
| 195,198,……,282,285 | 95,96,……,124,125 |
| 291,294 | 126,127 |
| 297,300,……,3162,3165(except multiple of 96) | 128,129,……,1054,1055 |
| 3171,3174,……,posRange-3, posRange | 1056,1057,……, posRange/3-2, posRange/3-1 |
| 1,4,……, posRange-5, posRange-2 | posRange/3, posRange/3+1, ……,posRange*2/3-2, posRange*2/3-1 |
| 2,5,……, posRange-4, posRange-1 | posRange*2/3, posRange*2/3+1, ……posRange, posRange-1 |

| Mapping of pos for depth = 0 |  |
| --- | --- |
| Pre-map value | Post-map value |
| 3 | 0 |
| 192,384,189 | 1, 2, 3 |
| 576 | 4 |
| 12,15,18 | 5, 6, 7 |
| 768 | 8 |
| 6, 9 | 9 ,10 |
| 195 | 11 |
| 21,24,27,30 | 12,13,14,15 |
| 960,1152 | 16,17 |
| 183,186,198 | 18,19,20 |
| 33,36,……,60,63 | 21,22,……,30,31 |
| 1344,1536 | 32,33 |
| 66,69,……,150,153 | 34,35,……,62,63 |
| 1728,1920,……,3648,3840 | 64,65,……,74,75 |
| 156,159,……,177,180 | 76,77,……,83,84 |
| 201,204,……,324,327 | 85,86,……,126,127 |
| 330,333,……,3834,3837(except multiple of 192) | 128,129,……,1278,1279 |
| 3843,3846,……,posRange-3, posRange | 1280,1281,……, posRange/3-2, posRange/3-1 |
| 1,4,……, posRange-5, posRange-2 | posRange/3, posRange/3+1, ……,posRange*2/3-2, posRange*2/3-1 |
| 2,5,……, posRange-4, posRange-1 | posRange*2/3, posRange*2/3+1, ……posRange, posRange-1 |

| Mapping of length (all depths) |  |
| --- | --- |
| Pre-map value | Post-map value |
| 3,6,……,45,48 | 0,1,……, 14,15 |
| 2,5……,20,23 | 16,17,……,22,23 |
| 51,54,……,69,72 | 24,25,……,30,31 |
| 4,7,……,22,25 | 32,33,……,38,39 |
| 75,78,……,93,96 | 40,41,……,46,47 |
| 26,29,……,44,47 | 48,49,……,54,55 |
| 28,31,……,46,49 | 56,57,……,62,63 |
| 99,102,……,141,144 | 64,65,……,78,79 |
| 50,53,……,68,71 | 80,81,……,86,87 |
| 52,55,……70,73 | 88,89,……94,95 |
| 147,150,……189,192 | 96,97,……,110,111 |
| 74,77,……,92,95 | 112,113,……118,119 |
| 76,79,……94,97 | 120,121,……,126,127 |
| 98,101,……,140,143 | 128,129,……,142,143 |
| 100,103,……,142,145 | 144,145,……,158,159 |
| 195,198,……,237,240 | 160,161,……,174,175 |
| 146,149,……,188,191 | 176,177,……,190,191 |
| 148,151,……,190,193 | 192,193,……,206,207 |
| 243,246,……,285,288 | 208,209,……,222,223 |
| 291,294,……,333,226 | 224,225,……,238,239 |
| 194,197,……,236,239 | 240,241,……254,255 |
| 196,199,……,238,241 | 256,257,……,270,271 |
| 339,342,……,621,624 | 272,273,……,366,367 |
| 627,630,……,1005,1008 | 368,369,……,494,495 |
| 242,245,……,620,623 | 496,497,……,622,623 |
| 244,247,……,622,625 | 624,625,……,750,751 |
| 1011,1014,……,12285,12288 | 752,753,……,4510,4511 |
| 626,629,……,1124,1127 | 4512,4513,……,4678,4679 |
| 628,631,……,1126,1129 | 4680,4681,……,4846,4847 |
| 1130,1133,……,12284,12287 | 4848,4849,……, 8566,8567, |
| 1132,1135,……,12283,12286 | 8568,8569,……,12285,12286 |

## Color quantization (CQ) pre-processing

A histogram-based qp-dependent CQ scheme similar to the one to select major-color and escape-color in palette coding is used as a pre-processing step of the input pixels.

## Memory bandwidth merit of 1D search (reconstructed pixel) range over 2D search (reconstructed pixel) range

Each 1-D reference string occupies continuous addressing unit, while each 2-D reference string is fragmented into multiple pieces of addressing unit. 1-D string matching method has much less (typically 70% ~ 50% less in DDR case and 50% ~ 30% less in SRAM case) memory bandwidth demand than 2-D string matching.

It was also observed that the number of strings (runs) in a CU is typically less than 10% of the number of pixels. Therefore, by imposing a constraint on the number of string per CU or per CTU, the worst case memory bandwidth can be reduced.

# Simulation results

As an implementation example, the proposed methods were tested using HM-14.0+RExt-7.0+SCM-1.0 as anchor. All the results were generated under the common test conditions and software reference configurations defined in Q1015 for SCC. The encoding and decoding runtime is not very accurate.

The experimental results show that 64KB search range (5.3 CTUs) captures most gain, 96% in lossy case and 90% in lossless case.

# Conclusions

This document presents CU level P2M with new matching parameter encoding method and color quantization. Results on HM-14.0+RExt-7.0+SCM-1.0 as the anchor demonstrate -14.1%, -9.1%, and -5.9% BD-rate savings for SC RGB sequences under AI, RA, and LB, respectively, -14.5%, -9.2%, and -4.9% BD-rate savings for YUV TGM 1080p sequences under AI, RA, and LB, respectively, 26.1%, 17.1%, and 13.6% lossless bit-rate savings (average) for YUV TGM 1080p sequences under AI, RA, and LB, respectively. String matching and copying method still has significant room for improvement. We suggest further study in CE and TE.

# References

[1]. T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG7: Full-chroma (YUV444) dictionary+hybrid dual-coder extension of HEVC”, JCT-VC Document, JCTVC-K0133, Shanghai, China, October 2012.

[2]. Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou, “AHG8: P2M based dual-coder extension of HEVC”, JCT-VC Document, JCTVC-L0303, Geneva, Switzerland,14–23 Jan. 2013.

[3]. Jing Ye, Shan Liu, Shawmin Lei, Xianyi Chen, Liping Zhao, Tao Lin, “Improvements on 1D dictionary coding”, JCT-VC Document, JCTVC-Q0124, Valencia, ES, 27 March – 4 April 2014

# Patent rights declaration(s)

MediaTek Inc. Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).