| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0063 |
| --- | --- |

| Title: | SCCE3: Test A.8 – Improvements on Palette Prediction Vector Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Wei Pu Vadim Seregin Rajan Joshi Joel Sole Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 martak@qti.qualcomm.com wpu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In Valencia meeting, SCCE3 was established to study palette related coding tools. This proposal reports SCCE3 Test A.8 results on improved palette prediction vector signaling. Compared with SCCE3 base software, it is reported that the proposed method provides (3.3%, 3.3%, 3.2%) and (3.4%, 3.6%, 3.5%) BD-rate reduction for 1080p GBR/YCbCr text & graphics test sequences in All Intra lossy configuration with full frame IntraBC search, respectively. (4.0%, 4.0%, 3.9%) and (3.9%, 4.1%, 4.1%) BD-rate reduction for 1080p GBR/YCbCr text & graphics test sequences in All Intra lossy configuration is achieved in 2CTU IntraBC search, respectively.

# Technical Description

It was reported in Valencia meeting [4][5] that expanding the palette predictor list size by stuffing more predictors from previously coded paltte entries into it can improve palette mode coding efficiency. However, a long predictor list results in more bits used to indicate whether each palette predictor is reused in the current block or not. Improvements on coding such prediction information were tested in SCCE3 Test A.8. A two level hierarchical structure is proposed. First, the binary palette prediction vector is separated into groups. Each group contains 4 bins except for the last group, which may have less than 4 bins.

At the group level, two bins are signalled for each group to indicate

- whether the group contains non zero bin

- whether this is the last group containing non zero bin.

These two flags may be skipped if boundary conditions are satisfied. If the group level bin indicates that there is non zero bin within the group, then the four bins in the group are directly signaled into the bitstream.

All the bins in the proposed method are coded as bypass bins.

Please refer to the working draft for detailed information.

# Simulation Results

The simulation conditions are configured according to SCC common conditions in [1, Section 4] using the software specified in [1, Section 3]. Simulations were performed in heterogeneour Linux cluster so that the running time information is not reliable. Table I and Table II present the AI lossy results for full frame IntraBC and 2 CTU IntraBC configuration, respectively. Complete results can be found in the attached spreadsheets.

Table I. SCCE3 Test A.8 v.s. SCCE3 base SW Anchor (Full Frame IntraBC)

Table II. SCCE3 Test A.8 v.s. SCCE3 base SW Anchor (2CTU IntraBC)

# Conclusions

# Based on the coding gain reported in the simulation results, it is suggested to adopt this palette prediction vector signaling for palette mode coding.

# Reference

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “Description of screen content core experiment 3 (SCCE3): Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1123, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- J. Chen, Y. Chen, T. Hsieh, R. Joshi, M. Karczewicz, W.-S. Kim, X. Li, C. Pang, W. Pu, K. Rapaka, J. Sole, L. Zhang, and F. Zou, “Description of screen content coding technology proposal by Qualcomm,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0031, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- W. Pu, F. Zou, R. Joshi, M. Karczewicz, and J. Sole, “AHG10: Simplification of palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0047, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- C. Gisquet, G. Laroche, and P. Onno, “AhG10: Palette predictor stuffing,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0063, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- W. Pu, F. Zou, R. Joshi, M. Karczewicz, and J. Sole, “Improvements to palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0225, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).