| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0065 |
| --- | --- |

| Title: | SCCE3: Test B.12 – Binarization of Escape Sample and Palette Index |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Vadim Seregin Rajan Joshi Marta Karczewicz Joel Sole Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-7749 wpu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In Valencia meeting, SCCE3 was established to study palette related coding tools. This proposal reports SCCE3 Test B.12 results on binarization of escape sample and palette index. Compared with SCCE3 base software, it is reported that the proposed method provides (1.6%, 1.6%, 1.6%) and (2.1%, 2.0%, 1.9%) BD-rate reduction for 1080p GBR/YCbCr text and graphics test sequences in All Intra lossy configuration with full frame IntraBC search, respectively. (2.8%, 2.8%, 2.7%) and (3.3%, 3.1%, 3.1%) BD-rate reduction for 1080p GBR/YCbCr text and graphics test sequences in All Intra lossy configuration are achieved in 2CTU IntraBC search, respectively.

# Binarization of Escape Sample and Index in SCCE3 base Software

In SCCE3 base software, escape pixel is coded using fixed length code. The most significant bin in each color component is coded using CABAC context while the other bins are coded as bypass. The index is also coded as fixed length code with all of the bins coded as bypass.

# Technical Description

## Binarization of Escape Sample and Palette Index in SCCE3 Test B.12

In SCCE3 Test B.12, it is proposed to use truncated binary code to code escape pixel and index. All of the bins are bypass coded to improve the entropy coder throughput.

### Truncated Binary Code (TB)

For a syntax element with value synVal, assuming that it maximum possible value cMax is known. Let n = cMax + 1, k = floor( log2(n) ) such that 2k ≤ n < 2k + 1 and let u = 2k+1 –  n.

If synVal < u, the TB codeword is specified by the binary representation of synVal with length k. Otherwise, the TB codeword is specified by the binary representation of synVal + u with length k + 1.

### Redundancy Removal in Index Coding

To code a index, a redundancy check is applied:

- If the previous neighboring position in raster scanning order is the end of a copy-left run mode, then the current value cannot be the same as its previous neighboring position’s index value. Te The reason is that if the two index values are the same, they would be merged into a longer run.

- If the previous neighboring position in raster scanning order is the end of a copy-above run mode, and the current position’s above neighbor is not a escape pixel, then the current value cannot be the same as its top neighbor’s index value. The reason is that if the two index values are the same, they would be merged into a longer run.

If either of the above two cases hold, the range (i.e. cMax in 1.2.1) can be reduced by one and save bits.

### Other Redundancy Removal

The following redundancies are removed.

- The longest run-length cannot exceed the last pixel in the block in raster scanning order;

- If current position’s above neighbor is within the current CU and is an escape pixel, then the copy-above run mode cannot start from the current position;

- If the current position’s left and above neighbors are within the current CU and are not escape pixel and their index values are the same, then copy-above cannot start from the current position.

# Simulation Results

The simulation conditions are configured according to [1, Section 4] using the software specified in [1, Section 3]. Simulations were performed in heterogeneous Linux cluster so the running time information is not reliable. Table I and Table II present the AI lossy results for full frame IntraBC and 2 CTU IntraBC configuration, respectively. Complete results can be found in the attached spreadsheets.

Table I. SCCE3 Test B.12 v.s. SCCE3 base SW Anchor (Full Frame IntraBC)

Conclusion

Table II. SCCE3 Test B.12 v.s. SCCE3 base SW Anchor (2CTU IntraBC)

# Based on the coding gain reported in the simulation results, it is suggested to adopt the proposed truncate binary binarization and related redundancy removal for palette mode coding.

# Reference

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “Description of screen content core experiment 3 (SCCE3): Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1123, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- J. Chen, Y. Chen, T. Hsieh, R. Joshi, M. Karczewicz, W.-S. Kim, X. Li, C. Pang, W. Pu, K. Rapaka, J. Sole, L. Zhang, and F. Zou, “Description of screen content coding technology proposal by Qualcomm,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0031, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- W. Pu, F. Zou, R. Joshi, M. Karczewicz, and J. Sole, “AHG10: Simplification of palette based coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0047, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).