| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0072 |
| --- | --- |

| Title: | SCCE5 3.1.2: Extended inter-component prediction (JCTVC-Q0036) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuyuki Miyazawa Shun-ichi Sekiguchi 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports experimental results of SCCE5 3.1.2 proposed in JCTVC-Q0036. The proposed scheme introduces an adaptive inter-component prediction which extends “LM chroma” mode to not only intra prediction PUs but also intra block copy PUs with adaptive reference component selection for 3rd color component. It has been confirmed that the proposed scheme achieves up to 7.6% and 1.4% bitrate savings relative to SCCE5 anchor for RGB sequences and YCbCr sequences under lossy configurations, respectively. For lossless configurations, the coding performance of the proposed scheme shows up to 3.5% and 2.1% gains for RGB sequences and YCbCr sequences, respectively.

Moreover, the modified chroma lambda value for the proposed scheme has been evaluated for YCbCr sequences. The experiment result has been shown that the modified version achieves up to 2.7% gain compared with the anchor for lossy case.

# Introduction

In JCTVC-Q0036 [1], the extended inter-component prediction (ICP) based on “LM chroma” mode is proposed. This scheme introduces a new chroma prediction mode using a scaling and an offset parameter. The parameters are derived by the same formula of “LM chroma” mode in JVCTVC-M0097 [2]. The proposed ICP mode is available for PUs applying intra prediction or intra block copy for luma component. On reference color component for prediction, a selection flag for each PU is introduced for the R (Cr) component. In this test, the proposed scheme has been evaluated by the following tests.

- Test 1 (common test)

This test evaluates the coding performance of the proposed scheme under SCCE5 test conditions [3].

- Test 2 (additional test)

This test evaluates the version using the modified chroma lambda value. Such modification has the effect that inter-component prediction works as the tool of bitrate reduction instead of that of chroma PSNR improvement [4]. In this test, the coding experiment conducts only for YCbCr sequences for lossy configurations. The test conditions of this test also conform to SCCE5 test conditions.

# Proposed schemes

The proposed scheme introduces an adaptive inter-component prediction (ICP) which extends “LM chroma” mode to not only intra prediction PUs but also intra block copy PUs [1]. A PU-level flag is signaled to indicate whether to use the proposed ICP or not. If the flag is enabled, inter-component prediction (ICP) using a scaling and an offset parameter is conducted at corresponding PU. Moreover, to select the best reference block from decoded color components, Cr (R) component selects a flag indicating which color component is used as a reference in Y (G) and Cb (B) components for each PU. ICP generates the prediction samples predC[x, y] using the following formula which is known as “LM chroma” prediction described in JCTVC-M0097 [2]:

predC[x,y]=α×PR[x,y]+β,	(1)

where PR indicates reference component samples. The scaling parameter α and the offset β is derived as follows:

α=R(PR,PC)R(PR,PR)β=M(PC)−α×M(PR).	(2)

PC indicates reconstructed samples around the block to be predicted, and M(A) represents the mean of A, and R(A,B) is defined as follows:

R(A,B)=M(A−M(A))×(B−M(B) .	(3)

The prediction sample generation process shown above is conducted for each TU and PU for the intra prediction CU and intra block copy CU, respectively. It is noted that the derivation process of the scaling parameter α and the offset β is performed for each CU not PU for intra block copy CU.

When combined with cross-component prediction (CCP) for residual samples, the proposed scheme can turn on CCP only for PUs which is disabled ICP to reduce the amount of coding bits related to these predictions, as well as to reduce the complexity at the decoder side.

# Simulation results

The performance evaluation of the proposed scheme has been conducted. The proposed scheme is implemented on top of SCM1.0 which is set to the anchor of SCCE5. The experimental conditions are subject to SCCE5 test configurations. From Table 1 to Table 4 show coding performance of the proposed scheme relative to the anchor. According to the results, in lossy case, the proposed scheme achieves 3.5%/2.5%/1.5% and 0.9%/0.7%/0.5% gains for RGB sequences and YCbCr sequences under AI/RA/LB configurations, respectively. In lossless case, as shown in Table 3 and Table 4, it has been confirmed that the coding gains of the proposed scheme are 1.0%/0.4%/0.3% and 0.7%/0.3%/0.3% for RGB and YCbCr, respectively.

Moreover, the coding performance of the modified scheme using the modified choma lambda value has been evaluated. The results show in Table 5 and Table 6. According the results, the modified scheme has 1.9%/1.6%/1.4% gains for YCbCr sequences under AI/RA/LB configurations.

Table 1  Cording performance of the proposed scheme with full-search IBC for lossy case

Table 2  Cording performance of the proposed scheme with 2CTUs-search IBC for lossy case

Table 3  Cording performance of the proposed scheme with full-search IBC for lossless case

Table 4  Cording performance of the proposed scheme with 2CTUs-search IBC for lossless case

Table 5  Cording performance of the modified scheme with full-search IBC for lossy case

Table 6  Cording performance of the modified scheme with 2CTUs-search IBC for lossy case

# Conclusions

In this contribution, we reported experimental results of SCCE5 3.1.2 introducing an adaptive inter-component prediction for intra prediction PUs and intra block copy PUs. From the experimental results, the proposed scheme achieves up to 7.6% and 1.4% gains relative to SCCE5 anchor for RGB sequences and YCbCr sequences in lossy case, respectively. For lossless configurations, the coding performance of the proposed scheme indicates up to 3.5% and 2.1% gains for RGB sequences and YCbCr sequences, respectively. Moreover, the modified version with the modified chroma lambda achieves up to 2.7% bitrate savings compared with the anchor for lossy case. We recommend that the proposed scheme is adopted into SCC extensions draft text.

# References

- R. Cohen, A. Minezawa, X. Zhang, K. Miyazawa, A. Vetro, S. Sekiguchi, K. Sugimoto, T. Murakami, “Description of screen content coding technology proposal by Mitsubishi Electric Corporation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0036, 17th Meeting: Valencia, ES, March-April 2014.

- J. Kim, “RCE1: The performance of extended chroma mode for non 4:2:0 format,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M0097, 13th Meeting: Incheon, KR, April, 2013.

- X. Xiu and J. Chen, “HEVC Screen Content Coding Core Experiment 5 (SCCE5): Inter-component prediction and adaptive color transforms,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1125, 17th Meeting: Valencia, ES, March-April 2014.

- K. Kawamura and S. Naito, “AHG5: Objective and subjective evaluations of cross-component decorrelation in RExt6.0 for range extensions profile,” ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Document JCTVC-Q0212, March-April 2014.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Normative description change to be reflected to JCTVC-P1005

The test change of the proposed scheme is highlighted in yellow.

#### 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| inter_comp_pred_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| if( ChromaArrayType = = 3 && inter_comp_pred_enabled_flag) { |  |
| pbOffsetX = ( PartMode = = PART_NxN ¦¦ PartMode = = PART_Nx2N) ? ( nCbS / 2 ) : nCbS |  |
| pbOffsetY = ( PartMode = = PART_NxN ¦¦ PartMode = = PART_2NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffsetY ) |  |
| for( i = 0; i < nCbS; i = i + pbOffsetX ) { |  |
| inter_comp_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if(inter_comp_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| inter_comp_ref_flag[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| { |  |
| if(inter_comp_pred_enabled_flag) |  |
| inter_comp_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| if(inter_comp_pred_flag[ x0 + i ][ y0 + j ]) |  |
| inter_comp_ref_flag[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| } |  |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.3.2	Sequence parameter set RBSP semantics

inter_comp_pred_enabled_flag equal to 1 specifies that inter-component prediction may be invoked in the decoding process for intra prediction. inter_comp_pred_enabled_flag equal to 0 specifies that inter-component prediction is not applied. When not present, the value of inter_comp_pred_enabled_flag is inferred to be equal to 0.

#### 7.4.9.5	Coding unit semantics

inter_comp_pred_flag[ x0 + i ][ y0 + j ] equal to 1 specifies that the current chroma prediction block is predicted in inter-component prediction. The array indices x0 + i, y0 + j specify the location ( x0 + i, y0 + j ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When inter_comp_pred_flag[ x0 + i ][ y0 + j ] is not present, it is inferred to be equal to 0.

inter_comp_ref_flag[ x0 + i ][ y0 + j ] equal to 0 specifies that the current Cr prediction block is predicted by referring luma component in inter-component prediction. inter_comp_ref_flag[ x0 + i ][ y0 + j ] equal to 1 specifies that the current Cr prediction block is predicted by referring Cb component in inter-component prediction. The array indices x0 + i, y0 + j specify the location ( x0 + i, y0 + j ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When inter_comp_ref_flag[ x0 + i ][ y0 + j ] is not present, it is inferred to be equal to 0.

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

When intra_chroma_pred_mode [ x0 ][ y0 ] is not present and inter_comp_pred_flag[ x0 + i ][ y0 + j ] equal to 1, it is inferred to be equal to 4.

### 8.4.1	General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

A variable nCbS is set equal to 1  <<  log2CbSize.

Depending on the values of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- When intra_bc_flag[ xCb ][ yCb ] is equal to 1, the derivation process for block vector components in intra block copying prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and variable log2CbSize as inputs, and the output being bvIntra.

- The general decoding process for intra blocks as specified in subclause  is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable interCompPred set equal to 0,　the variable interCompRef set equal to 0, the variable bvIntra, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to 0, the variable interCompPred set equal to 0,　the variable interCompRef set equal to 0, and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable interCompPred set equal to inter_comp_pred_flag[ xCb ][ yCb ],　the variable interCompRef set equal to inter_comp_ref_flag[ xCb ][ yCb ],  the variable bvIntra, and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable interCompPred set equal to inter_comp_pred_flag[ xCb ][ yCb ],　the variable interCompRef set equal to inter_comp_ref_flag[ xCb ][ yCb ], the variable bvIntra, and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, the variable interCompPred set equal to inter_comp_pred_flag[ xPb ][ yPb ],　the variable interCompRef set equal to inter_comp_ref_flag[ xPb ][ yPb ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, the variable interCompPred set equal to inter_comp_pred_flag[ xPb ][ yPb ],　the variable interCompRef set equal to inter_comp_ref_flag[ xPb ][ yPb ], and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

#### 8.4.4.1	General decoding process for intra blocks

Inputs to this process are:

–	a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current transform block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable predModeIntraBc specifying the intra block copying mode,

–	a variable bvIntra specifying the intra block copying vector,

–	a variable interCompPred specifying the inter component prediction enabling flag,

–	a variable interCompRef specifying the reference component for inter component prediction,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The luma sample location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC )	(8-4)

The variable splitFlag is derived as follows:

–	If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ].

–	Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

- cIdx is greater than 0

- split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ] is equal to 1

- log2TrafoSize is greater than 2

–	Otherwise, splitFlag is set equal to 0.

The variable alpha and beta are derived as follows:

- The variable nTbS is set equal to 1 <<  log2TrafoSize.

- When interCompPred is equal to 1, predModeIntraBc is equal to 1 and the variable trafoDepth is equal to 1, the derivation process of inter component prediction parameters as specified in subclause 8.4.2.X is invoked with the transform block location ( xTb0, yTb0 ), the transform block size nTbS, the reference component interCompRef, and the variable cIdx as inputs, and the outputs are the variable iAlpha, iBeta and iShift.

Depending on the value of splitFlag, the following applies:

–	If splitFlag is equal to 1, the following ordered steps apply:

- The variables xTb1 and yTb1 are derived as follows:

  - If either cIdx is equal to 0 or ChromaArrayType is not equal to 2, the following applies:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - Otherwise (ChromaArrayType is equal to 2 and cIdx is greater than 0), the following applies:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 2  <<  ( log2TrafoSize − 1 ) ).

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (splitFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( cIdx > 0  &&  ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- The variable nTbS is set equal to 1 <<  log2TrafoSize.

- The variable yTbOffset is set equal to blkIdx * nTbS.

- The variable yTbOffsetY is set equal to yTbOffset * SubHeightC.

- The variable residualDpcm is derived as follows:

  - If all of the following conditions are true, residualDpcm is set equal to 1.

    - implicit_rdpcm_enabled_flag is equal to 1.

    - either transform_skip_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ] is equal to 1, or cu_transquant_bypass_flag is equal to 1.

    - either predModeIntra is equal to 10, or predModeIntra is equal to 26.

  - Otherwise, residualDpcm is set equal to explicit_rdpcm_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ].

- Depending upon the value of predModeIntraBc and interCompPred, the following applies:

–	When interCompPred is equal to 1 and predModeIntraBc is equal to 0, the derivation process of inter component prediction parameters as specified in subclause 8.4.2.X is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the reference component interCompRef, and the variable cIdx as inputs, and the outputs are the variable iAlpha, iBeta and iShift.

–	When interCompPred is equal to 1, the inter component prediction process as specified in subclause 8.4.2.XX is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the reference component interCompRef, the variable iAlpha, the variable iBeta, and the variable iShift as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraBc is equal to 0 and interCompPred is equal to 0), the general intra sample prediction process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraBc is equal to 1 and interCompPred is equal to 0), the intra block copying process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the variable trafoDepth, the variable bvIntra, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xTbY, yTbY + yTbOffsetY ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

- When residualDpcm is equal to 1, depending upon the value of predModeIntraBc, the following applies:

–	When predModeIntraBc is equal to 0, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to predModeIntra / 26, the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

–	Otherwise, (predModeIntraBc is equal to 1), the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to explicit_rdpcm_dir_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ], the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- When cross_component_prediction_enabled_flag is equal to 1, ChromaArrayType is equal to 3, and cIdx is not equal to 0, the residual modification process for transform blocks using cross-component prediction as specified in subclause Error: Reference source not found is invoked with the current luma transform block location ( xTbY, yTbY ), the variable nTbS, the variable cIdx, the (nTbS)x(nTbS) array rY set equal to the corresponding luma residual sample array resSamples of the current transform block, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

#### 8.4.4.X	The derivation process of inter component prediction parameters

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable nS specifying the block size indicating a unit of inter component prediction parameters derivation,

–	a variable interCompRef specifying the reference component for inter component prediction,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are:

–	a variable iAlpha specifying a scaling parameter for inter component prediction,

–	a variable iBeta specifying a offset for inter component prediction,

–	a variable iShift specifying a shift parameter for inter component prediction.

The values of iAlpha, iBeta and iShift are derived as the following ordered steps:

- A picture array PR[ xB , yB ] is set as follows:

–	When interCompRef is equal to 0, PR[ xB , yB ] is set to a modified reconstructed picture of luma component before deblocking filtering,

–	Otherwise (interCompRef is equal to 1), PR[ xB , yB ] is set to a modified reconstructed picture of the component Cb before deblocking filtering.

- A picture array PC[ xB , yB ] is set to a modified reconstructed picture of the component indicated by a variable cIdx before deblocking filtering.

- The sample array pY and p are derived as:

–	pY[ x, −1 ] = PR[ xB + x, yB − 1 ] , with x = 0..nS−1	(8-xx)

–	pY[ −1, y ] = PR[ xB − 1, yB + y ], with y = 0..nS−1	(8-xx)

–	p[ x, −1 ] = PC[ xB + x, yB − 1 ] , with x = 0..nS−1	(8-xx)

–	p[ −1, y ] = PC[ xB − 1, yB + y ], with y = 0..nS−1	(8-xx)

- Variable k0 is derived as:

k0 = Max( 0, BitDepthC + Log2( nT) − 14 )		(8-xx)

- Variables l, c, ll, lc and k1 are derived as follows:

l = y=0nS−1pY[−1, y]+x=0nS−1pY[x, −1]>>k0		(8-xx)

c = y=0nS−1p[−1, y]+x=0nS−1p[x, −1]>>k0		(8-xx)

ll = y=0nS−1pY[−1, y]2+x=0nS−1pY[x, −1]2>>k0	(8-xx)

lc = y=0nS−1pY[−1, y]∗p[−1, y]+y=0nS−1pY[x, −1]∗p[x,−1]>>k0	(8-xx)

k1 = Log2( (2*nS) >> k0 )		(8-xx)

- Variables iAlpha, iBeta and iShift are specified by the following pseudo-code:

avgY’ = l >> k1
errY’ = l & ( ( 1 << k1 ) − 1 )
avgC = c >> k1
errC = c & ( ( 1 << k1 )  − 1 )
a1 = lc − ( 2 * nS * avgY’ * avgC + avgY’ * errC + avgC * errY’ )
a2 = ll − ( 2 * nS * avgY’2 + 2 * avgY’ * errY’ )
k2 = ( a1 = = 0) ? 0 : Max( 0, Floor( Log2( Abs( a1 ) ) ) − BitDepthC + 2 )
k3 = ( a2 = = 0) ? 0 : Max( 0, Floor( Log2( Abs( a2 ) ) ) − 5 )
k4 = k3 − k2 + BitDepthC − 2
a1s = a1 >> k2
a2s = a2 >> k3
lmDiv = ( 1 << ( BitDepthC + 4 ) + a2s / 2 ) / a2s
a2t = ( a2s < 32) ? 0 : lmDiv
a2t = Clip1C ( a2t )		(8-5)
if( a2s < 32 )
	a3 = 0
else if( a2s >= 32 && k4 >= 0 )
	a3 = ( a1s * a2t ) >> k4 )
else
	a3 = ( a1s * a2t ) << ( −k4 )
a4 = Clip3( − 28, 28 − 1, a3 )
a5 = a4 << 7
k5 = ( a5 = = 0 ) ? 0 : k5 = Floor( Log2( Abs( a5 ) + ( Sign2( a5 ) − 1)/2 ) ) − 5
iShift = 13 − k5
iAlpha = a5 >> k5
iBeta = avgC − ( ( iAlpha * avgY’ ) >> k )

#### 8.4.4.XX	The inter component prediction process

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	a variable nS specifying the block size,

–	a variable interCompRef specifying the reference component for inter component prediction,

–	a variable iAlpha specifying a scaling parameter for inter component prediction,

–	a variable iBeta specifying a offset for inter component prediction,

–	a variable iShift specifying a shift parameter for inter component prediction.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y = 0.. nS − 1.

The values of the prediction samples predSamples[ x, y ], with x, y = 0.. nS −1, are derived as the following ordered steps:

- A picture array PR[ xB , yB ] is set as follows:

–	When interCompRef is equal to 0, PR[ xB , yB ] is set to a modified reconstructed picture of luma component before deblocking filtering,

–	Otherwise (interCompRef is equal to 1), PR[ xB , yB ] is set to a modified reconstructed picture of the component Cb before deblocking filtering.

- The values of the prediction samples predSamples[ x, y ] are derived as:

predSamples[ x, y ] = Clip1C( ( ( PR[ xB + x, yB + y ] * iAlpha ) >> iShift ) + iBeta ), with x, y = 0..nS−1		(8-6)

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 9..11 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
|  | inter_comp_pred_flag | Table 9-XX | 0 | 1 | 2 |
|  | inter_comp_ref_flag | Table 9-YY | 0 | 1 | 2 |

Table 9-XX – Values of initValue for ctxIdx of inter_comp_pred_flag

| Initialization variable | ctxIdx of cu_residual_csc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-YY – Values of initValue for ctxIdx of inter_comp_ref_flag

| Initialization variable | ctxIdx of cu_residual_csc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-2 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] |  | - |
|  | rqt_root_cbf | FL | cMax = 1 |
|  | inter_comp_pred_flag | FL | cMax = 1 |
|  | inter_comp_ref_flag | FL | cMax = 1 |