| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0075r2 |
| --- | --- |

| Title: | Non-SCCE3: Encoder modification of palette mode for escape pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, 3Ching-Chieh Lin, Jih-Sheng Tu, and Chao-Hsiung Hung 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

This document proposes an encoder modification of palette mode. If only few pixels are quantized to a major color, the proposed method will modify the major color index as an escape index. The contribution is combined with JCTVC-R0051, and implemented on top of SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047. Experimental results show that, compared with SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047, –1.4% and –1.1% BD-rate savings under AI-Lossy coding are achieved for YUV Text & Graphics with motion sequences (1080p) on 2-CTU and full-frame IBC conditions, respectively.

# Introduction

The current encoder method of palette mode [1] does not contain escape pixels as palette size is smaller than maximum value, i.e., 31. However, some of major colors may only quantize few pixels in the CU. It could be a waste of bits to add the major color indexes for quantizing these few pixels.

# Proposed encoder modification for palette mode

This contribution proposes to modify a major color index as an escape index when only few pixels are quantized to the major color. Suppose M is a pre-assigned natural integer, and n is a major color index for palette mode. The algorithm is as follows:

If (Sum of pixels quantized to major color n is smaller than or equal to M)

Modify pixels quantized to major color n as escape pixels

# Test results

The proposed encoder method with M = 2 is combined with the CU-level escape flag on JCTVC-R0051 and is implemented on SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047. The experimental conditions are subject to the test conditions issued by JCT-VC in the 17th meeting [2]. Two anchors are evaluated:

Anchor 1-1: SCM-1.0 common test condition with intraBC using full frame search + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

Anchor 2-1: SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU) + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

Table 1 compares the proposed method with Anchor 1-1. Table 2 compares the proposed method with Anchor 2-1.

Table 1. Proposed vs. Anchor 1-1 (Lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.1% | -1.2% |
| RGB, text & graphics with motion,720p | -0.4% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.6% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.2% | -1.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.5% |
| YUV, mixed content, 1440p | -0.3% | -0.5% | -0.7% |
| YUV, mixed content, 1080p | -0.5% | -1.2% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.6% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.3% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | -0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.7% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.7% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.6% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.6% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | -0.4% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | -0.5% |
| YUV, mixed content, 1080p | -0.3% | -1.4% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2. Proposed vs. Anchor 2-1 (Lossy)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.4% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -0.6% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | -0.7% | -0.6% | -0.5% |
| RGB, mixed content, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.4% | -1.4% | -1.5% |
| YUV, text & graphics with motion,720p | -0.5% | -0.5% | -0.9% |
| YUV, mixed content, 1440p | -0.4% | -0.6% | -0.7% |
| YUV, mixed content, 1080p | -0.6% | -1.0% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.7% |
| RGB, text & graphics with motion,720p | -0.2% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.4% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.6% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.3% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | -0.5% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.4% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.8% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | -0.3% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.3% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.6% |
| YUV, text & graphics with motion,720p | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | -0.4% |
| YUV, mixed content, 1080p | -0.1% | -1.1% | -0.6% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

In this contribution, an encoder modification of palette mode is proposed, and is combined with JCTVC-R0051. Experimental results show that, compared with SCM-1.0 + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047, –1.4% and –1.1% BD-rate savings under AI-Lossy coding are achieved for YUV Text & Graphics with motion sequences (1080p) on 2-CTU and full-frame IBC conditions, respectively. According to the interesting gains compared to the negligible increase of complexity, it is recommended to further study the method.

# References

- W. Pu, X. Guo, P. Onno, P. Lai, and J. Xu, “AHG10: Suggested Software for Palette Coding based on RExt6.0,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0094, Mar. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, Apr. 2014.

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).