| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0077 |
| --- | --- |

| Title: | SCE1: Cross-Check of Test 5.1.4 (16ILR) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes | Tel: Email: | +33 299273242 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking results for SCE1 Test 5.1 (1.4) on improved weighted prediction for color gamut scalability. The simulation results reportedly matched those provided by the proponents.

# Introduction

SCE1-Test-5.1 aims to test utilizing Weighted Prediction (WP) design for coding of color gamut scalability.

Encoder only algorithmError: Reference source not found that improves the coding efficiency of weighted prediction enabled in SHVC for inter-layer reference was proposed in JCTVC-Q0056.

In this test, following encoder only algorithms for WP have been tested:

- More accurate estimation of WP parameters: The separation criteria is improved to take into account the triplet of luma and chroma values.

- Using more ILR pictures: More inter-layer reference pictures have been added to reference picture lists and separate weight and offset parameters are signaled for each one of those pictures. N=16 ILR pictures is used in this test (8 ILR in each list).

# Experimental results

We received the source code from the proponents, implemented on top of SHM-6.0 SCE1 anchor software. We ran simulations for the cases of AI-1x, AI-2x, RA-1x, RA-2x with SCE1 test sequences and test conditions Error: Reference source not found.

The results matched what were provided by the proponents and are summarized as follows.

Table 1: Cross-check results for the cases AI.

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | 7.1% | 15.1% | 17.8% | 5.9% | 8.0% | 10.1% |
| Overall (Test vs single layer) | 7.1% | 15.1% | 17.8% | 5.9% | 8.0% | 10.1% |
| Overall (Ref vs single layer) | 27.1% | 25.8% | 6.4% | 17.5% | 20.1% | 13.8% |
| EL only (Test vs Ref) | 18.7% | 10.4% | -9.0% | 10.9% | 11.4% | 3.7% |
| Overall (Test EL+BL vs single EL+BL) | 59.5% | 73.4% | 82.6% | 13.4% | 15.6% | 18.2% |
| Overall (Ref EL+BL vs single EL+BL) | -31.0% | -31.7% | -41.7% | -21.8% | -20.4% | -24.4% |
| Enc Time[%] | 244.0% |  |  | 310.7% |  |  |
| Dec Time[%] | 83.2% |  |  | 92.4% |  |  |

Table 2: Cross-check results for the cases RA.

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | 9.3% | 19.0% | 24.2% | 5.1% | 7.5% | 10.3% |
| Overall (Test vs single layer) | 9.3% | 19.0% | 24.2% | 5.1% | 7.5% | 10.3% |
| Overall (Ref vs single layer) | 41.2% | 44.9% | 21.2% | 27.5% | 31.0% | 23.7% |
| EL only (Test vs Ref) | 29.1% | 22.9% | -0.5% | 21.3% | 22.5% | 12.4% |
| Overall (Test EL+BL vs single EL+BL) | 44.6% | 59.7% | 70.3% | 10.8% | 13.1% | 16.0% |
| Overall (Ref EL+BL vs single EL+BL) | -22.5% | -20.5% | -32.6% | -14.3% | -11.7% | -16.3% |
| Enc Time[%] | 123.6% |  |  | 139.4% |  |  |
| Dec Time[%] | 65.7% |  |  | 83.1% |  |  |

# Conclusion

In this contribution, we have presented the results of our cross-check of SCE1-Test-5.1.4. The implemented algorithm is in line with the proponent’s description, and the simulation results also match those provided by the proponents.

Compared to the anchors, the use of WP with 16 ILR decreases the SHVC performance and significantly increases the complexity at the encoder side. Additionally, the memory bandwidth is increased (16 ILR) compared to the anchors (one ILR in AI and 8 ILR in RA).

The HEVC General tier and level limits specified in table A.1 allows to derive the corresponding MaxDpbSize for HDTV and UHDTV, as specified in A.4.1 .

In current applicative standard committees for specifying HEVC decoders and bit-streams, level 4.1 for HDTV and level 5.1 for UHDTV are recommended. This corresponds to upper limit of 6 frames in the DPB. To insure backward compatibility with legacy deployed HEVC decoders and re-use of existing hardware decoder platforms to deploy SHVC (with High level syntax up-date), the CE manufacturers may encourage adopting same levels for SHVC decoders, thus limiting the number of ILR to be 12 at most.

Table A1: General tier and level limits of HEVC. The two last columns have been added from A.4.1 .

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (1000 bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols | MaxDpbSize |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  | HDTV | UHDTV |
| 1 | 36864 | 350 | - | 16 | 1 | 1 | 6 | 6 |
| 2 | 122880 | 1500 | - | 16 | 1 | 1 | 6 | 6 |
| 2.1 | 245760 | 3000 | - | 20 | 1 | 1 | 6 | 6 |
| 3 | 552960 | 6000 | - | 30 | 2 | 2 | 6 | 6 |
| 3.1 | 983040 | 10000 | - | 40 | 3 | 3 | 6 | 6 |
| 4 | 2228224 | 12000 | 30000 | 75 | 5 | 5 | 6 | 6 |
| 4.1 | 2228224 | 20000 | 50000 | 75 | 5 | 5 | 6 | 6 |
| 5 | 8912896 | 25000 | 100000 | 200 | 11 | 10 | 16 | 6 |
| 5.1 | 8912896 | 40000 | 160000 | 200 | 11 | 10 | 16 | 6 |
| 5.2 | 8912896 | 60000 | 240000 | 200 | 11 | 10 | 16 | 6 |
| 6 | 35651584 | 60000 | 240000 | 600 | 22 | 20 | 16 | 16 |
| 6.1 | 35651584 | 120000 | 480000 | 600 | 22 | 20 | 16 | 16 |
| 6.2 | 35651584 | 240000 | 800000 | 600 | 22 | 20 | 16 | 16 |

# References

- A.Duenas, P.Andrivon, Y.Ye, E.Alshina, X.Li, K.Ugur, C.Auyeung, SH.Kim, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut Scalability,” JCTVC-Q1101, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.