| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0078 |
| --- | --- |

| Title: | Non-SCCE3: Modified copy above mode for palette based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianqing Zhu, Zhanglei Xu, Jiangli Ye 29F,Kerry Parkside,1155 fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 zhujianqing@cn.fujitsu.com xuzhanglei@cn.fujitsu.com threerabbit@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

Simplification of the process for copy above mode on palette based coding is presented. Use pixel value directly rather than index for encoding and decoding process for copy mode. And copy above mode is extended from 1 above direction to 3 above directions. Reported test results show gain of 0.1% AI-lossless for 1 above direction and 0.1-0.3% AI-lossless on SCs for 3 above direction on top of SCCE3 anchor(full frame IBC).

# Introduction

SCCE3[1] was established for study on Palette base coding tools in the 17th JCT-VC meetings in Valencia. And base software [2] was suggested. This contribution is an improvement on top of the base software.

A modified copy above mode for palette based coding [3] was proposed in Valencia meeting.

This contribution evaluates above method on top of SCCE3 anchor and extends it to multiple copy above directions.

# Proposed method

## Simplified process for copy above mode

Use pixel value directly for “copy-above” (in horizontal scan order) or “copy-left” (in vertical scan order) mode in encoder and decoder. For “copy run” derivation in encoder side, there is no index derivation. Just compare pixel value of current position and its above/left to judge whether it can be copy or not. In decoder side, for pixel in “copy mode”, the reconstructed pixel value of current position is the reconstructed pixel value of its above/left position.

In this way, copy mode will not care about the pixel value is in escape or not. The “copy run” might be larger so that coding efficient is higher. For the CU in Figure1 case, for location (0,1) (first pixel of second line), “copy run ” is 15 as shown in Figure2.

Figure 2

Meanwhile, calculation complexity is reduced since no index derivation process is needed for “copy mode”.

## Multiple directions copy above

Not only straight above location is considered for copy above, more above locations can be considered. As shown in figure 3, 3 directions are considered.

Figure 3

1) In “copy above left” mode, pixel(x,y)==pixel(x-1,y-1);

2) In “copy above” mode, pixel(x,y)==pixel(x,y-1);

3) In “copy above right” mode, pixel(x,y)==pixel(x+1,y-1);

A syntax element of CU level, copy_above_dir, will be used to indicate the direction. And only if copy above mode is used, it presents.

As same as used in base software, 1st line of CU does not use “copy above” mode. For “copy above left” mode, reconstructed pixels in left CU will be used . For “copy above right” mode and at the right most column, the padding process is as follows.

Reconstructed_pixel(CU_width,y)= Reconstructed_pixel(CU_width-1,y);

# Experimental results

The anchor is the base software of SCCE3(full frame IBC). Test condition is common conditions for screen content coding test in Q1015[4].

## Result of simplified process for copy above mode (1 direction )

Table 1. Simulation results in lossless coding mode

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, text & graphics with motion,720p | 0.1% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 87% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

## Result of simplified process for copy above mode (3 direction )

Table 2. Simulation results in lossless coding mode

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.3% | 0.2% | 0.0% | 0.7% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

# References

- Y.W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “Description of Screen Content Core Experiment 3 (SCCE3): Palette mode,” JCTVC-Q0094, Valencia, ES, April. 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, and J. Xu, “AHG10: Suggested Software for Palette Coding based on RExt6.0,” JCTVC-Q0094, Valencia, ES, March. 2014.

- J. Zhu, J. Ye, K. Kazui, “AHG10: Modified copy above mode for palette based coding,” JCTVC-Q0174, Valencia, ES, March. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu“Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, ES, April. 2014.

# Patent rights declaration

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).