| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0079 |
| --- | --- |

| Title: | Non-SCCE3: Skip palette size for palette based coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianqing Zhu, Jiangli Ye 29F,Kerry Parkside,1155 fangdian Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 zhujianqing@cn.fujitsu.com threerabbit@cn.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. |  |  |

_____________________________

# Abstract

By deriving palette size form index information, the syntax element of palette size is skipped for palette based coding. Reported test results show gain of 0.3-1.5% AI-lossless on top of SCCE3 anchor(2CTU-IBC).

# Introduction

SCCE3[1] was established for study on Palette base coding tools in the 17th JCT-VC meetings in Valencia. And base software [2] was suggested. This contribution is an improvement on top of the base software.

In base software, for a coding unit (CU) encoded in palette mode, palette information is first coded. Then pixel information are encoded. Pseudo syntax of a CU coded in palette mode is as follows.

palette_coding_component (nCbS ) // nCbS * nCbS is the size of coding unit

{

…

numPredPreviousPalette = 0

for( i = 0; i < previousPaletteSize; i++ ) {

previous_palette_entry_flag[ i ]

if ( previous_palette_entry_flag[ i ] ) {

numPredPreviousPalette++

}

}

palette_num_signalled_entries

for( i = 0; i < palette_num_signalled_entries; i++ )

palette_entrie[ numPredPreviousPalette + i ]

palette_size = numPredPreviousPalette + palette_num_signalled_entries

….

Pixel_index_infomation ( );

}

A palette size related syntax element is signaled. In above syntax, palette_num_signalled_entries specifies the number of entries in the palette that are explicitly signaled. The palette size is the sum of predicted palette entries number and explicitly signaled palette entries number.

This contribution presents a method to skip the syntax element indicating palette size.

# Proposed method

## Skip Palette Size

Pixel index information is coded firstly. Then palette information is coded. Palette size is equal to maximum value of index plus 1. So, no syntax element indicating palette size is needed.

To make pixel index information decodable, a syntax element, index_bit_depth, is present to indicate how many bits an index is expressed.

# Experimental results

The anchor is the base software of SCCE3(2CTU-IBC). Test condition is common conditions for screen content coding test in Q1015[3].

## Result of skip palette size

Table 1. Simulation results in lossless coding mode

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.5% | 1.5% | 0.8% | 1.9% |
| RGB, text & graphics with motion,720p | 0.7% | 0.5% | 0.0% | 1.0% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.2% | 0.5% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.5% | 1.7% | 0.8% | 3.0% |
| YUV, text & graphics with motion,720p | 0.5% | 0.4% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.1% | 0.5% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 85% |  |  |  |
| Dec Time[%] | 170% |  |  |  |

# References

- Y.W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “Description of Screen Content Core Experiment 3 (SCCE3): Palette mode,” JCTVC-Q0094, Valencia, ES, April. 2014.

- W. Pu, X. Guo, P. Onno, P. Lai, and J. Xu, “AHG10: Suggested Software for Palette Coding based on RExt6.0,” JCTVC-Q0094, Valencia, ES, March. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu“Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, ES, April. 2014.

# Patent rights declaration

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).