| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0081 |
| --- | --- |

| Title: | SCCE1 Test 3.3: BV prediction using the three last predictors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the results for the SCCE1 Test 3.3 on block vector prediction for the Intra block copy mode. The displacement block vector of the current CU is predicted by using the displacement vector of the previously IBC-coded CU in the current design of IBC. In this contribution, the 3 displacement vectors of previously IBC-coded CU are considered for the prediction of the current vector. The best predictor is indicated according to an index. It is reported that the proposed modifications provide an average bitrate reduction of 6.5% for text & graphics 1080p classes without impacting the encoding run time. This can reach 8% by modifying the encoder selection method.

# Introduction

In the current design of the intra block copy, the displacement block vector of the current CU is predicted by using the displacement vector of the previously IBC-coded CU. This method was considered as the best trade-off between complexity and coding efficiency performance. Yet, additional partitioning and larger search area for IBC offer the possibility to improve significantly the coding performance by introducing a competitive scheme for the current block vector. This contribution is presenting a simple modification of the current IBC displacement vector prediction design to get higher performance.

# Proposed method

For SCCE1 Test 3.3, we proposed to use a predictor list which includes the 3 last decoded IBC displacement vectors and indicates through a syntax element the predictor index to select the right predictor in that list. This is similar to the “ping-pong” SCCE1 Test 3.2 where the two last decoded IBC vectors were used and the selected one was signaled through a flag.

With the new IBC partition, the use of three predictors is the best trade-off between performance and memory storage compared to other prediction schemes like SCCE1 Test 3.1 or SCCE1 Test 3.4 and Test 2.3 where they use the displacement vectors of the top and left CUs. For SCCE1 Test 3.4, it is necessary to store a line and a column of IBC displacement vectors. So it is necessary to store up to 31 vectors for the worst case. For SCCE1 Test 2.3 and SCCE1 3.1, an additional line buffer is needed to store vector of the upper CTB.

Compared to the current design of using only the last displacement vector, the proposed method with the 3 last decoded vectors provides a coding efficiency gain since there is a better match of the current IBC block due to the new IBC partition and its associated scan order. This is not the case with the last decoded displacement vector which is unique. In terms of memory, the additional memory to store two additional vectors is negligible.

Moreover, in this proposal the reset of last predictor at CTB level is kept in order to offer an easy parallelization of IBC block estimation at CTB level.

# Encoder Method

## Initial encoder setting

For this SCCE1 Test 3.3, we propose the results of 3 configurations. For the first configuration, the 3 predictors are competing just before the real rate estimation. But to be fairly compared to other methods Test 2.3, 3.1, 3.2, 3.4, it seems interesting to modify the encoder RD selection for our Test 3.3.

So, the following modifications were integrated in the original source code:

1/ Bug fix on BV vector selection for Chroma (Q0175) including in test 3.2. + other minor bug fixes.

2/ BV costs are estimated by considering all predictors as in Test 3.2.

3/ BV predictor is RD estimated even if the predictor is outside the 2 CTBs range when Full frame is enabled, in order to do the same as Test 2.3 where the Merge predictors are systematically tested.

4/ BV predictors are estimated for 64x64 and 32x32 block sizes as for the Merge predictor Test 2.3.

Two configurations have been tested based on these 4 modifications corresponding to 2 complexity levels:

-Configuration 1 includes: 1/ + 2/ + 3/ + 4/

-Configuration 2 includes: 1/ + 2/ + 3/

Moreover for this test, we have disabled the CTB reset.

Configuration 1 seems to obtain a non-realistic compromise in terms of bitrate savings and run time increase compared to Configuration 2 which has a reasonable encoding time and interesting coding gains.

## Additional encoder setting

After studying carefully the different SCCE1 proposals, we found that a large part of the gain comes from the BV list which is tested during the estimation. In particular, the left and the above BVs are quite useful BVs to be tested in addition to the current list.

So in a third configuration: “Configuration 1 Bis”, we added to Configuration 1 the use of the left and the above vector in the list of predictors tested in 3/ and 4/. Please note that this modification doesn’t change the decoding process. It is only an encoder modification.

Similarly to “Configuration 1_Bis”, we did the same for Configuration 2 for the initialization of the BV estimation. This corresponding test is “Configuration 2 Bis”.

- Experimental results

The proposed method was tested in the SCM version 1.0 according to the SCCE1 Test conditions which use the full IBC search range.

For config1, the proposed method gives an average gain for all sequences of -4.2% for AI, -2.8% for RA and -1.8% for LDB configuration. But this configuration increases the encoding time especially for Intra configuration with 111% encoding run time.

Table 1: Lossy results of SCCE1 Test 3.3 Config 1.

# For config2, the proposed method gives an average gain for all sequences of -3.4% for AI, -2% for RA and 1.1% for LDB configuration. But this configuration doesn’t change the encoding run time compared to SCM1.0.

Table 2: Lossy results of SCCE1 Test 3.3 Config 2.

# Additional results

## Configuration_1_Bis

Table 3: Lossy results of SCCE1 Test 3.3 Config 1 Bis

For this additional experiment Config1 bis, the proposed method gives an average gain for all sequences of -4.8% for AI, -3.2% for RA and 2% for LDB configuration. But this configuration has a large encoding run time as Configuration 1.

## Configuration_2_Bis

Table 4: Lossy results of SCCE1 Test 3.3 Config 2 Bis

# Conclusion

This contribution reports the results of SCCE1 Test 3.3 on block vector prediction for Intra Block Copy. The proposed method uses the same design as the current one by using the three vector displacements of the last decoded IBC PU. This has a small impact on memory complexity compared to some other BV prediction methods tested in SCCE1. Moreover the proposed method doesn’t’ affect the CTB reset of the vector predictor in order to offer an easy parallelization of IBC block estimation at CTB level.

From a coding efficiency point of view, this method provides an average gain of 3.4% for all sequences of the AI configuration and reaches 6.5% for Screen content 1080p classes without impact on run time.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).