| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0082 |
| --- | --- |

| Title: | SCCE3 Test A.3: palette stuffing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution reports the results of SCCE3 test A.3 on the palette predictor update process called stuffing, corresponding to the JCT-VC proposal Q0063 presented during the 17th meeting in Valencia. It is proposed in the present contribution to combine the last coded palette and the last predictor palette by stuffing unused elements of the last predictor at the end of the last coded palette, using the resulting palette as predictor for the next palette-coded CU, and adding flags to indicate when there are no predicted elements left. It is reported that the use of such a prediction method provides gains, compared to SCCE3 palette anchor, of -1.5% (AI) / -1.2% (RA) / -0.6% (LDB) with IBC search area of 2 CTBs, and -1.0% (AI) / -0.8% (RA) / -0.5% (LDB).

# Description

## Problems with current prediction scheme

In the software for base experimentation of the palette coding technique derived from [1], a first problem is that, when a palette with a few elements occurs, the predictor for the next palette-coded CU is forced to have a few elements, thereby greatly reducing the number of potentially predicted elements.

On the other hand, a second problem is that, for a predictor with a lot of elements, a large number of flags must be transmitted. It may happen that the first ones may be the only ones needed for prediction, resulting in a larger number of flags transmitted.

## Improving the predictor generation

Current prediction scheme uses the last coded palette as predictor. It is proposed to stuff at the end of this predictor the unused values from the current predictor, as follows:

Figure 1: stuffing process

Let us consider the recursive construction on Figure 1: after CU1 has been encoded, a predictor is made available to CU2. Currently, it is CU1’s palette. Bitflags indicating the reuse of the predictor elements are generated and coded. CU2 palette would normally be used as the predictor of CU3. By checking the bitflags from the palette predictor of CU2, it is possible to determine which elements have not been reused. Those elements are added to the end of the predictor.

This is beneficial because those elements have actually occurred in the past, and may still occur. However, their probability is lower. By this recursive construction, a predictor is generated, where elements are somewhat ordered by probability. In addition, it is possible and beneficial to increase the size of the predictor, due to the high compression ratio a valid prediction offers.

This provides a solution with a very low overhead compared to the original bookkeeping.

## Improving the prediction flag coding

In particular when the size of the predictor is a lot larger than the actual palette to predict, it may happen that the palette is predicted by elements mostly at the start of the predictor, as depicted below:

Figure 2: prediction flags

As can be seen above, the prediction scheme may have a limited efficiency. A typical improvement would be CABAC coding of the elements, but this would require frequent updating for at least 32 flags. It is proposed instead to add intermediate flags, called “end-of-prediction”, to indicate when elements can be no longer predicted. Because this adds more flags, it however may cause an expansion of the bitmap of flags. Therefore, it is better to insert them at specific locations in the bitmap.

More refined solutions exist but careful attention must be paid to the complexity trade-off.

# Results

All results presented here are using SCCE3 anchor with the palette mode activated

## Full frame IBC search

## 2 CTBs search area

# Conclusion

# References

- W. Pu (Qualcomm), X. Guo (Microsoft), P. Onno (Canon), P. Lai (MediaTek), J. Xu (Sony), “AHG10: Suggested Software for Palette Coding based on RExt6.0,” JCTVC-Q0094, 17th JCT-VC meeting, March 2014, Valencia.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).