| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0083 |
| --- | --- |

| Title: | SCCE3 Test B.5: index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution reports the results of SCCE3 test B.5 on index coding, corresponding to the JCT-VC proposal Q0064 presented during the 17th meeting in Valencia. It is proposed in the present contribution to use truncated binary codes to encode the index, taking into account the fact some indexes are impossible. It is reported that the use of such a prediction method provides gains , compared to SCCE3 palette anchor, of -1.0% (AI) / -0.7% (RA) / -0.5% (LDB) with IBC search area of 2 CTBs, and -0.7% (AI) / -0.5% (RA) / -0.4% (LDB).

# Description

There are actually 3 things that are combined and together affect the index coding.

## Impossible values

Currently, when the mode on the left is copy-left, the index to code cannot be the index on the left, otherwise the copy left run would have been incremented by one. Similarly, when the mode on the left is copy up, the index to code cannot be the same as the one above, otherwise the copy up run would have been incremented by one.

This is cannot be exploited alone because of the FLC used for the index.

## Truncated binary coding

When the palette size is smaller than allowed by the binary FLC coding of the index, inefficiency occurs. A typical solution is to use VLCs, but unless they produce easily definable codewords (e.g. Exp/Rice-Golomb coding), they require the use of important value/length pair tables. Truncated binary coding is therefore a straightforward simple solution to this problem.

An example of such a coding is, for a palette size of P = 2N - R with N ≥ 0 and 0 < R ≤ 2N integers, one can code and index “idx” as follows:

- If idx < R, idx is coded on N-1 bits;

- Otherwise, idx+R is coded on N bits.

## Escape signaling

SCCE3 software basis [1] reserves the last palette index to signal the use of an escape-coded pixel. This method was considered to be a simplification, albeit with a non-negligible coding loss.

However, most of the loss can be overcome by employing a trick described in [2]: the bitstream can be made to guarantee that, whenever the palette size is below the maximum, there is no escape-coded pixel. Indeed, during palettization, such kind of pixels can be added to the palette until there is no such pixel left, or the maximum palette size has been reached. This is actually what already happens with the palettization method of [1].

As a consequence, the reservation of the last palette index can be only performed for palette sizes of 31 (i.e., index 31 represents the escape index). This again affects the index coding.

# Results

## Full frame IBC search

## 2 CTBs search area

# Conclusion

# References

- W. Pu (Qualcomm), X. Guo (Microsoft), P. Onno (Canon), P. Lai (MediaTek), J. Xu (Sony), “AHG10: Suggested Software for Palette Coding based on RExt6.0,” JCTVC-Q0094, 17th JCT-VC meeting, March 2014, Valencia, Spain.

- W. Pu, F. Zou, J. Sole, M. Karczewicz, R. Joshi (Qualcomm), “Non-RCE4: Refinement of the palette in RCE4 Test 2”, JCTVC-P0231, 16th Meeting: San Jose, USA, Jan. 2014

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).