| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0087 |
| --- | --- |

| Title: | On Sample Adaptive Band Value |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the results of SAO Band value presented in JCTVC-Q0068 on top of the SCM1.0. This SAO type is similar to the sample adaptive band offset. The difference is that all filtered pixels of a band are systematically equal to a decoded value instead of being equal to the addition of the decoded pixels and the decoded offset. The proposed method gives an average BDR gain of 1.1% for all sequences and all configurations compared to the SCM1.0. It is reported that the proposed method provides on average 2.5% BDR gain for 1080p text and graphics class under the SCC test conditions.

# Introduction

As shown in Error: Reference source not found and Figure 2 the histogram of a natural sequence and the histogram of a screen content sequence are different. Consequently, the current Band offset mode is not adapted to correct the distortion of screen content signal.

Figure 1: Example of a natural sequence and its related histogram.

Figure 2: Example of screen content sequence and its related histogram.

Our previous contribution JCTVC-Q0068 proposed a new SAO type dedicated to screen content coding. As shown in the Figure 3, the current SAO Band offset can not correct the distortion related to the quantification for screen content. The proposed SAO band value is a minor modification of the SAO Band offset method in order to correct the quantization error of the specific screen content signal.

| Original signal | Compressed signal |
| --- | --- |

Figure 3: Corresponding histogram of a natural asequence and a screen content coding sequence

# Proposed method: Sample Adaptive Band Value

## Filtering process

The proposed Sample Adaptive Band Value changes the filtering process of the SAO Band Offset. So for each pixel Pi of a band j, the filtered pixel P’i is obtained by the following operation:

P’i = Vj

where Vj is the value corresponding to the band j.

It is important to note that the determination of the Vj value at the encoder side is exactly the same as SAO Band Offset.

## Value coding

For each filtered band j, the Vj value is determined according to the value VCj transmitted in the bitstream. VCj is represented by a positive value and is coded with a fixed length code of 4 bits for 8 bits sequences. The value Vj is then computed according to the following pseudo code:

if(pos == 0) // first range of the full range

Vj = VCj

else if(pos = 31) //last range of the full range

Vj = (pos -1) x nbPelByBand + VCj

else

Vj = (pos x nbPelByBand) - nbPelByBand/2 + VCj

where pos is the position of the current band j (0..31) and nbPelByBand is the number of pixels per band (8 for a bitdetph=8). This pseudo code is used once for a Band Value CTB. As depicted in Figure 4, it means that the value Vj is a value between the half of the left band and the half of the right band.

Figure 4 Possible value Vj for the current band j.

## Band coding

For screen content coding, the difference between neighboring pixels when this difference exists is generally high and not limited to a small area of the histogram of pixels values. So, for the Sample Adaptive Band Value coding, the conventional band of SAO is split into 4 separate bands as depicted in Figure 5. Consequently 4 sao_band_positions are transmitted for the Sample Adaptive Value for each CTB.

Figure 5 SAO band value with 4 SAO band position.

## Coding type

The proposed Band Value is competing at encoder side with the 2 classical SAO types. In the bitstream, this new Band Value filtering is signaled as follows: when the Band Value is selected, the SAO type is set to 1 as for the Band Offset case, then we introduced a new flag called band_type flag to signal if this is the conventional Band Offset or the new Band Value method as proposed in this contribution.

# Experimental results

Table 1 gives the results for Lossy configurations of the SCM1.0 test conditions. The proposed method obtained -1%,-1.1% and -1.14% for respectively AI, RA and LDB configurations. For Screen content sequences, this gain is of course higher with 2.5% BDR gain for 1080p text and graphics class in average for all configurations. Please note that these gains are constant for the three configurations.  Moreover, the same results are obtained for the 2 CTB configurations.

Table 1 Lossy results of the proposed method.

# Conclusion

This contribution gives the results of the SAO Band value initially proposed during the previous meeting. At decoder side, this method is a minor modification of the classical SAO band offset. But as shown in the results, this mode seems really efficient for screen content sequences. According to the interesting coding efficiency related to the minor modification of the SAO band offset, we propose to adopt this additional SAO filtering process for the next SCM version.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).