| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0088 |
| --- | --- |

| Title: | Non-SCCE1: Vector residual coding for Inter and IBC modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Tangi Poirier Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to use the same adaptation of the vector residual coding for both Intra block copy and for the Inter mode. An average BRD gain of 1.3% is reported for all configurations and all sequences compared to the SCM1.0. Compared to the SCC1 Test 3.9 which uses a fixe Golomb order for IBC and another one for Inter, the proposed method gives an average gain of 3.8% for Screen content 1080p class for Inter configurations.

# Introduction

In the current design of IBC, the same vector residual coding is used for both IBC and Inter mode. In the core experiment SCCE1, which  is dedicated to the improvement of Intra block Copy method, Test 3.5 and Test 3.6 propose to modify the block vector residual coding. In Test 3.6 the Golomb order of the syntax element “abs_mvd_minus2” is set equal to 3 for the IBC mode. For Inter mode, this Golomb order is kept equal to 1.

This Golomb order modification for IBC gives interesting gains but it can be extended to the Inter mode for screen content coding. Indeed, for screen content the “abs_mvd_minus2” has larger values than for natural content in the same way as Intra Block Copy.

In this contribution, the same Golomb order is kept for both Intra block Copy and Inter mode as the current design.

# Proposed method

In the proposed method, the Golomb order is transmitted at slice level according to the flag slice_MVD_gol_order_flag. If the slice_MVD_gol_order_flag is equal to 0, the Golomb order for the “abs_mvd_minus2” syntax element is set equal to 1 and if slice_MVD_gol_order_flag is equal to 1, the Golomb order for the “abs_mvd_minus2” syntax element is set equal to 4.

At encoder side, the Golomb order is determined based on a non multipass encoding process. The selected Golomb order is the Golomb order which gives the best rate cost for the previous frame.

# Experimental results

Table 1 gives the results of the proposed method compared to SCM1.0. The method is efficient for Inter configurations, especially for screen content sequences. For 1080p screen content class the gain is in average -3.8%.

Table 1 Lossy results of the proposed method compared to SCM1.0.

SCCE1 Test 3.6, Change the MVD decoding for IBC only. This method consist to set equal to 3 the Golomb order if the syntax element “abs_mvd_minus2”. Table 2 gives the results of the proposed method compared to SCCE1 Test 3.6 for Inter configurations. The gain is quite interesting for Screen content sequence. So it seems interesting to adapt jointly the Golomb order for both IBC and Inter modes.

Table 2 Lossy results of the proposed method compared to SCCE1 Test 3.6.

# Conclusion

This contribution gives the results of a joint adaption of the MVD coding for both IBC and Inter mode. The method gives interesting gains for Inter configurations compared to change only the MVD coding method of IBC (cf SCCE1 Test 3.6). With the proposed method the same MVD decoding is used for both IBC and Inter mode and it is adapted at frame level. The gain is especially higher for screen content sequences. The gain reaches about 4% for 1080p sequence for Inter configuration. According to the interesting gains and the simple modification, we recommend to adopt the proposed modification.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).