| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0089 |
| --- | --- |

| Title: | Non-SCE1: IBC BV throughput issue |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, T. Poirier, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

The current design of the Intra Block Copy mode allows 4x4 partitions, using the inter MV coding for the signaling of the BVs of each PU. This causes a doubling in the required parsing throughput, potentially incurring incompatibilities with HEVC and other extensions. This proposal restricts the 4x4 BVs to be horizontal so as to halve current requirements of Intra Block Copy, back to those of HEVC. It is reported that this changes combined with adapted search area gives BDR gains with an encoding run time decreases compared to the SCM 1.0 for AI configuration. A second solution restricts the 4x4 BVs to be the horizontal or the vertical component of the 4x4 BVs. This second proposal increases the coding efficiency of the first solution.

# IBC design

## Status

In the 16th meeting held in San Jose in January 2014, NxN partitioning for the Intra Block Copy mode was adopted in the Range Extension Model at the time. The state of Intra Block Copy, based on the decisions during the 17th meeting, was used as a basis for the SCM for the Screen Content Coding extension.

Among its characteristics, the following properties are of interest:

- 2NxN and Nx2N PUs for CUs of 8x8 or more, and NxN PUs in addition for 8x8 CUs;

- MVD coding of the inter mode is reused to code the BV differences.

## Parsing throughput issue

HEVC and its extensions currently have the following worst case for the Inter PUs:

8x8 bidir

4x8 unidir

4x8 unidir

Figure 1: MV parsing worst case for inter mode in HEVC

All in all, this results in 4 MVDs to be coded per 8x8 block in each of these cases.

Corresponding worst case of IBC is doubling that:

Figure 2: BV parsing worst case for IBC mode in Screen Content Coding

In addition to doubling the worst case, the actual average case is also potentially doubled, as BV magnitude is very high. No reasonable amount of delay, or restriction in the number of BVs as seen in other standards, may alleviate easily the solution.

# Proposed solution 1

## Mono-dimensional restriction for 4x4 PU BVs

By restricting the BVs to be strictly 1D for an NxN CU, it is actually possible to pack 4 BV residuals into 2 MVDs and respect the inter mode constrain.

For the first solution, the 4 horizontal MVD components of the 4 IBC 4x4 PUs are packed into 2 MVD residuals.

## BV prediction

The BVs being strictly mono-dimensional for the 4x4 PUs, a 2D predictor makes no sense. Therefore, if a predictor is not strictly 1D, it can be replaced with a default value. We have selected -12 for it, but -8 as suggested by [1] is another possibility.

# Proposed solution 2

To improve the efficiency of the mono-dimensional restriction for 4x4 PU Block vector, in this second method, the horizontal or the vertical component can be selected at CU level. Yet to avoid any additional syntax, the direction is inferred in the sign of the MVD component corresponding to the first PU. This implies that the BV residual for it should have a predetermined sign, i.e. the predictor must guarantee this property of the residual, e.g. has a maximum magnitude of 4. However, for the next PUs, the following block vector components are predicted by the last block vector predictor. Please note that this second modification is only a decoding process change compared to the first method.

# Encoder selection

## Consequence on the search range

Figure 3: search areas with and without the restriction

The 2 above examples illustrate situations which are close to the least and worst changes in search areas. If we take for reference a 64x64 CTB, this results in:

- Example on the left:

  - Searched area is originally (64-4)x64 = 3584 test positions;

  - With horizontal restriction, (64-4) positions;

  - With restriction to either horizontal or vertical, same.

- Right example:

  - Searched area is (2x64-8-4)x64 + (8+4)x(64-8-2) + 4x4, around 8064 positions;

  - With horizontal restriction, (2x64-8-4) = 116 positions;

  - With restriction to either horizontal or vertical, (64-8-4)+116 = 168 positions

As a consequence the search area for both restrictions, and the related tested positions, is drastically reduced, to less than 2%.

This may results in a significant BD-rate loss only because of the fewer positions tested. While the correct setting is subject to analysis (e.g. using SCCE1 tools and experts opinion), we chose the following settings to limit the impact:

- 2CTBs search: lowering of various thresholds so that IBC search for various partitioning happens more frequently;

- Full frame search: the full causal line and/or column is explored.

# Results

Table 1 gives the lossy results of solution 1 compared to SCM1.0. The method gives gain for all configurations and an encoding runtime reduction is observed for AI configuration.

Table 1 Lossy results of solution 1.

Table 2 gives the Lossy results of solution 2. The gain is larger than solution 1 for all configurations.

Table 2 Lossy results of solution 2.

Error: Reference source not found gives the average BDR and the encoding run time for 4 configurations. The first configuration is the encoder modification only of the solution 1. It means that the search area of the 4x4 BV is restricted to the horizontal component ((X, 0) for all X). In the same way, the 3rd configuration is the encoder only modification of the solution 2. As the conclusion of this summary, solution 1 and solution 2 improves the coding efficiency of the encoder modification only. And of course solution 1 and solution 2 divide by 2 the worst case parsing throughput of the BV residual.

Table 3 Summary of experiments.

| MV Components |  |  | BDR Y AI | ENC Time AI |
| --- | --- | --- | --- | --- |
| Hor and Ver (Scm1.0) | Solution 1 Encoder only |  | -0.25% | 96% |
| Hor | Solution 1 |  | -0.60% | 97% |
| Hor and Ver (Scm1.0) | Solution 2 Encoder Only |  | -0.51% | 97% |
| Hor OR Ver | Solution 2 |  | -0.80% | 99% |

- Conclusion

This contribution proposes to reduce the worst case parsing throughput of the BV residual of IBC by keeping only 1 component on 2 for IBC 4x4 PU. 2 solutions are proposed and evaluated. For the first solution only the horizontal component of all 4x4 BV is keep. For the second one, only one component per BV residual is coded but it can be the horizontal or the vertical component. The component direction is inferred in the sign of the first component. So no additional syntax element is needed.

For both solutions, 2 MVD are decoded for 4 IBC 4x4 PU instead of 4 in the current SCM1.0. Both solutions give gain with an encoder run time decrease for AI configuration compared to the SCM 1.0.

# References

- B. Li, J. Xu, G. J. Sullivan, “On Intra BC mode,” JCTVC-O0183, 15th JCT-VC meeting, October 2013, Geneva (CH).

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).