| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0091 |
| --- | --- |

| Title: | SCCE3: Crosscheck of JCTVC-R0119 on Test A.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Email: | christophe.gisquet@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on contribution JCTVC-R0119 about the “Palette coding and palette predictor updates”. The verification work was performed by using the software provided by the proponent for the SCCE3 Test A.2. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. As a result, the BD rate results reported during this crosscheck match those provided in the contribution JCTVC-R0119.

# Analysis of the proposal

The contribution JCTVC-R0119 [1] proposes several tools:

- An update mechanism where the N palette elements of the last palette-coded CU (N being the size of the palette) replace the N first elements of the predictor, who size is 32;

- A flag to indicate the current palette is identical to the palette predictor (hence, no need to signal flags nor palette size), i.e. the signaling is skipped;

- A new signaling when the palette is not skip:

  - Total number of elements in the palette of the CU

  - For each element of the palette, whether it is predicted by the predictor element with same index

  - If not, explicit signaling of the new element components

The skipping of the palette is evaluated at the encoder by running a second palette evaluation with all elements predetermined and identical to the palette predictor

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the SCCE3 Testing Conditions [2]. Table 1 to Table 4 show the results of the proposed scheme compared to SCCE3 palette anchors for lossy and lossless configurations, and by using both the full IBC search or the 2CTB search settings.

Only the AI configuration is completed, further results will be uploaded in a revised document.

## Lossy results for IBC 2CTB search

Table 1: Lossy results (2 CTB search)

## Lossy results for IBC full search

Table 2: Lossy results (full search)

## Lossless results for IBC 2TCB search

Table 3: Lossless results (2CTB search)

## Lossless results for IBC full search

Table 4: Lossless results (full search)

# Conclusion

This contribution reports cross-verification results on contribution JCTVC-R0119, “Palette coding and palette predictor updates”. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. The BD rate results provided by this cross-check match the ones provided by the proponent of JCTVC-R0119.

# References

- Y.-C. Sun, P. Lai, T.-D. Chuang, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei (MediaTek), “SCCE3 Test A.2: Palette coding and palette predictor updates”, JCTVC-R0119, JCT-VC Meeting, Sapporo, June-July 2014.

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode”, JCTVC-Q11123, Valencia, Spain, 27 Mar. – 4 Apr. 2014.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).