| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 |  |  | Document: JCTVC-R0096 |
| --- | --- | --- | --- |
| Title: | Non-SCCE1: Intra block copy with modified motion vector predictor |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Jiang Xuerong Zhang Yaqiang Wu Zhepeng Wang Lenovo Research Institute Building B5,No.6,ShangDi West Rd, Beijing, P.R. China, 100085 | Tel: Email: | (86)18610861019 Jiangdong1@lenovo.com Zhangxr2@lenovo.com |
| Source: | Lenovo |  |  |

_____________________________

# Abstract

This contribution proposes an improved scheme of MODE_INTRABC by permitting the zero motion vector predictor in intra block copy motion search. Since there are no reconstructed pixels in current CU position, an interpolation using its neighbour pixels is needed. Due to the more accurate motion vector predictor, the encoder and decoder complexity is visibly reduced while the BD-rate reduction of -0.2%, -0.2%, 0.0% for LB, RA and AI lossy configurations respectively.

# Introduction

For intra block copy coding, the final motion vector of a previously MODE_INTRABC coded CU is recorded as the lastIntraBCMV, which will be used as the MV predictor of current CU. Experimental data show that MODE_INTRA occupies over 60% in all-intra scenario，means that only few MODE_INTRABC blocks are surrounded by a large number of MODE_INTRA blocks. This leads to the inaccurate motion vector predictor if the MVP derives from the last MODE_INTRABC coded CU. In RA and LB configurations, it’s the similar situation because there are fewer intra block copy coded CU compared to other coding modes.

# Proposal

The contribution proposes an improvement scheme of intra block copy by extending the search area to current CU position. With this method, the reference pixels of current CU position need to be interpolated by copying from the most right column of its left CU. During motion estimation, both the motion vector predictor and final motion vector allow to be zero. Moreover, if none of the neighbour of current CU is intra block copy coded, the initial motion vector will be set to zero.

Accordingly, in current SCM1.0 software the code which avoids MVP to be (0,0) will be removed like this:

If the motion vector is (0,0), current CU will be coded as MODE_INTRABC SKIP mode[1] regardless of whether the real residue is zero.

# Test results

The coding gain of the proposed method is implemented on SCM-1.0. The experimental conditions are subject to the test conditions issued by JCT-VC in the 17th meeting [2].

Results below are BD-rate and encoding/decoding time comparison between proposed scheme and anchor. The proposed method can get a slight BD-rate reduction as shown in the following tables. At the same time,the encoding time remains by 96%,97%,97% for lossy AI, RA and LB respectively, while the decoding time remains by 99%,95%,97% for lossy AI, RA,and LB respectively.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.1% | 0.1% | -0.1% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.2% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | -0.1% | 0.2% | 0.1% |
| YUV, Animation, 720p | 0.2% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 95% |  |  |

Table 3: Results for lossy Low delay B configuration

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.1% |
| RGB, text & graphics with motion,720p | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.2% | -0.5% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | 0.0% |
| YUV, mixed content, 1080p | 0.1% | -0.4% | 0.4% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.4% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 97% |  |  |

Table 4: Results for lossless configuration

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

# Conclusion

In this contribution, an improved derivation of motion vector predictor of a MODE_INTRABC coded CU was proposed by extending the reference area of zero motion position through interpolating the reference pixels of current CU position.

The proposed schemes lead to the encoding and decoding time reduction for all configurations.

According to the more accurate motion vector predictor and the reduction of complexity, it is recommended to consider the method.

# References

- B. Li, J. Xu, F. Wu, X. Guo, G. J. Sullivan, “Description of screen content coding technology proposal by Microsoft,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0035, Mar. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, Apr. 2014.

# Patent rights declaration(s)

Lenovo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).