| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0097 |
| --- | --- |

| Title: | SCCE1: Result of Test 2.2 2.3 and 4.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document reports the experimental results of test 2.2 2.3 and 4.2 in SCCE1. Skip mode, merge mode and flip mode for Intra BC have been tested in the CE. Experimental results show the proposed Intra BC merge mode improves the coding efficiency by 7.2% for RGB TGM 1080p AI lossy coding.

# Introduction

This document reports the experimental results of test 2.2 2.3 and 4.2 in SCCE1. Skip mode, merge mode and flip mode for Intra BC have been tested in the CE. Experimental results show the proposed Intra BC merge mode improves the coding efficiency by 7.2% for RGB TGM 1080p AI lossy coding.

# Solution

This document proposes three modifications for Intra BC, including skip mode, merge mode, and flip mode.

- Intra BC skip mode

A new mode (Intra BC skip mode) is introduced. Intra BC skip mode means the current CU uses Intra BC prediction and the partition size is 2Nx2N and there is no residue.

- Intra BC merge mode

Intra BC merge mode is used to signal the block vector. Intra BC merge mode is only enabled for Intra BC CU with partition size of 2Nx2N. Two merge candidates are enabled, the BV from the left block and the BV from the above block.

- Intra BC with flipping

Vertical flipping is enabled for the CU using Intra BC prediction. One flag is signaled to distinguish flipping or not for every PU using Intra BC prediction (except the CU in Intra BC merge mode).

# Experimental Results

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as anchor. The common test condition specified in [1] is followed. The summarized experimental results are shown in Table 1~6 respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental result for lossy coding (Intra BC Merge mode)

Table 2 Experimental results for lossless coding (Intra BC Merge mode)

Table 3 Experimental result for lossy coding (Intra BC Skip mode)

Table 4 Experimental results for lossless coding (Intra BC Skip mode)

Table 5 Experimental result for lossy coding (Intra BC Flip mode)

Table 6 Experimental results for lossless coding (Intra BC Flip mode)

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The modifications are relative to JCTVC-P1005_v1.doc.

The modifications on SCCE1.2.2 (Intra BC skip mode) is marked as yellow.

The modifications on SCCE1.2.3 (Intra BC merge mode) is marked as green.

The modifications on SCCE1.4.2 (Intra BC block flipping) is marked as pink.

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) { |  |
| intra_bc_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_skip_flag[ x0 ][ y0 ] ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) && !intra_bc_skip_flag[ x0 ][ y0 ] ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( PartMode == PART_2Nx2N ) { |  |
| intra_bc_merge_flag[ x0 ][ y0 ] | ae(v) |
| if(intra_bc_merge_flag[ x0 ][ y0 ]) |  |
| intra_bc_merge_index[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( !intra_bc_merge_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ !( PartMode = = PART_2Nx2N && intra_bc_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.8.9 Motion vector difference syntax

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| if( refList == 2 ) |  |
| intra_bc_flip_flag[ x0 ][ y0 ] | ae(v) |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

7.4.9.5 Coding unit semantics

intra_bc_skip_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in skipped intra block copying mode (intra block copying with PartMode equaling to PART_2Nx2N and without any residue). intra_bc_skip_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in skipped intra block copying mode. When not present, the value of intra_bc_skip_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_bc_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in intra block copying mode. intra_bc_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded according to pred_mode_flag. When not present, when intra_bc_skip_flag[ x0 ][ y0 ] is equal to 1, the value of intra_bc_flag is inferred to be equal to 1. Otherwise, the value of intra_bc_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_bc_merge_flag[ x0 ][ y0 ] equal to 1 specifies that the block vector current prediction unit is coded in merge mode (merge with the left block vector or the above block vector). intra_bc_merge_flag[ x0 ][ y0 ] equal to 0 specifies that the block vector of current prediction unit is not coded in merge mode. When not present, the value of intra_bc_merge_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_bc_merge_index[ x0 ][ y0 ] equal to 1 specifies that the block vector of current prediction unit is the same as the above block vector. intra_bc_merge_index[ x0 ][ y0 ] equal to 0 specifies that the block vector of current prediction unit is the same as the left block vector. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

7.4.9.9 Motion vector difference semantics

intra_bc_flip_flag [ x0 ][ y0 ] equal to 1 specifies that the current prediction unit is predicted in vertically flipped mode. intra_bc_flip flag[ x0 ][ y0 ] equal to 0 specifies that the current prediction unit is predicted in normal mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

8.4.4 Derivation process for block vector components in intra block copying prediction mode

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- If intra_bc_merge_flag[ xPb ][ yPb ] is equal to 1, the following applies.

intra_bc_merge_candidate[0][ compIdx ] is set equal bvIntra[ xPb - 1 ][ yPb ][ compIdx ]

intra_bc_merge_candidate[1][ compIdx ] is set equal bvIntra[ xPb ][ yPb - 1 ][ compIdx ]

idx is set equal to intra_bc_merge_flag[ xPb ][ yPb ]

bvIntra[ xPb ][ yPb ][ compIdx ] is set equal to intra_bc_merge_candidate[idx][ compIdx ]

Otherwise (intra_bc_merge_flag[ xPb ][ yPb ] is equal to 0), Ddepending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] − nCbS	(8-1)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ]	(8-2)

- Otherwise, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] + BvpIntra[ 0 ]	(8-3)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ] + BvpIntra[ 1 ]	(8-4)

The value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ].

8.4.5.2.7 Specification of intra block copying prediction mode

……

–	The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, is derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

Depending on intra_bc_flip_flag[ xTb0][ yTb0 ], the following applies. If intra_bc_flip_flag[ xTb0][ yTb0 ] is euqal to 1

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + nTbS-1- y + bv[ 1 ] )

Otherwise, (intra_bc_flip_flag[ xTb0][ yTb0 ] is euqal to 0)

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-5)

- Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Table 9- | 0 | 1 | 62 |
|  | intra_bc_skip_flag[][] | Table 9-XX | 0 | 1 | 2 |
|  | intra_bc_merge_flag[][] | Table 9-XX | 0 | 1 | 2 |
|  | intra_bc_merge_index[][] | Table 9-XX | 0 | 1 | 2 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 9..11 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | Error: Reference source not found |  | 0 | 2 |
|  | abs_mvd_greater1_flag[ ] | Error: Reference source not found |  | 1 | 3 |
|  | intra_bc_flip_flag[][] | Table 9-XX | 0 | 1 | 2 |

Table 9-XX – Values of initValue for ctxIdx of intra_bc_skip_flag

| Initialization variable | ctxIdx of intra_bc_skip_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-XX– Values of initValue for ctxIdx of intra_bc_merge_flag

| Initialization variable | ctxIdx of intra_bc_merge_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-XX – Values of initValue for ctxIdx of intra_bc_merge_index

| Initialization variable | ctxIdx of intra_bc_merge_index |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-XX– Values of initValue for ctxIdx of intra_bc_flip_flag

| Initialization variable | ctxIdx of intra_bc_flip_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

| Table 9-2 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | intra_bc_skip_flag | FL | cMax = 1 |
|  | intra_bc_merge_flag | FL | cMax = 1 |
|  | intra_bc_merge_index | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | intra_bc_flip_flag | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_bc_skip_flag | 0 | na | na | na | na | na |
| intra_bc_merge_flag | 0 | na | na | na | na | na |
| intra_bc_merge_index | 0 | na | na | na | na | na |
| intra_bc_flip_flag | 0 | na | na | na | na | na |