| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0099_r1 |
| --- | --- |

| Title: | SCCE5: Result of Test 3.2.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document reports the experimental results of test 3.2.2 in SCCE5. Adaptive color space is designed to improve the coding efficiency of RGB (GBR) video. Besides GBR, coding in YCoCg, RGB, and BGR spaces are enabled for intra predicted CU for lossy coding. For lossless coding, only reordering (e.g. RGB, BGR) is allowed. Adaptive color space coding is studied in the CE. Experimental results show the proposed adaptive color space coding improves the coding efficiency by 20.0% for RGB Animation 720p AI lossy coding. Performance gain is also obtained for other test cases.

# Introduction

This document reports the experimental results of test 3.2.2 in SCCE5. Adaptive color space is designed to improve the coding efficiency of RGB (GBR) video. Besides GBR, coding in YCoCg, RGB, and BGR spaces are enabled for intra predicted CU for lossy coding. For lossless coding, only reordering (e.g. RGB, BGR) is allowed. Adaptive color space coding is studied in the CE. Experimental results show the proposed adaptive color space coding improves the coding efficiency by 20.0% for RGB Animation 720p AI lossy coding. Performance gain is also obtained for other test cases.

# Solution

Adaptive color space coding is introduced to improve the RGB (GBR) video coding efficiency, as shown in figure 1. Besides GBR, coding in YCoCg, RGB, and BGR spaces are enabled for Intra predicted lossy coded CU. As GBR to YCoCg conversion is not reversible without a bit-depth increase, it is not used in lossless coding. The pixel values are stored in the original color space (e.g. GBR), and when needed, the necessary pixels (at most 4N+1 pixels used for intra prediction) are converted to a specific color space. The reconstructed pixels are converted back to the main color space (e.g. GBR) before storing. Thus, no additional storage memory is required. The conversion is in the prediction loop so that it requires decoding process changes.

Figure 1. Adaptive color space coding at the decoder.

Please note that the normalization during YCoCgRGB conversion is also taken into consideration. When YCoCg color space is used, QP offsets −5, −3, and −5 are applied to Y, Co, and Cg component respectively.

# Experimental Results

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as anchor. The common test condition specified in [1] is followed. The summarized experimental results (full picture Intra BC) are shown in Table 1~6. More detailed experimental results (including 2CTU Intra BC results) could be found in the attached spreadsheets.

Table 1 Experimental result for lossy coding (both reorder and YCoCg, full picture Intra BC)

Table 2 Experimental results for lossless coding (both reorder and YCoCg both reorder and YCoCg, full picture Intra BC)

Table 3 Experimental result for lossy coding (reorder, full picture Intra BC)

Table 4 Experimental results for lossless coding (reorder, full picture Intra BC)

Table 5 Experimental result for lossy coding (YCoCg, full picture Intra BC)

Table 6 Experimental results for lossless coding (YCoCg, full picture Intra BC)

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text modifications are based on JCTVC-P1005_v1.

#### 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| adaptive_color_space_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| } |  |
| …… |  |

#### 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| …… |  |
| } else { |  |
| if( adaptive_color_space_enable_flag) |  |
| cu_color_space[ x0 ][ y0 ] | ae(v) |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| …… |  |

#### 7.4.3.2	Sequence parameter set RBSP semantics

adaptive_color_space_enabled_flag equal to 1 specifies that adaptive color space coding may be enbaled in the decoding process for intra prediction. adaptive_color_space_enabled_flag equal to 0 specifies that adaptive color space coding is not applied. When not present, the value of adaptive_color_space_enabled_flag is inferred to be equal to 0.

7.4.9.5 Coding unit semantics

cu_color_space[ x0 ][ y0 ] specifies the color space used for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

8.4.1 General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

A variable nCbS is set equal to 1  <<  log2CbSize.

Depending cu_color_space[ x0 ][ y0 ], the following applies

If cu_color_space[ x0 ][ y0 ] is not equal to 0 (adaptive color space coding is used, the current chroma format should be 444), following storage process is performed. The surrounding will be stored in an array savedPixels[] with a size of 2*nCbS+1.

savedPixels[x] will be set as the pixel value at (xCb-1+x, yCb-1 ), where x is in range of 0...nCbS, if the pixel is inside the picture.

savedPixels[nCbS+1+y] will be set as the pixel value at (xCb-1, yCb+y), where y is in range of 0…(nCbs-1), if the pixel is inside picture.

- If cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0 and cu_color_space[ x0 ][ y0 ] is equal to 1 (GBR to YCoCg), the pixels in the above range are modified as follows

  - offset=1<<(BitDepthY-1)

  - maxValue=(1<<BitDepthY)-1

  - R = SCr[i][j]

  - G =SL[i][j]

  - B = SCb[i][j]

  - SL[i][j] = Clip3( 0, maxValue, (2*R+4*G+2*B)/4 )

  - SCb[i][j] = Clip3( 0, maxValue, (R-B)/2+offset )

  - SCr[i][j] = Clip3( 0, maxValue, (2*G-R-B)/2+offset )

- Otherwise, if cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0 and cu_color_space[ x0 ][ y0 ] is equal to 2 (GBR to RGB) or cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and cu_color_space[ x0 ][ y0 ] is equal to 1 (GBR to RGB), the pixels in the above range are modified as follows

  - R = SCr[i][j]

  - G =SL[i][j]

  - B = SCb[i][j]

  - SL[i][j] = R

  - SCb[i][j] = G

  - SCr[i][j] = B

- Otherwise, if cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0 and cu_color_space[ x0 ][ y0 ] is equal to 3 (GBR to BGR) or cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and cu_color_space[ x0 ][ y0 ] is equal to 2 (GBR to BGR), the pixels in the above range are modified as follows

- R = SCr[i][j]

- G =SL[i][j]

- B = SCb[i][j]

- SL[i][j] = B

- SCb[i][j] = G

- SCr[i][j] = R

…… (Normal Intra decoding process)

Depending cu_color_space[ x0 ][ y0 ], the following applies

If cu_color_space[ x0 ][ y0 ] is not equal to 0 (adaptive color space coding is used, the current chroma format should be 444), following recover process is performed. The surrounding will be recovered from the array savedPixels[] with a size of 2*nCbS+1.

savedPixels[x] will be set to the pixel at (xCb-1+x, yCb-1 ), where x is in range of 0...nCbS, if the pixel is inside the picture.

savedPixels[nCbS+1+y] will be set to the pixel at (xCb-1, yCb+y), where y is in range of 0…(nCbs-1), if the pixel is inside picture.

- If cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0 and cu_color_space[ x0 ][ y0 ] is equal to 1 (GBR to YCoCg), the pixels in the range of i=yCb…(yCb+nCbS-1) and i=xCb…(xCb+nCbS-1) are modified as follows

  - offset=1<<(bitDepthY-1)

  - maxValue=(1<<BitDepthY)-1

  - Y = SL[i][j]-offset

  - Co = SCr[i][j]

  - Cg = SCb[i][j]-offset

  - SCr[i][j] = Clip3( 0, maxValue, Y+Co-Cg )

  - SL[i][j] = Clip3( 0, maxValue, Y+Cg )

  - SCb[i][j] = Clip3( 0, maxValue, Y-Co-Cg )

- Otherwise, if cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0 and cu_color_space[ x0 ][ y0 ] is equal to 2 (GBR to RGB) or cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 1 and cu_color_space[ x0 ][ y0 ] is equal to 1 (GBR to RGB), the pixels in the range of i=yCb…(yCb+nCbS-1) and i=xCb…(xCb+nCbS-1) are modified as follows

  - R = SL[i][j]

  - G = SCr[i][j]

  - B = SCb[i][j]

  - SL[i][j] = G

  - SCb[i][j] = B

  - SCr[i][j] = R

- Otherwise, if cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0 and cu_color_space[ x0 ][ y0 ] is equal to 3 (GBR to BGR) or cu_transquant_bypass_flag[ x0 ][ y0 ] is equal to 0 and cu_color_space[ x0 ][ y0 ] is equal to 2 (GBR to BGR), the pixels in the range of i=yCb…(yCb+nCbS-1) and i=xCb…(xCb+nCbS-1) are modified as follows

  - R = SCr[i][j]

  - G = SCb[i][j]

  - B = SL[i][j]

  - SL[i][j] = G

  - SCb[i][j] = B

  - SCr[i][j] = R

8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y + (!cu_transquant_bypass_flag[ xTbY ][ yTbY ] && cu_color_space[ xTbY ][ yTbY ]==1 ? −5 : 0)		(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb+ (!cu_transquant_bypass_flag[ xTbY ][ yTbY ] && cu_color_space[ xTbY ][ yTbY ]==1 ? −3 : 0)		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr+ (!cu_transquant_bypass_flag[ xTbY ][ yTbY ] && cu_color_space[ xTbY ][ yTbY ]==1 ? −5 : 0)		(8-3)

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 9..11 | 1..4 | 5..8 |
|  | cu_color_space | Table 9-XX | 0..2 | 3..5 | 6..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |

Table 9-X – Values of initValue for ctxIdx of cu_color_space

| Initialization variable | ctxIdx of cu_color_space |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

| Table 9-2 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | cu_color_space | TR | cMax = cu_transquant_bypass_flag ? 3 : 4, cRiceParam = 0 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_color_space | 0 | 1 | 2 | na | na | na |