| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0104 |
| --- | --- |

| Title: | On HAD calculation in the lossless RDO process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes to get rid of HAD calculation in the RDO process for lossless coding. The experimental results show that the proposed method brings some bit saving (1.3% for RGB TGM 1080p RA, 1.4 for YUV TGM 1080p RA, 1.4% for RGB TGM LB, 1.5% for YUV TGM 1080p LB on average) for lossless coding under SCC CTC. Some encoding time saving is also observed.

# Introduction

This document proposes to get rid of HAD calculation in the RDO process for lossless coding. The experimental results show that the proposed method brings some bit saving (1.3% for RGB TGM 1080p RA, 1.4 for YUV TGM 1080p RA, 1.4% for RGB TGM LB, 1.5% for YUV TGM 1080p LB on average) for lossless coding under SCC CTC. Some encoding time saving is also observed.

# Problem

HAD is used to estimate the cost after DCT transform. However, transform is not applied for lossless mode. Thus, this document proposes not to calculate HAD (instead, calculate SAD) for lossless mode during the RDO process (e.g. intra prediction direction decision and fractional motion estimation). The proposed algorithm only includes non-normative change.

# Experimental Results

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as anchor. The common test condition specified in [1] is followed. The summarized experimental result for lossless coding is shown in Table 1. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossless coding

We also performed test based on HM-14.0, and obtained similar results.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.