| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0105 |
| --- | --- |

| Title: | On motion estimation start point |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes to modify the start point of motion estimation in the SCM software. The proposed modification brings about 14% encoding time saving for both RA and LB configurations in lossy coding and about 19% and 20% encoding time saving for RA and LB in lossless coding. Some coding efficiency improvements are also observed (about 0.9% bit saving for RGB mixed content 1440p and YUV mixed content 1440p in RA lossy coding).

# Introduction

This document proposes to modify the start point of motion estimation in the SCM software. The proposed modification brings about 14% and 14% encoding time saving for RA and LB in lossy coding and about 19% and 20% encoding time saving for RA and LB in lossless coding. There is also some coding efficiency improvements brought by the proposed method (about 0.9% bit saving for RGB mixed content 1440p and YUV mixed content 1440p in RA lossy coding structure).

# Problem

In the current implementation of SCM, the start point of motion estimation is selected from one of the two MVPs, or the (0, 0) point. (It also seems that bTestOtherPredictedMV does not work, as the surrounding MVs may not be set properly.)

The problem is the MVP. If the MVP comes from a different reference picture, it will be scaled according to POC difference. In this case, using this MVP as motion estimation start point may not be a good solution. Using an MV obtained from the same reference picture as the start point of motion estimation may be a better solution.

This document proposes to store the integer MV obtained for every 2Nx2N PU by reference picture. And when performing motion estimation for other PU sizes (or smaller 2Nx2N PU), the 2Nx2N MV from the same reference picture will be used as another motion estimation start point candidate.

The modification proposed in this document is non-normative, introducing only about 50 lines of code.

# Experimental Results

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as anchor. The common test condition specified in [1] is followed. The summarized experimental results for lossy coding and lossless coding are shown in Table 1 and 2 respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental result for lossy coding

Table 2 Experimental results for lossless coding

We also performed the test based on HM-14.0. Similar experimental result (encoding time saving with minor bit saving) is also obtained.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.