| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0106_r1 |
| --- | --- |

| Title: | Adaptive motion vector resolution for screen content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary J. Sullivan You Zhou Bruce Lin | Email: | libin@microsoft.com jzxu@microsoft.com garysull@microsoft.com youzhou@microsoft.com blin@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes and reports performance measurements for disabling fractional pixel motion compensation in high-level syntax, as previously proposed in JCTVC-P0277 and JCTVC-Q0155. It is observed that when encoding video content captured from screens, most of the spatial displacements are full pixel displacements – few of them are at fractional pixel positions. Considering the fact that no bits need to be spent on the fractional parts of motion vectors for such content, the proposed scheme can reduce the bit rate needed to encode such content. Since fractional-sample interpolation is avoided, the decoding complexity is reduced when the scheme is applied. Depending on how the encoder operates, the encoding process can also be simplified, as the fractional-sample search is unnecessary when disabled for the decoding process. The proposed modification, which is slightly different than previously proposed (simplifying the MV derivation by eliminating the need for a rounding operation for MV predictors), does not change the syntax or parsing process (except for adding a control indication at the header level). The modification is shown to improve coding efficiency as much as 7.3% for some screen content. For RGB text and graphics with motion (TGM) 1080p sequences, 2.8% and 4.2% bit saving on average is obtained for RA and LB lossy coding. To speed up the encoding process, hash based decision on MV resolution is developed. The single-pass encoding is shown to improve coding efficiency as much as 7.0% for some screen content. For RGB TGM 1080p sequences, 1.6% and 3.0% bit saving on average is obtained for RA and LB lossy coding. The experimental results show that most coding gain could be obtained without increasing the encoding complexity.

# Introduction

The idea is based on our observation that when encoding video that is captured from a computer desktop, most of the motion vectors in the encoded bitstream point have integer values; very few of them are at fractional positions. While encoders normally interpret motion vectors in bitstreams as ¼ pixel offsets, we can often save bit rate by encoding these motion vectors for screen coding applications as 1 pixel offsets, reducing the precision of the motion vectors by a factor of four, and thereby reducing the number of bits needed to code them. Considering that fractional motion vector values are still very useful for camera-captured content, we propose to enable encoders to select the motion vector precision in a flexible way: when the video source is from a captured screen and there is no fractional pixel motion, we transmit the motion vector in units of 1 pixel, and for camera-content video we transmit motion vectors with ¼-pixel fractional precision.

# Solution

The encoder signals a flag at the slice header level to indicate whether integer-precision MV or quarter-pel precision MVs are used for the current slice. The encoder can then make decisions for the MV precision on a slice-by-slice basis. As tested, each slice is an entire picture. When integer-precision MV is used, the decoder only needs interpret the decoded MV as an integer MV when performing the motion compensation process – it does not need to change the MV prediction process. This is slightly different than previously proposed. Here, we propose to change only the interpretation of the MV values – not their prediction or parsing processes. This eliminates the need to round MV predictors from quarter-pel precision to integer-pel precision. As proposed here, the decoder performs the ordinary process for MV prediction and delta MV decoding, regardless of whether the derived value represents a quarter-pel MV or an integer MV. (The decoder also ignores which MV precision was used for the reference pictures when performing temporal MV prediction – it simply performs operations on values that may either represent quarter-sample units or whole-integer units.)

Single-pass encoding strategy: To decide the MV precision of one picture, the encoder performs the following check with the help of hash. For every non-overlapped 8x8 block in a picture, the encoder checks whether it can find a matching block by hash in the first reference picture in list 0. The number of matched blocks is denoted as M. For the unmatched blocks, the decoder check the number of different colors in the first component (Y or G) in the block. If the number of different colors is less than or equal to 8, we treat it as a screen content block. Otherwise, we treated it as camera-captured content block. The number of screen content blocks and camera-captured content blocks are denoted as S and C, respectively. T=M+S+C. If all the following conditions are satisfied, we set the MV precision of the current picture to integer-pel precision. Otherwise, we set it to quarter-pel precision.

- T>=10

- C/T <= 3%

- M/T>25%

# Experimental results

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results for lossy and lossless coding are shown in Table 1 and 2, respectively. More detailed experimental results could be found in the attached spreadsheets. Table 3 and 4 show the experimental results with single-pass encoding.

Table 1 Experimental results for lossy coding

Table 2 Experimental results for lossless coding

Table 3 Experimental results for lossy coding (single-pass encoding)

Table 4 Experimental results for lossless coding (single-pass encoding)

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- JCTVC-P0277, “Motion Vector Resolution Control for Screen Content Coding,” San José, US, 9–17 Jan. 2014.

- JCTVC-Q0155, “RCE1: Subtest 1 - Motion Vector Resolution Control,” Valencia, ES, 27 March – 4 April 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The text provided in this section is based on JCTVC-Q1005_v9.

##### 7.3.2.2.1	General sequence parameter RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| sps_range_extensions_flag | u(1) |
| sps_screen_extensions_flag | u(1) |
| sps_extension_76bits | u(76) |
| } |  |
| if( sps_range_extensions_flag ) |  |
| sps_range_extensions( ) |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

##### 7.3.2.2.3	Sequence parameter set screen extensions syntax

| sps_range_extensions( ) { | Descriptor |
| --- | --- |
| adaptive_mv_resolution_enabled_flag | u(1) |
| } |  |

#### 7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| if (adaptive_mv_resolution_enabled_flag) |  |
| use_integer_mv_flag | u(1) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| …… |  |

##### 7.4.3.2.1	General sequence parameter set RBSP semantics

sps_screen_extensions_flag equal to 1 specifies that the sps_screen_extensions( ) syntax structure is present in the SPS RBSP syntax structure. sps_screen_extensions_flag equal to 0 specifies that this syntax structure is not present. When not present, the value of sps_screen_extensions_flag is inferred to be equal to 0.

sps_extension_76bits equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. When present, sps_extension_76bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Values of sps_extension_76bits not equal to 0 are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of sps_extension_76bits to be not equal to 0 and shall ignore all sps_extension_data_flag syntax elements in an SPS NAL unit. When not present, the value of sps_extension_76bits is inferred to be equal to 0.

##### 7.4.3.2.3	Sequence parameter set screen extensions semantics

adaptive_mv_resolution_enabled_flag equal to 1 specifies that use_integer_mv_flag may be signaled at slice header. adaptive_mv_resolution_enabled_flag equal to 0 specifies that use_integer_mv_flag is not signaled at slice header.  When not present, the value of adaptive_mv_resolution_enabled_flag is inferred to be equal to 0.

#### 7.4.7.1	General slice segment header semantics

use_integer_mv_flag equal to 1 specified that the motion vector precision is integer in the decoding process of current slice. use_integer_mv_flag equal to 0 specified that the motion vector precision is qual-pixel in the decoding process of current slice.

#### 8.5.3.3	Decoding process for inter prediction samples

##### 8.5.3.3.1	General

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCbS specifying the size of the current luma coding block,

–	two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

–	the luma motion vectors mvL0 and mvL1,

–	when ChromaArrayType is not equal to 0, the chroma motion vectors mvCL0 and mvCL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags, predFlagL0, and predFlagL1.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCb of chroma prediction samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

When use_integer_mv_flag is equal to 1, mvLX and mvCLX (where X equals to 0 or 1) are modified as mvLX <<= 2, mvCLX <<= 2.

The variable nCbSL is set equal to nCbS. When ChromaArrayType is not equal to 0, the variable nCbSwC is set equal to nCbS / SubWidthC and the variable nCbShC is set equal to nCbS / SubHeightC.

Let predSamplesL0L and predSamplesL1L be (nPbW)x(nPbH) arrays of predicted luma sample values and when ChromaArrayType is not equal to 0, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr, and predSamplesL1Cr be (nPbW / SubWidthC)x(nPbH / SubHeightC) arrays of predicted chroma sample values.

……