| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0112 |
| --- | --- |

| Title: | Non-SCCE3: Bi-color intra mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, 2Chun-Lung Lin, 3Ching-Chieh Lin, Jih-Sheng Tu, and Chao-Hsiung Hung 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

This document proposes a bi-color intra mode for screen content coding. The proposed bi-color intra mode first transmits an index to specify which two colors are selected out of available spatial neighboring pixels of the current CU. Then an index map is established by the selected two colors for the CU. An index map coding is used to transmit the index map. Experimental results show that, compared with SCM-1.0, -2.9% and –2.7% BD-rate savings under AI-Lossy coding are achieved for YUV Text & Graphics with motion sequences (1080p) on 2-CTU and full-frame IBC conditions, respectively.

# Introduction

Because screen content video content contains a lot of smooth area with similar sample values, the samples within the smooth area could share single sample value. Therefore, a single color intra mode is proposed in JCTVC-Q0093 [1] to reconstruct the current coding unit (CU) as a smooth area with single sample value. However, it may require two colors to fully represent the samples of the smooth area within the CU. In this contribution, a bi-color intra mode is proposed to reconstruct the CU as a smooth area with two sample values.

Figure 1. The spatial neighboring samples used to construct the two-color candidate list.

# Proposed bi-color intra mode

In slice level, one flag is signaled in the slice header to flexibly enable/disable the bi-color mode for the entire slice. When the slice-level flag is enabled, one CU-level flag is further signaled to indicate the enabling/disabling of the bi-color mode. The proposed bi-color mode uses five spatial neighboring samples as shown in Figure 1 (each small square represents a sample) to construct a two-color candidate list in the order: {Am, Bn}, {Am, B0}, {Bn, B0}, {Am, A0}, {Bn, A0}, {B0, A0}, {Am, D}, {Bn, D}, {B0, D}, and {A0, D}. A two-color candidate will not be inserted into the two-color candidate list if the values are identical to the values of the previously derived two-color candidates. In the proposed mode, the size of the sample candidate list is fixed to 2. If empty entry exists in the two-color candidate list after the derivation of candidates from the spatial neighboring samples, a default candidate with value equal to 1<<(g_bitDepth-1) will be used to fill in those empty entries. An index is transmitted to specify which two-color candidate is selected out of candidate list. Then an index map for the CU is established by the selected two-color candidate. An index map coding containing two run-copy modes for palette CU is used to transmit the index map:

- “Copy Left Run” mode: An ‘index’ is signalled for the index of the map, but for the rest of the pixels, the transmission of ‘index’ is skipped. A ‘run’ value is also transmitted to decoder for this mode.

- “Copy Above Run” mode: A ‘run’ value is transmitted to decoder.

# Test results

The proposed bi-color mode is implemented and evaluated based on the the experimental conditions issued by JCT-VC in the 17th meeting [2]. There are four anchors:

Anchor 1-1: SCM-1.0 common test condition with intraBC using full frame search + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

Anchor 1-2: SCM-1.0 common test condition with intraBC using full frame search

Anchor 2-1: SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU) + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

Anchor 2-2: SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU)

Tables 1-4 compare the proposed method with Anchor 1-1. Tables 5-8 compare the proposed method with Anchor 1-2. Tables 9-12 compare the proposed method with Anchor 2-1. Tables 13-16 compare the proposed method with Anchor 2-2. Note that coding time is inaccurate due to results collected from different clusters.

Table 1. AI lossy result for Anchor 1-1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.9% | -2.0% | -2.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.7% | -2.1% | -1.9% |
| YUV, text & graphics with motion,720p | -0.6% | -0.4% | -0.9% |
| YUV, mixed content, 1440p | -0.2% | -0.7% | -0.8% |
| YUV, mixed content, 1080p | -0.2% | -0.5% | -0.6% |
| YUV, Animation, 720p | 0.1% | -0.4% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 147% |  |  |
| Dec Time[%] | 104% |  |  |

Table 9. AI lossy result for Anchor 2-1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.4% | -2.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.9% | -2.7% | -2.5% |
| YUV, text & graphics with motion,720p | -0.6% | -0.4% | -0.9% |
| YUV, mixed content, 1440p |  |  |  |
| YUV, mixed content, 1080p | -0.3% | -0.5% | -0.5% |
| YUV, Animation, 720p | 0.1% | -0.4% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 153% |  |  |
| Dec Time[%] | 107% |  |  |

# Conclusion

In this contribution, bi-color intra mode for screen content coding is proposed. Experimental results show that, compared with SCM-1.0, -2.9% and -2.7% BD-rate savings under AI-Lossy coding are achieved for YUV Text & Graphics with motion sequences (1080p) on 2-CTU and full-frame IBC conditions, respectively. According to the coding gain of the proposed mode, it may be interested to further study to combine the proposed bi-color mode with the single color mode.

# References

- Y. W. Chen, Y. C. Sun, Y. W. Huang, and S. Lei, “Single color intra mode for screen content coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0093, Mar. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, Apr. 2014.

# Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).