| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0113 |
| --- | --- |

| Title: | Non-SCCE3: Improved Palette Index Coding with Contextualization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Laude (Univ. Hannover) Institut für Informationsverarbeitung Leibniz Universität Hannover 30167 Hannover Germany | Tel: Email: | +49 511 762 19588 laude@tnt.uni-hannover.de |
| Source: | Leibniz Universität Hannover |  |  |

_____________________________

# Abstract

This contribution presents a modified palette index coding method. In particular, it is reported that the most significant bin (MSB) of the palette_index syntax element is coded using contextualization rather than using the CACAC bypass mode. The modified palette index coding reportedly shows Y/G BD-rate changes of {-0.7%, -0.5%, 0,0%, -0.1%, 0.0%, 0.0%, -1.0%, -0.4%, 0.0%, -0.2%, 0.0%, 0.0%}, {-0.4%, -0.5%, -0.1%, -0.1%, 0.0%, 0.0%, -0.4%, -0.3%, -0.2%, -0.1%, 0.1%, 0.0%} and {-0.3%, -0.5%, -0.2%, 0.1%, 0.0%, 0.0%, -0.3%, -0.2%, -0.4%, -0.1%, 0.0%, 0.0%} for AI, RA and LD compared to the SCCE3 palette mode software under SCC common test conditions.

# Introduction

Palette coding is one method which is studied in the context of the Screen Content Coding (SCC) standardization [1].

In the latest version of the palette coding method [2], the palette is generated in such a way, that the palette entries are ordered by their corresponding frequency of appearance, i.e. the entries with the highest frequency of appearance in a CU are assigned with the smallest indices. However, while applying the palette coding method, the indices are coded using fixed length binarization along with the CABAC bypass mode.

Taking into account the palette generation method, the probability that the most significant bin (MSB) of the binarized code word is equal to zero may be higher than the probability that the MSB is equal to one. Thus, it is proposed to apply more sophistical entropy coding that considers the statistical distribution of the palette indices.

In particular, it is proposed to code the MSB of the palette_index syntax element using a context.

v2: Complete simulation results.

# Text Specification

The following table shows the proposed text specification. All changes relative to the latest palette mode text specification [2] are highlighted in yellow.

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | Na | na |
| pred_mode_flag | 0 | na | na | na | Na | na |
| palette_mode_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | Na | na |
| previous_palette_entry_flag | bypass | na | na | na | na | na |
| palette_num_signalled_entries | bypass | bypass | bypass | bypass | Bypass | bypass |
| palette_entries | bypass | bypass | bypass | bypass | Bypass | bypass |
| palette_escape_val | cIdx | bypass | bypass | bypass | Bypass | bypass |
| palette_run_type_flag | 0, 1 (subclause Error: Reference source not found) | na | na | na | Na | na |
| palette_index | 0 | bypass | bypass | bypass | Bypass | bypass |
| palette_run | 0 | 1 | 2 | bypass | Bypass | bypass |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | Na | Na |

# Simulation results

The anchor for the simulation results the latest palette coding software which is used as anchor for the SCCE3. SCC common test conditions are applied [3]. Runtimes are not accurate due to a heterogeneous cluster system.

Table 1 shows the results for lossy coding.

Table 1: Lossy Simulation Results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% |
| RGB, text & graphics with motion,720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.0% | -0.9% |
| YUV, text & graphics with motion,720p | -0.4% | -0.7% | -0.8% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | -0.3% | -0.6% | -0.9% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 111% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.5% | -0.6% | -0.5% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | 0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.2% | -0.4% |
| YUV, mixed content, 1440p | -0.4% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -1.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.2% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 119% |  |  |
| Dec Time[%] | 119% |  |  |

Simulation times are inaccurate.

# Conclusion

This contribution presents an improved palette index coding method. In particular, it is reported that the most significant bin (MSB) of the palette_index syntax element is coded using contextualization rather than using the CACAC bypass mode. The modified palette index coding reportedly shows Y/G BD-rate changes of {-0.7%, -0.5%, 0,0%, -0.1%, 0.0%, 0.0%, -1.0%, -0.4%, 0.0%, -0.2%, 0.0%, 0.0%}, {-0.4%, -0.5%, -0.1%, -0.1%, 0.0%, 0.0%, -0.4%, -0.3%, -0.2%, -0.1%, 0.1%, 0.0%} and {-0.3%, -0.5%, -0.2%, 0.1%, 0.0%, 0.0%, -0.3%, -0.2%, -0.4%, -0.1%, 0.0%, 0.0%} for AI, RA, LD under SCC common test conditions.

# Referenes

[1]	Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “JCT-VC Q1123: HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode. 17th Meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Valencia, Spain,” 2014.

[2]	“Text Specification for Palette Mode used as a basis in Screen Content Coding Core Experiment 3 (SCCE3) and corresponding software.”

[3]	H. Yu, R. Cohen, K. Rapaka, and J. Xu, “JCT-VC Q1015: Common Conditions for Screen Content Coding Tests. 17th Meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Valencia, Spain,” 2014.

# Patent rights declaration

Institut für Informationsverarbeitung, Leibniz Universität Hannover may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).