| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0114 |
| --- | --- |

| Title: | Optionally disabling the usage of intra boundary filters for SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to conditionally disable the boundary filters for Intra horizontal, vertical, and DC modes altogether for both lossless and lossy coding with flag controls in SPS and slice header. Experimental results reportedly show that when the Intra boundary filters are always disabled in SPS, the lossy coding BD-rates in AI, RA and LD are as follows:

-1.0%, -0.5% and -0.2% for RGB, text & graphics with motion, 1080p
-1.5%, -1.2% and -0.7% for RGB, text & graphics with motion,720p
-2.0%, -1.2% and -0.5% for RGB, mixed content, 1440p
-1.9%, -1.2% and -0.4% for RGB, mixed content, 1080p
-0.8%, -0.4% and -0.2% for YUV, text & graphics with motion, 1080p
-0.4%, -0.2% and -0.3% for YUV, text & graphics with motion,720p
0.0%, 0.1% and -0.1% for YUV, mixed content, 1440p
-0.3%, 0.0% and -0.1% for YUV, mixed content, 1080p.

# Introduction

In HEVC, for intra DC, horizontal and vertical modes, boundary filters are applied. The boundary prediction samples of DC mode will be smoothed with an [1, 3] or [1, 2, 1] filter to reduce the blocking artifact, as shown in Fig. 1. For vertical and horizontal intra prediction directions, a gradient based boundary filter is applied. Fig. 2 shows an example for the gradient based boundary filter for vertical intra prediction direction. In the current HEVC-SCC platform SCM 1, the gradient filters in Intra horizontal and vertical modes are disabled when the derived value of disableIntraBoundaryFilter is 1. This occurs only when implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1.

.

Fig. 1.  DC filter

Fig. 2.  Gradient filter for vertical prediction

# Proposed method

It is proposed to add flags in SPS and slice header, which control the disabling of two boundary filters (Intra DC and Intra gradient filters) in both vertical and horizontal directions. In the lossless and lossy test results provided in this contribution, the boundary filters are always disabled using these control flags. Note that it is an encoder choice to turn on or off the Intra boundary filters.

# Experimental results

The proposed methods are integrated on HM-14.0+RExt-7.0+SCM-1.0 [2].

With the introduced flags, Intra boundary filters are always disabled for lossless and lossy CTC conditions [3] in the following results shown in Table 1 and Table 2.

Table 1.  Results for lossless coding under CTC

Table 2.  Results for lossy coding under CTC

# Text specification changes

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| cross_component_prediction_enabled_flag | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| intra_boundary_filter_pic_disable_flag | u(1) |

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_qp_adjustment_enabled_flag ) |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| if(!intra_boundary_filter_pic_disable_flag) |  |
| intra_boundary_filter_slice_disable_flag | u(1) |

intra_boundary_filter_pic_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_pic_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_slice_disable_flag.

intra_boundary_filter_slice_disable_flag equal to 1 specifies IntraBoundaryFilterSliceDisable equal to 1. intra_boundary_filter_slice_disable_flag equal to 0 specifies IntraBoundaryFilterSliceDisable equal to 0. When not present, IntraBoundaryFilterSliceDisable equal to intra_boundary_filter_pic_disable_flag.

8.4.5.2.5 Specification of intra prediction mode INTRA_DC

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

……

Depending on the value of the colour component index cIdx, the following applies:

- If disableIntraBoundaryFilter is equal to 0, cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-1)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-2)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-3)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-4)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-5)

8.4.5.2.6

The variable disableIntraBoundaryFilter is derived as follows:

–	If IntraBoundaryFilterSliceDisable is equal to 1, or implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, disableIntraBoundaryFilter is set equal to 1.

–	Otherwise, disableIntraBoundaryFilter is set equal to 0.

# Conclusion

This contribution proposes to optionally disable the DC boundary filter with gradient filters for lossless coding. Experimental results reportedly show 0-2.0% BD-rate savings.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen content coding test model 1,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1014, April 2014

- HM-14.0+RExt-7.0+SCM-1.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-1.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, April 2014