| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, ES, 30 June – 9 July 2014 | Document: JCTVC-R0115 |
| --- | --- |

| Title: | SCCE5 Test 3.1.3: Adaptive alpha-parameter coding in cross-component prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianguo Zhang1, Kai Zhang1, Jicheng An1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {xianguo.zhang, kai.zhang, jicheng.an, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The current alpha coding method for cross-component prediction simply utilizes the least bins to signal the smaller alpha values. However, it is observed that alpha is often chosen to be 4 or 8 for RGB sequences. Therefore, it is proposed to code the alpha parameter adaptively, either using the least bins or the most bins to signal the larger non-zero alpha values. In the proposed method, since the inter-component residual prediction enabling flag is carried in PPS, a flag scale_parameter_coding_mode in PPS extension is signalled to indicate whether a new signalling scheme is applied. In the encoder implementation, RGB videos always utilize the most bins to signal the smaller alpha values, YUV videos are opposite. The lossy coding BD-rates for RGB videos are as follows.

0.0%,-0.2%,and -0.1% for RGB, text & graphics with motion, 1080p
0.0%,-0.1%, and -0.2% for RGB, text & graphics with motion,720p
-0.2%,-0.2%, and -0.3% for RGB, mixed content, 1440p
-0.2%,-0.1%, and -0.7% for RGB, mixed content, 1080p
-0.2%,-0.3%, and -0.3% for RGB, Animation, 720p
-0.1%,-0.3%, and -0.4% for RGB, camera captured, 1080p

# Introduction

In the current inter-component residual prediction of SCM1 [1], an alpha parameter is coded with a sign flag and an absolute value. This alpha parameter is signalled by syntax elements of res_scale_sign_flag and log2_res_scale_abs_plus1. The latter represents the absolute values of alpha and is binarized as a truncated unary code as shown in Table 1. log2_res_scale_abs_plus1 utilizes 4 context models in total. Such a design assumes alpha values of 0 and 1 are more chosen. However, it is observed that alpha being equal to 4 or 8 often occurs in RGB sequences.

Table 1. Binarization for res_scale_abs_val and the corresponding alphas.

| Abosulte ResScaleVal | log2_res_scale_abs_plus1 [ c ] |
| --- | --- |
| 0 | 0(0) |
| 1 | 1(10) |
| 2 | 2(110) |
| 4 | 3(1110) |
| 8 | 4(1111) |

# Proposed method

In the new signalling, the alpha parameter is represented by log2_res_scale_pred_4minus and log2_res_scale_neg as depicted in Table 2. The binarization for the two syntax elements and the corresponding alphas are demonstrated in Table 3. Five context models are used by the five bins separately for log2_res_scale_pred_4minus. And log2_res_scale_neg is coded in the by-pass mode simply. In summary, alpha parameter equals to 1<<(4–log2_res_scale_4minus) when scale_parameter_coding_mode equal to 1 and log2_res_scale_4minus is equal to neither 0 nor 5. If log2_res_scale_4minus equals to 5, alpha parameter equals to – (1<< log2_res_scale_neg). If log2_res_scale_4minus equals to 0, alpha parameter equals to 0. In this proposal, the flag scale_parameter_coding_mode is set to be 1 for RGB sequences, and set to be 0 for YUV sequences.

Table 2. Proposed alpha parameter syntax

| cross_comp_pred ( x0, y0, c ) { | Descriptor |
| --- | --- |
| if(scale_parameter_coding_mode = = 0) { |  |
| log2_res_scale_abs_plus1[ c ] | ae(v) |
| if( log2_res_scale_abs_plus1[ c ] != 0 ) |  |
| res_scale_sign_flag[ c ] | ae(v) |
| } |  |
| else { |  |
| log2_res_scale_pred_4minus[ c ] | ae(v) |
| if(log2_res_scale_4minus [ c ] == 5) |  |
| log2_res_scale_neg [ c ] | ae(v) |
| } |  |
| } |  |

Table 3. New binarization and corresponding alphas

| Alpha | log2_res_scale_pred_4minus[ c ] | log2_res_scale_neg [ c ] |
| --- | --- | --- |
| 0 | 0(0) | Na |
| 8 | 1(10) | Na |
| 4 | 2(110) | Na |
| 2 | 3(1110) | Na |
| 1 | 4(11110) | Na |
| -1 | 5(11111) | 0(00) |
| -2 | 5(11111) | 1(01) |
| -4 | 5(11111) | 2(10) |
| -8 | 5(11111) | 3(11) |

.

# Experimental results

The proposed methods are integrated on HM-14.0+RExt-7.0+SCM-1.0 [2].

Table 1 and Table 2 present the results under CTC, Table 3 and 4 give the results under CTC with 2-CTU search range.

Table 1.  Results for lossless coding under CTC with full-frame IBC search range

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.1% | -0.1% | 0.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.1% | -0.1% | 0.4% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 2.  Results for lossy coding under CTC with full-frame IBC search range

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | -0.2% | 0.0% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | 0.0% | -0.1% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.1% | -0.2% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | -0.3% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.3% | -0.2% | -0.2% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.7% | -0.2% | -0.6% |
| RGB, Animation, 720p | -0.3% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.4% | -0.2% | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |

Table 3.  Results for lossless coding under CTC with 2-CTU IBC search range

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% | 0.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.1% | -0.1% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.1% | -0.1% | 0.4% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | -0.1% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 4.  Results for lossy coding under CTC with 2-CTU IBC search range

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.1% | 0.1% | -0.1% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.3% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.4% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.5% |
| RGB, Animation, 720p | -0.3% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.3% | -0.2% | -0.2% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 96% |  |  |

# Text specification changes

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| cross_component_prediction_enabled_flag | u(1) |
| if ( cross_component_prediction_enabled_flag ) |  |
| scale_parameter_coding_mode | u(1) |
| chroma_qp_adjustment_enabled_flag | u(1) |
| } |  |

cross_component_prediction_enabled_flag equal to 1 specifies that log2_res_scale_abs_plus1 and res_scale_sign_flag or log2_res_scale_pred_4minus and log2_res_scale_neg may be present in the transform unit syntax for pictures referring to the PPS. cross_component_prediction_enabled_flag equal to 0 specifies that log2_res_scale_abs_plus1, res_scale_sign_flag, log2_res_scale_pred_4minus and log2_res_scale_neg are not present for pictures referring to the PPS. When not present, the value of luma_chroma_prediction_enabled_flag is inferred to be equal to 0. When ChromaArrayType is not equal to 3, it is a requirement of bitstream conformance that the value of luma_chroma_prediction_enabled_flag shall be equal to 0.

scale_parameter_coding_mode When not present, the value of scale_parameter_coding_mode is inferred to be equal to 0. If scale_parameter_coding_mode equal to 0, log2_res_scale_pred_4minus and log2_res_scale_neg are not present in the transform unit syntax for pictures referring to the SPS; If scale_parameter_coding_mode equal to 1, log2_res_scale_abs_plus1 and res_scale_sign_flag are not present for pictures referring to the PPS.

| cross_comp_pred( x0, y0, c ) { | Descriptor |
| --- | --- |
| if(scale_parameter_coding_mode = = 0) { |  |
| log2_res_scale_abs_plus1[ c ] | ae(v) |
| if( log2_res_scale_abs_plus1[ c ] != 0 ) |  |
| res_scale_sign_flag[ c ] | ae(v) |
| } |  |
| else { |  |
| log2_res_scale_pred_4minus[ c ] | ae(v) |
| if(log2_res_scale_4minus [ c ] == 5) |  |
| log2_res_scale_neg [ c ] | ae(v) |
| } |  |
| } |  |

log2_res_scale_pred_4minus [ c ] minus 4 specifies the negative value of base 2 logarithm of the magnitude of the scaling factor ResScaleVal used in luma-chroma residual prediction unless log2_res_scale_pred_4minus [ c ] is equal to 0 or 5. When not present, log2_res_scale_abs_plus1 is inferred equal to 0.

- If log2_res_scale_pred_4minus [ c ] is equal to 0, the corresponding ResScaleVal has a 0 value;

- Otherwise if log2_res_scale_pred_4minus [ c ] is equal to 5, the corresponding ResScaleVal has a negative value;

- Otherwise, the corresponding ResScaleVal has a positive value.

log2_res_scale_neg [ c ] multiplies -1 specifies the value of base 2 logarithm of the magnitude of the scaling factor ResScaleVal used in luma-chroma residual prediction when log2_res_scale_pred_4minus [ c ] is equal to 5.

The variable ResScaleVal[ cIdx ][ x0 ][ y0 ] is derived as follows:

if scale_parameter_coding_mode is equal to 0

If log2_res_scale_abs_plus1[ cIdx − 1 ] is equal to 0, the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = 0		(7-1)

Otherwise (log2_res_scale_abs_plus1[ cIdx − 1 ] is not equal to 0), the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = ( 1  <<  ( log2_res_scale_abs_plus1[ cIdx − 1 ] − 1 ) ) *	(7-2)
			( 1 − 2 * res_scale_sign_flag[ cIdx − 1 ] )

Otherwise (scale_parameter_coding_mode is not equal to 0),

If log2_res_scale_pred_4minus[ cIdx − 1 ] is equal to 0, the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = 0		(7-3)

Otherwise, if log2_res_scale_4minus[ cIdx − 1 ] is not equal to 5, the following applies:

ResScaleVal[ cIdx ][ x0 ][ y0 ] = ( 1  <<  (4 − log2_res_scale_abs_4minus[ cIdx − 1 ]) ) 	(7-4)
Otherwise,

ResScaleVal[ cIdx ][ x0 ][ y0 ] = − ( 1  <<  (log2_res_scale_neg[ cIdx − 1 ]) ) 	(7-5)

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process (in Section 9.3.2.2)

| … | Syntax element | ctxTable | InitType=0 | InitType=1 | InitType=2 |
| --- | --- | --- | --- | --- | --- |
| luma_chroma_pred( ) | log2_res_scale_abs_plus1[ ] | Error: Reference source not found | 0..7 | 8..15 | 16..23 |
|  | res_scale_sign_flag[ ] | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | Log2_res_scale_pred_4minus[ ] | Error: Reference source not found | 0..8 | 9..17 | 18..26 |

Table 9-xx – Values of initValue for ctxIdx of log2_res_scale_pred_4minus[ ][ ][ ] (in Section 9.3.2.2)

| Initialization variable | ctxIdx of log2_res_scale_abs_plus1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |
|  | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |
|  | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

| Table 9-2 – Syntax elements and associated binarizations (in Section 9.3.3.1) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| … | … |  |  |  | … |  | … |  |  |  |  |
| inter_component_resi_pred( ) | log2_res_scale_pred_4minus |  |  |  | TR |  | cMax =5, cRiceParam = 0 |  |  |  |  |
|  | log2_res_scale_neg |  |  |  | FL |  | cMax =3 |  |  |  |  |
| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins (in Section 9.3.4.2.1) |  |  |  |  |  |  |  |  |  |  |  |
| … |  | … | … | … |  | … |  | … | … |  |  |
| log2_res_scale_abs_plus1[ c ] |  | 4 * c + 0 | 4 * c + 1 | 4 * c + 2 |  | 4 * c + 3 |  | na | na |  |  |
| res_scale_sign_flag[ c ] |  | c | na | na |  | na |  | na | na |  |  |
| log2_res_scale_pred_4minus |  | 5 * c + 0 | 5 * c + 1 | 5 * c + 2 |  | 5 * c + 3 |  | 5 * c + 4 | na |  |  |
| log2_res_scale_neg |  | bypass | bypass | na |  | na |  | na | na |  |  |

# Conclusion

This contribution proposed to code the alpha parameter adaptively, either using the least bins or the most bins to signal the larger non-zero alpha values. The formal is utilized for RGB video coding and the latter is for YUV videos in the encoder implementation.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen content coding test model 1,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1014, April 2014

- HM-14.0+RExt-7.0+SCM-1.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-1.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, April 2014