| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0116 |
| --- | --- |

| Title: | SCCE1 Test 4.1: Symmetric intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang1, Jicheng An1, Xianguo Zhang1, Han Huang1, Shawmin Lei2 1 MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 2 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {kai.zhang, jicheng.an, xianguo.zhang, h.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of symmetric intra block copy (SIBC) in SCCE1 Test 4.1. Horizontal SIBC and vertical SIBC are both tested. Experimental results reportedly show that the combination of horizontal SIBC and vertical SIBC can achieve BD-rate savings in AI configurations as follows:
1.3%, RGB, text & graphics with motion, 1080p
0.8%, RGB, text & graphics with motion,720p
1.0%, RGB, mixed content, 1440p
0.8%, RGB, mixed content, 1080p
1.2%, YUV, text & graphics with motion, 1080p
0.9%, YUV, text & graphics with motion,720p
1.0%, YUV, mixed content, 1440p
0.8%, YUV, mixed content, 1080p.

# Introduction

Intra block copy (IBC) considers reduplicated patterns in a picture [1]. As depicted in Fig.1, a reference block in the current picture is copied to the current block as the prediction if IBC is applied. The samples in the reference block must have been reconstructed already before the current block is coded or decoded.

Fig.1. Intra block copy.

Besides reduplication, symmetry is often observed in natural or screen-content pictures. Fig. 2 demonstrates examples for horizontal symmetry and vertical symmetry in screen-content pictures. Fig.3 demonstrates an example for horizontal symmetry in a natural picture. The current IBC cannot deal with these cases efficiently.

Fig. 2.  Symmetric examples in screen content pictures.

Fig. 3.  An approximate horizontal symmetric example in a natural picture.

# Proposed method

Symmetric intra block copy (SIBC) is proposed. When SIBC is applied, the reference block is flipped horizontally or vertically as depicted in Fig. 4 and Fig. 5 before it is used to predict the current block.

In the proposed design, SIBC is only allowed for IBC with block size 2Nx2N. A flag is signaled to indicate whether the current block chooses SIBC or normal IBC. If horizontal SIBC and vertical SIBC are both utilized, one more flag is signaled to indicate which one is selected. The encoder can make the decision during the BV searching process based on the SAD criterion. When horizontal SIBC is on, BVDy is not signaled and BVy is inferred to be 0. Similarly, when vertical SIBC is on, BVDx is not signaled and BVx is inferred to be 0.

Fig. 4.  Horizontal flipping.

Fig. 5.  Vertical flipping.

# Experimental results

The proposed method is integrated on SCM-1.0 [2]. Three tests are conducted under the common test condition (CTC) [3] for SCC with the full picture IBC search range.

- Test1: only horizontal SIBC is applied.

- Test2: only vertical SIBC is applied.

- Test 3: both horizontal and vertical SIBC are applied.

Table 1 and Table 2 show the results for lossy and lossless coding respectively for test 1. Table 3 and Table 4 show the results for lossy and lossless coding respectively for test 2. Table 5 and Table 6 show the results for lossy and lossless coding respectively under CTC for test 3.

Table 1.  Results for lossy coding under CTC (only horizontal SIBC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | -0.4% | -0.5% | -0.4% |
| RGB, mixed content, 1440p | -0.8% | -0.8% | -0.8% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -0.9% | -1.0% |
| YUV, text & graphics with motion,720p | -0.5% | -0.7% | -0.7% |
| YUV, mixed content, 1440p | -0.8% | -1.0% | -1.0% |
| YUV, mixed content, 1080p | -0.7% | -0.7% | -0.7% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.6% |
| RGB, text & graphics with motion,720p | -0.3% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1080p | -0.3% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.6% |
| YUV, text & graphics with motion,720p | -0.3% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | -0.4% | -0.7% | -0.6% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.3% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.3% |
| RGB, text & graphics with motion,720p | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.5% | -0.3% |
| RGB, mixed content, 1080p | -0.2% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | -0.2% | 0.2% | -0.6% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.6% |
| YUV, mixed content, 1080p | 0.3% | -0.2% | 0.3% |
| YUV, Animation, 720p | 0.1% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2.  Results for lossless coding under CTC (only horizontal SIBC)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.5% | 1.5% | 1.4% | 1.5% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.2% | 0.5% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.5% | 1.5% | 1.4% | 1.8% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.2% | 0.5% |
| YUV, mixed content, 1440p | 0.2% | 0.3% | 0.1% | 0.4% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.9% | 0.7% | 1.1% |
| RGB, text & graphics with motion,720p | 0.2% | 0.3% | 0.0% | 0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.9% | 0.9% | 0.9% | 1.1% |
| YUV, text & graphics with motion,720p | 0.2% | 0.3% | 0.1% | 0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.6% | 0.9% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.8% | 0.8% | 0.8% | 0.8% |
| YUV, text & graphics with motion,720p | 0.2% | 0.2% | 0.0% | 0.5% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

Table 3.  Results for lossy coding under CTC (only vertical SIBC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.2% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | -0.8% | -0.9% | -0.8% |
| YUV, mixed content, 1440p | -0.4% | -0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.1% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.5% |
| YUV, text & graphics with motion,720p | -0.6% | -0.4% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | -0.5% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.2% | 0.0% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | -0.3% | -0.2% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | -1.0% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |

Table 4.  Results for lossless coding under CTC (only vertical SIBC)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.1% | 1.0% | 1.0% | 1.1% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.3% | 0.9% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.1% | 1.0% | 1.1% |
| YUV, text & graphics with motion,720p | 0.4% | 0.5% | 0.3% | 1.1% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.6% | 0.7% | 0.5% | 0.9% |
| RGB, text & graphics with motion,720p | 0.1% | 0.3% | 0.1% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.7% | 0.6% | 0.9% |
| YUV, text & graphics with motion,720p | 0.2% | 0.4% | 0.1% | 0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.5% | 0.8% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.6% | 0.5% | 0.8% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.5% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 5.  Results for lossy coding under CTC (both horizontal and vertical SIBC)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -0.8% | -0.8% | -0.8% |
| RGB, mixed content, 1440p | -1.0% | -0.9% | -0.9% |
| RGB, mixed content, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.3% | -1.3% |
| YUV, text & graphics with motion,720p | -0.9% | -1.0% | -1.0% |
| YUV, mixed content, 1440p | -1.0% | -1.1% | -1.2% |
| YUV, mixed content, 1080p | -0.8% | -1.0% | -0.9% |
| YUV, Animation, 720p | -0.1% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.6% | -0.4% | -0.6% |
| RGB, mixed content, 1440p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.8% |
| YUV, text & graphics with motion,720p | -0.6% | -0.5% | -0.6% |
| YUV, mixed content, 1440p | -0.4% | -0.8% | -0.6% |
| YUV, mixed content, 1080p | -0.5% | -0.7% | -0.8% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | -0.3% | -0.5% | -0.3% |
| RGB, mixed content, 1080p | -0.5% | -0.1% | -0.6% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | -0.3% | -0.2% | -0.4% |
| YUV, mixed content, 1440p | -0.3% | -0.7% | -0.5% |
| YUV, mixed content, 1080p | 0.1% | -0.6% | -0.3% |
| YUV, Animation, 720p | 0.1% | -0.4% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

Table 6.  Results for lossless coding under CTC (both horizontal and vertical SIBC)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.8% | 1.8% | 1.6% | 1.9% |
| RGB, text & graphics with motion,720p | 0.5% | 0.6% | 0.4% | 0.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.9% | 1.9% | 1.7% | 1.9% |
| YUV, text & graphics with motion,720p | 0.5% | 0.7% | 0.4% | 1.0% |
| YUV, mixed content, 1440p | 0.3% | 0.3% | 0.1% | 0.4% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.0% | 1.1% | 0.7% | 1.5% |
| RGB, text & graphics with motion,720p | 0.2% | 0.4% | 0.1% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.0% | 1.1% | 1.0% | 1.4% |
| YUV, text & graphics with motion,720p | 0.3% | 0.5% | 0.1% | 0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.0% | 1.0% | 0.7% | 1.2% |
| RGB, text & graphics with motion,720p | 0.2% | 0.4% | 0.1% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.9% | 1.0% | 0.8% | 1.1% |
| YUV, text & graphics with motion,720p | 0.2% | 0.4% | 0.1% | 0.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

# Conclusions

This contribution proposes a symmetric intra block copy mode. Horizontal or vertical flipping is supported for 2Nx2N CU. Experimental results reportedly show that the combination of horizontal SIBC and vertical SIBC can achieve 1.3%, 0.8%, 1.0%, 0.8%, 1.2%, 0.9%, 1.0%, and 0.8% BD-rate savings respectively for RGB, text & graphics with motion, 1080p / RGB, text & graphics with motion,720p / RGB, mixed content, 1440p/ RGB, mixed content, 1080p/ YUV, text & graphics with motion, 1080p/ YUV, text & graphics with motion,720p/ YUV, mixed content, 1440p/ YUV, mixed content, 1080p classes in AI configurations under the common test condition for screen content coding.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen content coding test model 1,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1014, April 2014

- HM-14.0+RExt-7.0+SCM-1.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-1.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, April 2014