| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0119 |
| --- | --- |

| Title: | SCCE3 Test A.2: Palette coding and palette predictor updates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun1, PoLin Lai2, Tzu-Der Chuang1, Yi-Wen Chen1, Shan Liu2, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yc.sun, polin.lai, peter.chuang, yiwen.chen, shan.liu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a new palette coding method including palette copy (i.e., using one CU-level flag to indicate reuse of the last palette coded CU’s palette) and full palette propagation (i.e., propagating unused colors in the palette predictor across CUs). Experimental results show that compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047, the BD-rates are -4.5%, -2.7%, and -1.9% for “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and are -4.9%, -2.7%, and -1.5% for “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

# Introduction

In the palette in JCTVC-Q0094 [1], a palette coded CU contains a palette with N major color vectors. The maximum color vector index of a palette (i.e., maximum value of N) is set to 31 (i.e., Nmax =31). The palette information is predicted and coded using the palette of the last palette coded CU as the predictor.

The contribution proposes a new palette coding method. Two modifications are proposed. The first one is to directly copy entire palette information from the predictor. The second one is to propagate unused colors in the palette predictor across CUs. Detail is introduced in the Section 2.

# Proposed methods

During decoding, the decoder maintains a palette predictor of (Nmax-1) major color vectors from previous CUs for predicting the palette of the current CU. The number of color vectors (i.e., the palette size, denoted as N) of the last palette coded CU are also stored as Nlast.

Two palette prediction modes are proposed: palette copy mode and palette modification mode.

#### Palette copy mode

Basically, the palette copy mode is to copy the palette predictor. Fig. 1 shows an example of palette copy. In this example, the last palette has Nlast (=10) major color vectors, and the palette predictor always has Nmax (=30) color vector predictors regardless of the value of Nlast. The palette size of the current CU, N, is set to Nlast, the first Nlast color vector predictors are copied as the first Nlast color vectors, and the vector index equal to Nlast represents an escape color vector.

Fig. 1.  An example of palette copy mode

#### Palette modification mode

In the palette modification mode, information of palette modifications is signaled to indicate how to derive a palette from the palette predictor. Fig. 2 shows an example of palette modification. In this example, the palette predictor is first created as the palette copy mode. Next, the new palette size, N (=3), is signaled to indicate the number of the major color vectors for the current CU. Then, N flags are signaled to indicate if the major color vector is the same as the major color vector predictor, and the corresponding new vectors (e.g., P’[0] and P’[2]) are further signaled. Finally, the N major color vectors for the current CU are copied from either P[] or P’[] according to the flags, and the major color vector index equal to N represents an escape vector.

Fig. 2.  An example of palette modification mode

#### Palette predictor update

After palette coding for the current CU, the palette predictor is updated. Fig. 3 shows an example of palette predictor update. In this example, the updated last palette size, Nlast, is set to the current palette size, N (=3), and the first N color vector predictors are copied from the N color vectors of the current palette. Note that regardless of the updated value of Nlast, the palette predictor always has Nmax (=31) color vector predictors. We call this property as full palette propagation, which can enhance the palette prediction efficiency across different CU sizes.

Fig. 3.  An example of palette predictor update

# Experimental results

The proposed method is implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 [1]. The tests were conducted under the common test conditions in SCCE3 [1].

## SCCE3 test condition 1

For test condition 1, experiments are conducted with intraBC using full frame search. Table 4 shows the lossy results of the proposed method using Anchor 1-1 in [1]. Table 5 shows lossless results of the proposed method using Anchor 1-1.

Experimental results show that, compared with Anchor 1-1, 4.5%, 2.7%, and 1.9% BD-rate savings are observed on “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and 4.9%, 2.7%, and 1.5% BD-rate savings are observed on “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

The results using Anchor 1-2 in [1] are shown in the attached Excel files, JCTVC-R0119_TestA.2_2CTU_ref=scm.xlsm and JCTVC-R0119_TestA.2_2CTU_ref=scm_qp0.xlsm.

## SCCE3 test condition 2

For test condition 2, experiments are conducted with intraBC using 2-CTU search. Table 6 shows the lossy results of the proposed method using Anchor 2-1 in [1]. Table 7 shows the lossless results of the proposed method using Anchor 2-1

Experimental results show that, compared with Anchor 2-1, 3.1%, 1.8%, and 1.4% BD-rate savings are observed on “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and 3.3%, 1.8%, and 1.1% BD-rate savings are observed on “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

The results using Anchor 2-2 in [1] are shown in the attached Excel files, JCTVC-R0119_TestA.2_FF_ref=scm.xlsm and JCTVC-R0119_TestA.2_FF_ref=scm_qp0.xlsm.

# Conclusion

In this contribution, a new palette coding method is proposed and implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. Experimental results show that, compared with Anchor 1-1, BD-rates are -4.5%, -2.7%, and -1.9% for “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and -4.9%, -2.7%, and -1.5% for “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1123, April, 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, April, 2014.

Table 4  Lossy results of the proposed method under SCCE3 test condition 1 with Anchor 1-1

Table 5  Lossless results of the proposed method under SCCE3 test condition 1 with Anchor 1-1

Table 6  Lossy results of the proposed method under SCCE3 test condition 2 with Anchor 2-1

Table 7  Lossless results of the proposed method under SCCE3 test condition 2 with Anchor 2-1