| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0120 |
| --- | --- |

| Title: | SCCE3 Test B.2: Transition copy mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tzu-Der Chuang, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yc.sun, peter.chuang, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a new run coding mode: transition copy. Experimental results show that compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047, the BD-rates are -3.2%, -1.6%, and -1.0% for “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and are -3.1%, -1.6%, and -0.8% for “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

# Introduction

In the palette in JCTVC-Q0094 [1], the color indices in a CU are run-coded. There are two run modes: run and copy run. In run mode, an index would be signaled and following indices in the run are equal to the index. In copy run, following indices in the run are equal to the indices in their upper indices in the above line.

This contribution proposes a new run mode: transition copy (TC) run. In the TC run mode, a decoder records the adjacent color index patterns and uses it to predict to-be-coded indices. The detail is introduced in the section 2.

# Proposed methods

A transition copy (TC) run is considered as a new run mode in color index run coding. In each CU, at the first sample a TC table is reset when the current CU is the first CU of the slice or inherits the TC table of the previous CU when the current CU is a non-first CU of the slice. We call this inheriting property “TC table propagation”. The TC table is maintained and updated as the samples in the CU are coded. As shown in Table 1, for each to-be-coded non-first-column sample, the input of the TC table is called a pilot color index and is set to the color index of the left sample, and the outputs of the TC table are two TC color indices, where the two TC color indices are likely to occur right after an occurrence of the pilot color index according to the causal area of the CU. As for each to-be-coded first-column sample, the input of the TC table is unavailable, and the two outputs of the TC table are set to 0 and 1. The TC table is updated after each non-first-column sample in the CU is coded as follows. If the current color index is not the same as any of the pilot color index and the 1st TC color index derived from the TC table using the pilot color index as the input, the 2nd TC color index is replaced with the 1st TC color index, and the 1st TC color index is replaced with the current color index.

Table 1.  Illustration of a transition copy table

An example of the TC color indices is shown in Fig. 1. The current sample, C, has a pilot color index equal to 1. The shaded squares with numbers labeled are coded samples. From the causal area of coded samples, it can be found that when a color index equal to 1 is coded, the next color index is likely to be 2 or 3. These transition patterns (i.e., 1=>2 and 1=>3) can be recorded in the TC table to enhance color index prediction efficiency.

Fig. 1.  An example of deriving the 1st TC color index and the 2nd TC color index, where each square denotes a sample, C denotes the current sample, and the numbers in the squares denote the coded color indices

Adding new TC run mode, there are three run types: run, copy run, and TC run. Therefore, palette_run_type_flag in JCTVC-Q0094 is proposed to be modified as Table. 2. And, when the TC run is used, tc_cand and palette_run will be signaled to indicate following palette_run samples are equal to TC[Pilot Color Index][tc_cand].

Table 2.  Proposed palette_run_type_flag syntax modification

|  | palette_run_type_flag in JCTVC-Q0094 | palette_run_type_flag in proposed syntax |
| --- | --- | --- |
| Run | 0 | 00 |
| Copy run | 1 | 1 |
| TC run | N/A | 01 |

# Experimental results

The proposed method is implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 [1]. The tests were conducted under the common test conditions in SCCE3 [1].

## SCCE3 test condition 1

For test condition 1, experiments are conducted with intraBC using full frame search. Table 3 shows the lossy results of the proposed method using Anchor 1-1 in [1]. Table 4 shows lossless results of the proposed method using Anchor 1-1.

Experiments show that, compared with Anchor 1-1, -1.4%, -0.7%, and -0.6% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and -1.4%, -0.7%, and -0.5% bit-rate saving is observed on “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

The results using Anchor 1-2 in [1] are shown in the attached excel files, JCTVC-R0120_TestB.2_FF_ref=scm.xlsm and JCTVC-R0120_TestB.2_FF_ref=scm_qp0.xlsm.

## SCCE3 test condition 2

For test condition 2, experiments are conducted with intraBC using 2-CTU search. Table 5 shows the lossy results of the proposed method using Anchor 2-1 in [1]. Table 6 shows the lossless results of the proposed method using Anchor 2-1

Experiments show that, compared with Anchor 2-1, -3.2%, -1.6%, and -1.0% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and -3.1%, -1.6%, and -0.8% bit-rate saving is observed on “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

The results using Anchor 2-2 in [1] are shown in the attached excel files, JCTVC-R0120_TestB.2_2CTU_ref=scm.xlsm and JCTVC-R0120_TestB.2_2CTU_ref=scm_qp0.xlsm.

## Supplementary test: TC table propagation

The contribution proposed that TC table inherits the TC table of the previous CU when the current CU is a non-first CU of the slice, called TC table propagation.

We conducted experiments to show the BD rate difference between with/without TC table propagation. Table 7 shows the results with intraBC using 2CTU search. The anchor is the proposed method without TC table propagation. The test is the proposed method with TC table propagation.

Experiments show that -0.4% and –0.4% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” and “YUV, text and graphics with motion, 1080p” sequences under AI, respectively.

## Supplementary test: TC table propagation combining palette copy mode in A.2

We found synergies between TC table propagation and palette copy mode in A.2. We conducted experiments to combine the proposed TC method and palette copy mode in A.2 and show the difference between with/without TC table propagation. Table 8 shows the results with intraBC using 2CTU search. The anchor is palette copy mode + the proposed method without TC table propagation. The test is palette copy mode + the proposed method with TC table propagation.

Experiments show that -1.0% and –1.2% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” and “YUV, text and graphics with motion, 1080p” sequences under AI, respectively. Combining palette copy mode, the benefit of TC table propagation will be enlarged from -0.4% to -1.2%.

# Conclusion

In this contribution, a new run coding mode: transition copy is proposed and implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. Experimental results show that, compared with Anchor 1-1, -3.2%, -1.6%, and -1.0% bit-rate saving is observed on “RGB, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively, and -3.1%, -1.6%, and -0.8% bit-rate saving is observed on “YUV, text and graphics with motion, 1080p” sequences under AI, RA, and LB, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1123, April, 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, April, 2014.

Table 3  Lossy results of the proposed method under SCCE3 test condition 1 with Anchor 1-1.

Table 4  Lossless results of the proposed method under SCCE3 test condition 1 with Anchor 1-1.

Table 5  Lossy results of the proposed method under SCCE3 test condition 2 with Anchor 2-1.

Table 6  Lossless results of the proposed method under SCCE3 test condition 2 with Anchor 2-1.

Table 7 Lossy results of TC table propagation under SCCE3 test condition 2. The anchor is the proposed method without TC table propagation. The test is the proposed method with TC table propagation.

Table 8 Lossy results of TC table propagation combining palette copy mode in A.2 under SCCE3 test condition 2. The anchor is palette copy mode + the proposed method without TC table propagation. The test is palette copy mode + the proposed method with TC table propagation.