| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0121 |
| --- | --- |

| Title: | SCCE3 Test B.1: Line-based color index map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Chen Sun, Shih-Ta Hsiang, Yi-Wen Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { peter.chuang, yc.sun, shih-ta.hsiang, yiwen.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a line-based color index map coding method is proposed for palette mode. For a palette CU, the color indices of all samples in the CU are coded line by line. The sample lines are predicted by three line modes which include vertical mode, horizontal mode, and normal mode. In normal mode, the color indices of the line are signaled sample by sample. For each sample, the neighboring color indices and coded color indices are used to predict the current color index. The adaptive index map scan order is also proposed. The proposed method is implemented on top of the SCCE3 palette coding base software. Experimental results reported show that compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 with 2-CTU IntraBC coding, 6.7% and 7.0% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration. Compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 with full-frame IntraBC coding, 3.9% and 4.4% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

# Introduction

In JCTVC-Q0083 [1], a triplet palette signaling and a line-based palette color index map coding method were proposed and are studied in SCCE3 [2]. In this contribution, the line-based palette color index map coding method of JCTVC-Q0083 is implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. Different from the run-based color index map coding used in the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047, the line-based color index map is proposed to code the color indices of all samples in the CU line by line. The details of the proposed methods are described in the rest of this contribution.

# Proposed methods

For a palette coded CU, samples are converted into color indices. The color indices of all samples in the CU are coded line by line. For each sample line, an “index line mode” is signaled to indicate the coding mode. The index line mode includes vertical mode, horizontal mode, and normal mode:

- If the vertical mode is selected, the color indices of the current line are copied from the above line.

- Else if the horizontal mode is selected, one color index is signaled and the color indices of all rest samples in the current line are set to the signaled color index.

- Else (the normal mode is selected), the color indices of the line are signaled sample by sample. For each sample, a color prediction index is signaled to specify how to predict the color index, and a color index of the sample is transmitted directly when the color prediction index indicates no prediction.

In this proposal, escape sample values can only occur when the normal line mode is selected. After the color indices of each line with the normal index line mode are coded, escape sample values, if exist, are fixed-length coded, where the codeword length of each escape sample value depends on the sample bit depth and the quantization parameter.

## Coding of index line mode

A CU-level flag is first signaled to indicate whether the color indices of all samples are zero. If the flag is false, the index line mode is coded line by line, and the following applies.

The last index line mode is used as a predictor for the current index line mode. One flag is signaled to indicate whether the current index line mode is the same as the last index line mode. If the current index line mode is not the same as the last index line mode, another flag is signaled to select between the rest two modes. For the first line of the CU, its last index line mode is set to normal mode.

## Coding of color index (only for samples of the normal index line mode)

When the normal mode is selected as the current index line mode, the color index of each sample is signaled one by one using color indices of coded samples as color index predictors. For each sample, a color index candidate list is constructed and a color prediction index is signaled to specify which color predictor is used. The related details are specified as follows.

### Spatial color index prediction

The spatial neighboring color indices of current sample are considered in the color index candidate list. The spatial neighboring samples are the left, the above, and the above-left samples.

### Transition copy prediction

A transition copy (TC) candidate is considered in the color index candidate list. In each CU, at the first sample a TC table is reset when the current CU is the first CU of the slice or inherits the TC table of the previous CU when the current CU is a non-first CU of the slice. The TC table is maintained and updated as the samples in the CU are coded. As shown in Table 1, for each to-be-coded non-first-column sample, the input of the TC table is called a pilot color index and is set to the color index of the left sample, and the outputs of the TC table are two TC color indices, where the two TC color indices are likely to occur right after an occurrence of the pilot color index according to the causal area of the CU. As for each to-be-coded first-column sample, the input of the TC table is unavailable, and the two outputs of the TC table are set to 0 and 1. The TC table is updated after each non-first-column sample in the CU is coded as follows. If the current color index is not the same as the pilot color index and the 1st TC color index derived from the TC table using the pilot color index as the input, the 2nd TC color index is replaced with the 1st TC color index, and the 1st TC color index is replaced with the current color index.

Table 1.  Illustration of a transition copy table

An example of the TC candidate is shown in Fig. 6. The current sample, C, has a pilot color index equal to 1. The shaded squares with numbers labeled are coded samples. From the causal area of coded samples, it can be found that when a color index equal to 1 is coded, the next color index is likely to be 2 or 3. These transition patterns (i.e., 1=>2 and 1=>3) can be recorded in the TC table to enhance color index prediction efficiency.

Fig. 6.  An example of deriving the 1st TC color index and the 2nd TC color index, where each square denotes a sample, C denotes the current sample, and the numbers in the squares denote the coded color indices

### Color index candidate list construction with a fixed candidate list size

For each samples in normal line mode, the spatial neighboring color indices and the color indices derived from TC prediction are used as the color index predictors. The initial color index candidate list can be constructed as follow.

If the color indices of the above and the left samples are the same, the initial candidate list is {above, left, TC1, TC2}.

Otherwise, the initial candidate list is {above, TC1, TC2, above-left}.

The four color index candidates in the initial candidate list and a new color index candidate form the final candidate list. The new color index candidate indicates that the current color index is not predictive coded and the color index value is directly signaled. The final candidate list size is fixed to 5. The codewords of the selected candidate index are shown in Table 2.

Table 2.  The codeword design of the selected candidate index

| Selected Candidate Index | Codeword |
| --- | --- |
| 0: Color index candidate 0 | 1 |
| 1: Color index candidate 1 | 01 |
| 2: Color index candidate 2 | 001 |
| 3: Color index candidate 3 | 0001 |
| 4: New color index candidate | 0000 |

### Redundant index removal and fixed-length coding of color index with skipped bins

When the new color index candidate is selected (i.e., the candidate index is equal to 4 in Table 2), a color index is transmitted. Since all the candidate predictors are not selected, the color index of the above sample is removed. The color index to be signaled depends on the color index of the above sample as follow.

If the above sample is not escape sample and the color index of current sample is larger than or equal to the color index of the above sample, the color index to be signaled is equal to the color index to be coded of current sample minus 1.

Otherwise, the color index to be signaled is equal to the color index of current sample.

Then the color index to be signaled is first fixed-length binarized, where the length is equal to ceil(log2M), where M is the maximal value of the color index to be signaled. Then, the binarization codeword is coded from the most significant bin toward the least significant bin. The first bin is coded with a single context and the rest bins are coded with bypass bins. The bins may lead to a value greater than the maximum codeword value, M, are inferred to be equal to 0 and are not coded.

## Adaptive index map scan

For a palette coded CU, a adaptive color index map scan flag is signaled. If the flag is equal to 0, the index map is coded with the horizontal raster scan. Otherwise, the index map is coded with the vertical raster scan.

# Experimental results

The proposed method is implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. The tests were conducted under the test conditions defined in SCCE3 [2]. Two kinds of configurations are tested, which are the IntraBC with 2-CTU search range and the IntraBC with full-frame search range.

Table 3 and 4 shows the lossy and the lossless results of the proposed method using SCCE3 base software as the anchor with 2-CTU IntraBC.

Table 4 and 5 shows the lossy and the lossless results of the proposed method using SCCE3 base software as the anchor with full-frame IntraBC.

# Conclusion

In this contribution, a line-based color index map coding method is proposed and implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. Experimental results reportedly show that compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 with 2-CTU IntraBC coding, 6.7% and 7.0% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration. Compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 with full-frame IntraBC coding, 3.9% and 4.4% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-C. Sun, T.-D. Chuang, P. Lai, S.-T. Hsiang, Y.-W. Chen, X. Zhang, S. Liu, Y.-W. Huang, S. Lei, “AHG10: A triplet palette mode combining JCTVC-P0108 and JCTVC-P0198,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0083, Mar. 2014.

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1123, Mar. 2014.

Table 3.  Lossy results of the proposed method with 2-CTU IntraBC

Table 4.  Loslessy results of the proposed method with 2-CTU IntraBC

Table 5.  Lossy results of the proposed method with full-frame IntraBC

Table 6.  Lossless results of the proposed method with full-frame IntraBC