| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0122 |
| --- | --- |

| Title: | SCCE3 Test C.1: Combination of SCCE3 Test A.2 and Test B.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang1, Yu-Chen Sun1, PoLin Lai2, Shih-Ta Hsiang1, Xianguo Zhang3, Yi-Wen Chen1, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA 3MediaTek Beijing: Building 1-B, No.6 Park, Jiuxianqiao Road, Chaoyang District, Beijing, China, 100015 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, yc.sun, polin.lai, shih-ta.hsiang, xianguo.zhang, yiwen.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of combining JCTVC-R0119 SCCE3 Test A.2 palette coding and palette predictor updates and JCTVC-R0121 SCCE3 Test B.1 line-based color index map coding. Experimental results reportedly show that compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 with 2-CTU IntraBC, 12.4% and 13.8% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration, respectively. The corresponding BD-rate savings become 8.3% and 9.3% with full-frame IntraBC.

The palette generation method, which is encoder-only and non-normative and proposed in JCTVC-Rxxxx SCCE3 Test A.6, is also integrated. Compared with the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047, 14.9% and 16.4% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration with 2-CTU IntraBC, respectively; the corresponding BD-rate savings are 10.8% and 12.0% with full-frame IntraBC. Compared with SCM-1.0 (palette-off) with 2-CTU IntraBC, 26.7% and 27.1% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration, respectively; the corresponding BD-rate savings become 20.5% and 21.3% with full-frame IntraBC.

# Proposed methods

In JCTVC-Q0083 [1], a triplet palette signaling and a line-based palette color index map coding method were proposed and are studied in SCCE3 [2]. The palette coding and palette predictor updates of JCTVC-Q0083 are studied in JCTVC-R0119 [3] SCCE3 Test A.2, and the line-based palette color index map coding of JCTVC-Q0083 is studied in JCTVC-R0121 [4] SCCE3 Test B.1. In this contribution, the proposed methods in SCCE3 Test A.2 and SCCE3 Test B.1 are integrated and implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. The palette mode flag is also moved after the IntraBC mode flag.

Since the SCCE3 Test A.2 and SCCE3 Test B.1 do not improve the palette generation, the palette generation method proposed in JCTVC-Rxxxx [5] SCCE3 Test A.6 is also integrated for higher coding efficiency.

# Experimental results

The proposed method is implemented on top of the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047. The tests were conducted under the test conditions defined in SCCE3 [2]. Two kinds of configurations are tested, which are the IntraBC with 2-CTU search range and the IntraBC with full-frame search range.

Table 1 and 2 shows the lossy and the lossless results of the proposed method using the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 as the anchor with 2-CTU IntraBC. In lossy case, 12.4% and 13.8% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

Table 3 and 4 shows the lossy and the lossless results of the proposed method using the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 as the anchor with full-frame IntraBC. In lossy case, 8.3% and 9.3% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

The palette generation method proposed in SCCE3 Test A.6 is also integrated. Table 5 and 6 shows the lossy and the lossless results of the proposed method with SCCE3 Test A.6 palette generation using the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 as the anchor with 2-CTU IntraBC. In lossy case, 14.9% and 16.4% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

Table 7 and 8 shows the lossy and the lossless results of the proposed method with SCCE3 Test A.6 palette generation using the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047 as the anchor with full-frame IntraBC. In lossy case, 10.8% and 12.0% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

Table 9 shows the lossy results of the proposed method with SCCE3 Test A.6 palette generation using SCM-1.0 (palette-off) as the anchor with 2-CTU IntraBC. 26.7% and 27.1% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

Table 10 shows the lossy results of the proposed method with SCCE3 Test A.6 palette generation using SCM-1.0 (palette-off) as the anchor with full-frame IntraBC. 20.5% and 21.3% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

# Conclusion

In this contribution, the combined results of JCTVC-R0119 SCCE3 Test A.2 and JCTVC-R0121 SCCE3 Test B.1 are provided. The results with SCCE3 Test A.6 palette generation are also included. The results reported show that comparing to SCM-1.0 (palette-off) with 2-CTU IntraBC, the proposed method with SCCE3 Test A.6 palette generation provides 26.7% and 27.1% BD-rate savings for RGB and YUV text & graphics with motion 1080p sequences under AI configuration. Comparing to SCM-1.0 (palette-off) with full-frame IntraBC, 20.5% and 21.3% BD-rate savings are achieved for RGB and YUV text & graphics with motion 1080p sequences under AI configuration.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-C. Sun, T.-D. Chuang, P. Lai, S.-T. Hsiang, Y.-W. Chen, X. Zhang, S. Liu, Y.-W. Huang, S. Lei, “AHG10: A triplet palette mode combining JCTVC-P0108 and JCTVC-P0198,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0083, Mar. 2014.

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1123, Mar. 2014.

- Y.-C. Sun, P. Lai, T.-D. Chuang, Y.-W. Chen,  S. Liu, Y.-W. Huang, S. Lei, “SCCE3 Test A.2: Palette coding and palette predictor updates,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0119, June 2014.

- T.-D. Chuang, Y.-C. Sun, S.-T. Hsiang, Y.-W. Chen, Y.-W. Huang, S. Lei, “SCCE3 Test B.1: Line-based color index map coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0121, June 2014.

- C.-M. Tsai, X. Xiu, Y. He, Y. Ye, “SCCE3 Test A.6: Palette table generation,” Document of Joint Collaborative Team on Video Coding, JCTVC-Rxxxx, June 2014.

- R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye, “Screen content coding test model 1 (SCM 1),” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1014, Mar. 2014.

Table 1.  Lossy results of the proposed method with 2-CTU IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 2.  Loslessy results of the proposed method with 2-CTU IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 3.  Lossy results of the proposed method with full-frame IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 4.  Lossless results of the proposed method with full-frame IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 5.  Lossy results of the proposed method and SCCE3 Test A.6 palette generation with 2-CTU IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 6.  Loslessy results of the proposed method and SCCE3 Test A.6 palette generation with 2-CTU IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 7.  Lossy results of the proposed method and SCCE3 Test A.6 palette generation with full-frame IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 8.  Lossless results of the proposed method and SCCE3 Test A.6 palette generation with full-frame IntraBC. Anchor is the palette in JCTVC-Q0094 plus the palette bug-fixes in JCTVC-Q0047.

Table 9.  Lossy results of the proposed method and SCCE3 Test A.6 palette generation with 2-CTU IntraBC. Anchor is SCM-1.0 (palette-off).

Table 10.  Lossy results of the proposed method and SCCE3 Test A.6 palette generation with 2-CTU IntraBC. Anchor is SCM-1.0 (palette-off).