Text specification for C.1 (SCCE3)

This text specification is on top of JCTVC-P1005_v4.

## Known issues in the text:

- max_palette_size is never signalled.

- Palette mode for 4:2:2 and 4:2:0 formats may not work correctly.

- Problems in the reconstruction of escape values

## Syntax

7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| palette_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| palette_coding(x0, y0, CbWidth, CbHeight) { | Descriptor |
| --- | --- |
| if( ChromaArrayType = = 3 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 3 ) |  |
| } else if( ChromaArrayType = = 2 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 1 ) |  |
| palette_coding_component( x0, y0, CbWidth >> 1, CbHeight, 2 ) |  |
| } else if( ChromaArrayType = = 1 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 1 ) |  |
| palette_coding_component( x0, y0, CbWidth >> 1, CbHeight >> 1, 2 ) |  |
| } else if( ChromaArrayType = = 0 ) { |  |
| palette_coding_component( x0, y0, CbWidth, CbHeight, 1 ) |  |
| } |  |

| palette_coding_component( x0, y0, CbWidth, CbHeight, NumComp ) { | Descriptor |
| --- | --- |
| compOffset = ( NumComp = = 3 ) ? 0 : ( NumComp – 1 ) |  |
| nCbS = ( 1 << log2CbSize ) |  |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| if(palette_share_flag[ x0 ][ y0 ]){ |  |
| palette_size = previousPaletteSize |  |
| for( n = 0; n < palette_size; n++ ) { |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| palette_entries[ cIdx ][ n ] = previousPaletteEntries[ cIdx ][n] |  |
| } |  |
| } |  |
| } else { |  |
| palette_size | ae(v) |
| for( n = 0; n < palette_size; n++ ) { |  |
| if(n < previousValidPaletteSize){ |  |
| previous_palette_entry_flag[ i ] | ae(v) |
| }else |  |
| { |  |
| previous_palette_entry_flag[ i ]=0 |  |
| } |  |
| if(previous_palette_entry_flag[ i ]){ |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| palette_entries[ cIdx ][ n ] = previousPaletteEntries[ cIdx ][n] |  |
| } |  |
| }else |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| palette_entries[ cIdx ][ n ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| verticle_scan_flag[ x0 ][ y0 ] | ae(v) |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteSize; i++ ) { |  |
| previous_palette_entry_flag[ i ] | ae(v) |
| if ( previous_palette_entry_flag[ i ] ) { |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ i ] |  |
| numPredPreviousPalette++ |  |
| } |  |
| } |  |
| if( numPredPreviousPalette < max_palette_size) |  |
| palette_num_signalled_entries | ae(v) |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| for( i = 0; i < palette_num_signalled_entries; i++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette + i ] | ae(v) |
| palette_size = numPredPreviousPalette + palette_num_signalled_entries |  |
| previous_run_type_flag = INDEX_MODE |  |
| scanPos = 0 |  |
| all_zero_idx_mode_flag | ae(v) |
| if ( all_zero_idx_mode_flag == 0){ |  |
| for( j = 0; j < nCbS; j ++) |  |
| for(i = 0; i < nCbS; i++) { |  |
| paletteMap[ i ][ j ] = 0 |  |
| } |  |
| } else { |  |
| cIdx = 0; |  |
| for( j = 0; j < nCbS; j ++) { |  |
| line_mode[i] | ae(v) |
| if(line_mode[ j ] == HOR_MODE) { |  |
| palette_index | ae(v) |
| for(i = 0; i < nCbS; i++) { |  |
| paletteMap[ i ][ j ] = palette_index |  |
| } |  |
| } else if(line_mode [j] == VER_MODE) { |  |
| for(i = 0; i < nCbS; i++) { |  |
| paletteMap[ i ][ j ] = paletteMap[ i ][ j − 1 ] |  |
| update_ tc_table ( i, j) |  |
| } |  |
| } else { //NOR_MODE |  |
| for(i = 0; i < nCbS; j++) { |  |
| index_pred_symbol[ i ][ j ] | ae(v) |
| if(index_pred_symbol[ i ][ j ] == NEW_INDEX ) { |  |
| palette_index | ae(v) |
| if (paletteMap[ i ][ j-1 ] == palette_size) |  |
| paletteMap[ i ][ j ] = palette_index |  |
| else { |  |
| paletteMap[ i ][ j ] = (palette_index >= paletteMap[ i ][ j-1 ]) ? palette_index : palette_index + 1 |  |
| } |  |
| } else { |  |
| paletteMap[ i ][ j ] = paletteIndexCandList[index_pred_symbol[ i ][ j ]] |  |
| } |  |
| update_ tc_table ( i, j) |  |
| } |  |
| } |  |
| for(i = 0; i < nCbS; i++) { |  |
| if(paletteMap[ i ][ j ] == palette_size) { |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) { |  |
| palette_escape_val [ cIdx ] [i][ j ] | ae(v) |
| if( verticle_scan_flag[ x0 ][ y0 ] ){ |  |
| samples_array[ cIdx ][ j ][ i ]= rec_palette_escape_val[cIdx] |  |
| }else{ |  |
| samples_array[ cIdx ][ i ][ j ]= rec_palette_escape_val[cIdx] |  |
| } |  |
| } |  |
| } else { |  |
| for( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| if( verticle_scan_flag[ x0 ][ y0 ] ){ |  |
| samples_array[ cIdx ][ j ][ i ] = palette_entries[ cIdx ][ paletteMap[ i ][ j ]] |  |
| }else{ |  |
| samples_array[ cIdx ][ i ][ j ] = palette_entries[ cIdx ][ paletteMap[ i ][ j ]] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| previousPaletteSize = palette_size |  |
| for( i = 0; i < palette_size; i++ ) { |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| previousPaletteEntries[ cIdx ][ i ] = palette_entries[ cIdx ][ i ] |  |
| } |  |
| } |  |
| previousValidPaletteSize = (previousValidPaletteSize> palette_size) ?previousValidPaletteSize: palette_size |  |

| update_ tc_table ( xC, yC) { | Descriptor |
| --- | --- |
| if(xC!=0 && paletteMap[ xC -1][ yC ] != paletteMap[ xC][ yC ] && paletteMap[ xC][ yC ] != tc_table[0][ paletteMap[ xC -1][ yC ]] && paletteMap[ xC -1][ yC ] != palette_size) |  |
| { |  |
| tc_table[0][ paletteMap[ xC -1][ yC ]] = paletteMap[ xC][ yC] |  |
| tc_table[1][ paletteMap[ xC -1][ yC ]] = tc_table[0][ paletteMap[ xC -1][ yC ]] |  |
| } |  |
| } |  |

7.4.3.2 Sequence parameter set RBSP semantics

sps_extension_flag[ 0 ] equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, palette_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, fast_rice_adaptation_enabled_flag, and cabac_bypass_alignment_enabled_flag are present in the SPS RBSP syntax structure. sps_extension_flag[ 0 ] equal to 0 specifies that these syntax elements are not present.

palette_enabled_flag equal to 1 specifies that the palette mode may be used for intra blocks. palette_enabled_flag equal to 0 specifies that the palette mode is not applied. When not present, the value of palette_enabled_flag is inferred to be equal to 0.

## Semantics

palette_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using the palette mode. palette_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded using the palette mode.

When palette_mode_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_share_flag[ x0 ][ y0 ] equal to 1 specifies that the entire palette equal to the previous palette.

vertical_scan_flag[ x0 ][ y0 ] equal to 1 specifies that, in index coding, the horizontal and vertical axis of sample position is switched.

previous_palette_entry_flag[ i ] equal to 1 specifies that the palette entry from the previous used palette is copied. previous_palette_entry_flag [ i ] equals to 0 specifies that the palette entry from the previous used palette is not copied.

The previousPaletteSize is the size of the palette of the previous CU decoded in palette mode. The variable previousPaletteSize is set equal to 0 for the first decoded palette CU of each CTB line as well as slice.

The previousValidPaletteSize is the size of valid color in the palette predictor. The variable is set equal to 0 for the first decoded palette CU of each CTB line as well as slice.

palette_num_signalled_entries specifies the number of entries in the palette that are explicitly signalled for the current coding unit.

When palette_num_signalled_entries is not present, it is inferred to be equal to 0.

palette_entries[ cIdx ][ i ] specifies the i-th element in the palette for the color component cIdx.

The array previousPaletteEntries is equal to the array palette_entries of the last CU decoded in palette mode.

index_pred_symbol [ i ][ j ] specifies the prediction symbol of the index for the current sample in position ( i, j ). index_pred_symbol [ i ][ j ] equal to NEW_INDEX specifies that index value of current sample cannot be derived from the coded sample.

palette_escape_val specifies the escape pixel value.

palette_run_type_flag equal to 1 specifies that the decoding process is COPY_ABOVE_MODE where the decoded pixel value is equal to the pixel at the same location in the above row. palette_run_type_flag equal to 0 specifies that the decoding process mode is INDEX_MODE where the pixel’s palette index is coded in the bitstream. .

When palette_run_type_flag is not present, it is inferred to be equal to INDEX_MODE.

palette_index is an index to the palette entries. If palette_index is equal to palette_size, the pixel is coded as ESCAPE_PIXEL.

8.4.1 General decoding process for coding units coded in intra prediction mode

Depending on the values of pcm_flag[ xCb ][ yCb ], palette_mode_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise if (pcm_flag[ xCb ][ yCb ] is equal to 0 and palette_mode_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows:

For each i, j = 0..nCbS – 1,

SL[ xCb + i ][ yCb + j ] = sample_array[ 0 ][ i ][ j ]

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

SCb[ xCb + i ][ yCb + j ] = sample_array[ 1 ][ i ][ j ]

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

SCr[ xCb + i ][ yCb + j ] = sample_array[ 2 ][ i ][ j ]

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, palette_mode_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xC ][ yC ] is equal to 0 palette_mode_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

…

8.4.4.3 Scaling process for escape pixels

Inputs to this process are:

–	a value escValue specifying the sample value of the escape pixel within the current block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled escape pixel sample value escReco.

The variable escReco is derived as follows:

bdShift = escapeShift[qP]

If bdShift is larger then 1, addi is set to ( 1 << ( bdShift – 1 ) ). Otherwise, addi is set to 0.

–	The scaled escape pixel sample value escReco is derived as follows:

escReco = ( escValue << bdShift	) + addi

Table x-x – Specification of bit shift for escape pixel

| qP | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| escapeShift | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| qP | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| escapeShift | 1 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 5 |
| qP | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |  |  |  |  |  |
| escapeShift | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |  |  |  |  |  |

8.4.4.4 Decoding process for TC_index

Inputs to this process are:

–	a value paletteMap[][] specifying the index values in paletted coded CU.

–	tc_cand[ xC ][ yC ] specifying the candidate index in tc_table[].

Output of this process is:

–	two values tc_index_0 and tc_index_1 specifying the index predictors

The variable tc_index_0 and tc_index_1 are derived as follows:

–	If xC is equal to 0 or paletteMap[ xC -1][ yC ] == palette_size,

tc_index_0 = 0, tc_index_1 = 0,

–	Otherwise,

tc_index_0 = tc_table[0][paletteMap[ xC -1][ yC ]]

tc_index_1 = tc_table[1][paletteMap[ xC -1][ yC ]]

8.4.4.4.1 Updating process for tc_table

Inputs to this process are:

–	a value paletteMap[][] specifying the index values in paletted coded CU.

–	tc_table_prev[][].

Output of this process is:

–	tc_table[][].

The variable tc_table[] is derived as follows:

–	If xC is equal to 0, paletteMap[ xC -1][ yC ] == paletteMap[ xC][ yC ], paletteMap[ xC][ yC ]== tc_table[0][ paletteMap[ xC -1][ yC ]] or paletteMap[ xC -1][ yC ] == palette_size,

tc_table[0][i] = tc_table[0][i],

tc_table[1][i] = tc_table[1][i] for all i,

–	Otherwise,

tc_table[0][i] = paletteMap[ xC][ yC ],

tc_table[1][i] = tc_table[0][i] when i == paletteMap[ xC -1][ yC ],

tc_table[0][i] = tc_table[0][i],

tc_table[1][i] = tc_table[1][i] when i != paletteMap[ xC -1][ yC ].

8.4.4.5 Decoding process for generating palette index candidate list

Inputs to this process are:

–	a value paletteMap[][] specifying the index values in paletted coded CU.

–	two index values tc_index_0 and tc_index_1 specifying the index predictors derived in 8.4.4.4.

–	a luma location ( xC, yC ) specifying the position of the current sample relative to the top-left sample of the current block.

Output of this process is:

–	a list paletteIndexCandList specifying the palette index candiate predictors of the current sample

The indices in paletteIndexCandList are derived as follows:

–	If paletteMap[xC][yC-1] is equal to paletteMap[xC-1][yC],

paletteIndexCandList[0] = paletteMap[xC][yC-1]

paletteIndexCandList[1] = tc_index_0

paletteIndexCandList[2] = tc_index_1

paletteIndexCandList[3] = paletteMap[xC-1][yC-1]

–	Otherwise,

paletteIndexCandList[0] = paletteMap[xC][yC-1]

paletteIndexCandList[1] = paletteMap[xC-1][yC]

paletteIndexCandList[2] = tc_index_0

paletteIndexCandList[3] = tc_index_1

Table 9-4 Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | palette_mode_flag[ ][ ] | Table 9-XX | 0..2 | 3..5 | 6..8 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |

| Table 9-1 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | palette_mode_flag[ ][ ] | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| palette_coding( ) | palette_share_flag[] | FL | cMax = 1 |
|  | verticle_scan_flag [] | FL | cMax = 1 |
|  | previous_palette_entry_flag[] | FL | cMax = 1 |
|  | palette_num_signalled_entries | TR | cMax = 31, cRiceParam = 0 |
|  | palette_entries | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | palette_escape_val | FL | BitDepth - escapeShift[qP] |
|  | all_zero_idx_mode_flag | FL | cMax = 1 |
|  | line_mode | TR | cMax = 2, cRiceParam = 0 |
|  | palette_index | 9.3.3.x | cMaxPaletteIndex |
|  | index_pred_symbol | TR | cMax = 5, cRiceParam = 0 |

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| palette_share_flag | 0 | na | na | na | Na | na |
| palette_mode_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| verticle_scan_flag | 0,1 (subclause Error: Reference source not found) | na | na | na | Na | na |
| all_zero_idx_mode_flag | 0 | na | na | na | na | na |
| previous_palette_entry_flag | bypass | na | na | na | na | na |
| palette_num_signalled_entries | bypass | bypass | bypass | bypass | Bypass | bypass |
| palette_entries | bypass | bypass | bypass | bypass | Bypass | bypass |
| palette_escape_val | cIdx | bypass | bypass | bypass | Bypass | bypass |
| line_mode | 0..5 (subclause Error: Reference source not found8 ) |  | na | na | na | na |
| palette_index | 0 | bypass | bypass | bypass | Bypass | bypass |
| index_pred_symbol | 0..7 (subclause Error: Reference source not found9 ) |  |  |  | na | na |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | Na | na |

Table 9-2 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_bc_flag[ x0 ][ y0 ] | intra_bc_flag[ xNbL ][ yNbL ] | intra_bc_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| palette_mode_flag[ x0 ][ y0 ] | palette_mode_flag [ xNbL ][ yNbL ] | palette_mode_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| verticle_scan_flag[ x0 ][ y0 ] | verticle_scan_flag[ xNbL ][ yNbL ] | Na | ( condL && availableL ) |

9.3.3.x Palette index binarization process

Input to this process is a request for a palette index binarization for a syntax element with value paletteIdx and cMaxPaletteIndex.

Output of this process is the palette index binarization of the syntax element.

The bin string of the palette index binarization process of a syntax element paletteIdx is specified as follows, where each call of the function put( X ), with X being equal to 0 or 1, adds the binary value X at the end of the bin string:

numBits = Floor(Log2(cMaxPaletteIndex))

mask = 1 << numBits

curVal = 0

for ( idx = 0; idx <= numBits; idx++ ) {

if ( curVal + mask <= cMaxPaletteIndex ) {

if (paletteIdx < curVal + mask)

put(0)

else {

put(1)

curVal = curVal + mask

}

}

mask >>= 1

}

9.3.4.2.8 Derivation process of ctxInc for the syntax element line_mode

Input to this process is line_mode_prev which specifying the line mode of previous line.

Output of this process is the variable ctxInc.

The variables ctxInc is derived as follows:

ctxInc = binIdx * 3 + line_mode_prev

9.3.4.2.9 Derivation process of ctxInc for the syntax element index_pred_symbol

Input to this process is index_pred_symbol_left which specifying the index_pred_symbol of its left sample

Output of this process is the variable ctxInc.

The variables ctxInc is derived as follows:

ctxInc = binIdx * 2 + (index_pred_symbol_left == binIdx ? 0 : 1)