| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0123 |
| --- | --- |

| Title: | Non-SCCE1: On block vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method for coding delta block vectors that is intended to provide optimal performance over multiple search areas.  In this method, the block vector coding is modified to use coeff_abs_level_remaining syntax (without the gr1 flag), with Rice parameter adaptation applied separately for the horizontal and vertical components of the block vector.  For full-frame simulations, luma TGM 1080P results show YUV gains of -3.2%, -1.5% and -1.0% in AI, RA and LB respectively and RGB gains of -2.7%, -1.3% and -0.9% in AI, RA and LB respectively.  For 2CTU simulations, luma TGM 1080P results show YUV gains of -2.2%, -1.0% and -0.6% in AI, RA and LB respectively and RGB gains of -1.9%, -1.0% and -0.6% in AI, RA and LB respectively.

# Problem Statement

In the SCM1.0, delta block vectors for intra-block copy use the pre-existing ‘mvd_coding’ syntax.  This syntax is optimised for coding values with a strong disposition towards low magnitudes.  However, in intra-block copy, the distribution of vectors can be expected to have greater uniformity across the range of values searched by the encoder.

In the SCM1.0, the supported encoder search range corresponds to all reconstructed samples in the frame.  In order to reduce complexity, some encoders may search a smaller space than the supported region.

Then, a method for coding delta block vectors that performs well in both the full-frame search and the local search is desirable.

# Description of the proposed change

To facilitate modification of the bit cost estimation code, functions related to intra-block copy search were refactored to remove duplicated code and duplicated function calls (e.g. to getCost()).

Then, the following bug-fixes that were distributed to participants in SCCE1 were incorporated into the software and were also used in the anchor for the presented results:

#define BUGFIX_FULL_FRAME_SEARCH_PREDICTORS               1

#define BUGFIX_Q0175                                      1

#define BUGFIX_COMP_SIGNED_UNSIGNED                       1 // bug fix comp sign / const unsigned

The SCM-1.0 anchor was also regenerated using the above bug-fixes.

In the method, the coeff_abs_level_remaining coding is applied to the delta block vectors as follows:

- A separate Rice parameter initialization statistic is kept for each component of the delta block vector and updated on each usage of the corresponding component.  Thus, two Rice-parameter contexts are added.

- A separate context is used to code the significance flag of each component. No ‘gr1’ flag is present.  As the gr0 flag previously shared a context for each components, removal of the ‘gr1’ flag results in no net change in the number of CABAC contexts.

- The sign of each component is bypass coded (as before).

# Experiment Results

Lossy results are presented in Table 1 and Table 2 below, with full-frame search area and 2 CTU search area respectively.  Lossless results are presented in Table 3 and Table 4 below, with full-frame search area and 2 CTU search area respectively.  Simulations were performed under the Screen content coding common conditions [1]. Refer to the accompanying spreadsheet for results with greater precision. Execution took place on a Linux cluster using GCC 3.4.6.

Table 1. Lossy simulation result with full-frame search and bug-fixes applied.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.7% | -2.6% | -2.6% |
| RGB, text & graphics with motion,720p | -1.7% | -1.6% | -1.6% |
| RGB, mixed content, 1440p | -0.8% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -1.1% | -0.9% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.2% | -2.9% | -2.9% |
| YUV, text & graphics with motion,720p | -2.1% | -1.9% | -1.9% |
| YUV, mixed content, 1440p | -1.1% | -1.0% | -1.0% |
| YUV, mixed content, 1080p | -1.3% | -1.1% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.2% | -1.2% |
| RGB, text & graphics with motion,720p | -1.1% | -1.1% | -1.1% |
| RGB, mixed content, 1440p | -0.7% | -0.5% | -0.5% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.4% | -1.5% |
| YUV, text & graphics with motion,720p | -1.4% | -1.3% | -1.2% |
| YUV, mixed content, 1440p | -0.6% | -0.6% | -0.7% |
| YUV, mixed content, 1080p | -0.9% | -0.8% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | -0.5% | -0.3% | -0.5% |
| RGB, mixed content, 1440p | -0.4% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.5% | -0.4% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.0% | -0.9% |
| YUV, text & graphics with motion,720p | -0.9% | -0.8% | -0.6% |
| YUV, mixed content, 1440p | -0.7% | -1.0% | -0.3% |
| YUV, mixed content, 1080p | -0.7% | -0.6% | -1.0% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2. Lossy simulation result with 2CTU search area and bug-fixes applied.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.9% | -1.8% | -1.9% |
| RGB, text & graphics with motion,720p | -1.1% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.7% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.7% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.2% | -2.1% | -2.1% |
| YUV, text & graphics with motion,720p | -1.3% | -1.3% | -1.3% |
| YUV, mixed content, 1440p | -0.8% | -0.8% | -0.8% |
| YUV, mixed content, 1080p | -1.0% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | -0.7% | -0.7% | -0.6% |
| RGB, mixed content, 1440p | -0.4% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.4% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -0.9% | -1.0% |
| YUV, text & graphics with motion,720p | -0.8% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.4% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.7% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.5% |
| RGB, text & graphics with motion,720p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.4% | -0.3% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.6% |
| YUV, text & graphics with motion,720p | -0.4% | -0.6% | -0.4% |
| YUV, mixed content, 1440p | -0.4% | -0.1% | -0.2% |
| YUV, mixed content, 1080p | -0.5% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.6% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3. Lossless simulation result with full-frame search area and bug-fixes applied.

TBP

Table 4. Lossless simulation result with 2CTU search area and bug-fixes applied.

TBP

Table 5 shows the result of Table 1 but with the anchor left as SCM1.0.  The increase in gain is due to the bugfix macros shown above.  For convenience, a spreadsheet showing just the coding gain of the bugfixes is also included in the package.

Table 5. Lossy simulation result with full-frame search and bug-fixes applied, but SCM1.0 (no bugfixes) in anchor.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.3% | -4.2% | -4.2% |
| RGB, text & graphics with motion,720p | -3.0% | -3.0% | -2.9% |
| RGB, mixed content, 1440p | -1.5% | -1.4% | -1.5% |
| RGB, mixed content, 1080p | -1.9% | -1.7% | -1.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.8% | -4.5% | -4.5% |
| YUV, text & graphics with motion,720p | -3.5% | -3.4% | -3.4% |
| YUV, mixed content, 1440p | -1.9% | -1.8% | -1.8% |
| YUV, mixed content, 1080p | -2.0% | -1.9% | -1.9% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.1% | -2.2% |
| RGB, text & graphics with motion,720p | -2.0% | -1.9% | -2.0% |
| RGB, mixed content, 1440p | -1.0% | -0.9% | -0.8% |
| RGB, mixed content, 1080p | -1.2% | -1.1% | -1.2% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.3% | -2.2% | -2.3% |
| YUV, text & graphics with motion,720p | -2.3% | -2.1% | -2.1% |
| YUV, mixed content, 1440p | -1.1% | -1.2% | -1.2% |
| YUV, mixed content, 1080p | -1.4% | -1.6% | -1.3% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.5% | -1.5% | -1.4% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.7% |
| RGB, mixed content, 1440p | -0.7% | -0.7% | -0.5% |
| RGB, mixed content, 1080p | -0.8% | -0.6% | -1.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.5% | -1.5% |
| YUV, text & graphics with motion,720p | -1.1% | -0.9% | -1.0% |
| YUV, mixed content, 1440p | -0.8% | -1.3% | -0.9% |
| YUV, mixed content, 1080p | -0.8% | -1.2% | -0.8% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

# Working Draft changes

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| bvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| bvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| bvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| bvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| bvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| bvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| …….. |  |
| } else { |  |
| } |  |
| } |  |
| } |  |

| bvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_bvd_greater0_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| abs_bvd_greater0_flag[ x0 ][ y0 ] [ 1 ] | ae(v) |
| if( abs_bvd_greater0_flag[ x0 ][ y0 ] [ 0 ] ) { |  |
| abs_bvd_minus1[ x0 ][ y0 ] [ 0 ] | ae(v) |
| bvd_sign_flag[ x0 ][ y0 ] [ 0 ] | ae(v) |
| } |  |
| if( abs_bvd_greater0_flag[ x0 ][ y0 ] [ 1 ] ) { |  |
| abs_bvd_minus1[ x0 ][ y0 ] [ 1 ] | ae(v) |
| bvd_sign_flag[ x0 ][ y0 ] [ 1 ] | ae(v) |
| } |  |
| } |  |

| Table 9-6 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| bvd_coding( ) | abs_bvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_bvd_minus1[ ] | 9.3.3.10 | - |
|  | bvd_sign_flag[ ] | FL | cMax = 1 |

#### 9.3.3.10 Binarization process for abs_bvd_minus1[]

Input to this process is a request for a binarization for the syntax element abs_bvd_minus1 [ n ], where n is the component index of the delta block vector, with x=0 indicating the horizontal component and x=1 indicating the vertical component.

Output of this process is the binarization of the syntax element.

The variable initRiceValue is derived as follows:

- The variable sbType is assigned the value n.

- The variable initRiceValue is derived as follows:

initRiceValue = StatCoeffBvd[ sbType ] / 4

- StatCoeffBvd[ sbType ] is modified as follows:

if(abs_bvd_minus1 [ n ] >= ( 3  <<  ( StatCoeffBvd[ sbType ] / 4 ) ) )
	StatCoeffBvd[ sbType ]+ +
else if( 2 * abs_bvd_minus1[ n ] < ( 1  <<  ( StatCoeffBvd[ sbType ] / 4 ) ) )
	if( StatCoeffBvd[ sbType ] > 0 )
	           StatCoeffBvd[ sbType ]− −

The variable cMax is derived from cRiceParam as:

cMax = 4  <<  cRiceParam

The binarization of the syntax element abs_bvd_minus1 [ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of abs_bvd_minus1, prefixVal, is derived as follows:

prefixVal = Min( cMax, abs_bvd_minus1 [ n ] )	(9-1)

- The prefix bin string is specified by invoking the TR binarization process as specified in subclause Error: Reference source not found for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_qp_delta_abs, suffixVal, is derived as follows:

suffixVal = abs_bvd_minus1[ n ] − cMax	(9-2)

- The suffix bin string is specified by invoking the EGk binarization process as specified in subclause Error: Reference source not found for suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

# Conclusion

Presented are methods for binarizing delta block vectors for intra block copy.  Consideration of these methods for adoption into HEVC Screen content coding is requested.

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, Joint Collaborative Team on Video Coding, JCTVC-Q1015, Valencia, March, 2014.

- C. Pang, J. Sole, T. Hsieh, M. Karczewicz, “Intra block copy with larger search region”, Joint Collaborative Team on Video Coding, JCTVC-Q0139, Valencia, March, 2014.

- M. Karczewicz, L. Guo, J. Sole, R. Joshi, K. Sharman, N. Saunders, J. Gamei, “RCE2: Results of Test 1 on Rice Parameter Initialization”, Joint Collaborative Team on Video Coding, JCTVC-P0199, San Jose, January, 2014.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).