| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0124 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0033 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Improvements and clean-ups on external base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

On the current design of external base layer, the following improvements and clean-ups are proposed.

- Allowing the motion vector (or others) prediction from the base layer externally provided.

- Replacing the syntax vps_base_layer_internal_flag in VPS extension by a flag BaseLayerInternalFlag which is set by external means.

- Restrictions on vps_base_layer_internal_flag

  - It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, vps_max_layers_minus1 shall be greater than 0.

3.2. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, vps_max_layer_id shall be greater than 0.

3.3. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0 and the current nuh_layer_id is equal to TargetDecLayerIdList[1], pps_infer_scaling_list_flag and sps_infer_scaling_list_flag shall be equal to 0.

3.4. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, single_layer_for_non_irap_flag and higher_layer_irap_skip_flag shall be equal to 0.

3.5. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, base_layer_parameter_set_compatibility_flag[ i ] shall be equal to 0.

- Optimization of sub-DPB for the externally provided base layer

When all pictures directly referencing the decoded picture of the base layer are decoded, the sub-DPB for the base layer is set to be empty.

- Allowing any layer can be externally provided.

# Proposals

  - Relaxing the restriction of dependency type

It is proposed to allow the motion vector (or others) prediction from the base layer which is not available in the bitstreams and externally provided. There are no needs to impose the restriction that the base layer externally provided shall be compatible with video CODEC other than HEVC. If the base layer is compatible with HEVC, the motion vector prediction will be definitely efficient in terms of coding efficiency. If the base layer is not compatible with HEVC, direct_dependency_type[i][0] will be set equal to 0.

Proposed text changes:

| if( default_direct_dependency_flag ) |  |
| --- | --- |
| default_direct_dependency_type | u(v) |
| else { |  |
| for( i = vps_base_layer_internal_flag ? 1 : 2; i <= MaxLayersMinus1; i++ ) |  |
| for( j = vps_base_layer_internal_flag ? 0 : 1; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. direct_dependency_type[ i ][ j ] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction but not for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction but not for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for both inter-layer motion prediction and inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. The length of the direct_dependency_type[ i ][ j ] syntax element is direct_dep_type_len_minus2 + 2 bits. Although the value of direct_dependency_type[ i ][ j ] shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_type[ i ][ j ] in the range of 3 to 232 − 2, inclusive, to appear in the syntax.

When vps_base_layer_internal_flag is equal to 1 and direct_dependency_type[ i ][ j ] is not present, the value of direct_dependency_type[ i ][ j ] is inferred to be equal to default_direct_dependency_type.

When vps_base_layer_internal_flag is equal to 0, the value of direct_dependency_type[ i ][ 0 ] for i in the range of 1 to MaxLayersMinus1, inclusive, is inferred to be equal to 0.

  - Indication of base layer availability by external means

It is proposed that the syntax vps_base_layer_internal_flag in VPS extension is replaced by a flag BaseLayerInternalFlag which is set by external means, and the syntax changes affected by vps_base_layer_internal_flag revert to the original. If we allow a bitstream without a base layer, a sub-bitstream can be extracted by excluding a base layer from the original bitstream. Then, the value of vps_base_layer_internal_flag shall be changed. However, the availabilities of layers in the bitstream received by decoders are usually indicated by external means. Therefore, it is more reasonable to remove vps_base_layer_internal_flag and the syntax changes dependent on vps_base_layer_internal_flag to set the flag BaseLayerInternalFlag by external means.

1.3. Restrictions on vps_base_layer_internal_flag (or BaseLayerInternalFlag)

The following restrictions are proposed on vps_base_layer_internal_flag (or BaseLayerInternalFlag).

1.3.1. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, vps_max_layers_minus1 shall be greater than 0.

1.3.2. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, vps_max_layer_id shall be greater than 0.

1.3.3. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0 and the current nuh_layer_id is equal to TargetDecLayerIdList[1], pps_infer_scaling_list_flag and sps_infer_scaling_list_flag shall be equal to 0.

1.3.4. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, single_layer_for_non_irap_flag and higher_layer_irap_skip_flag shall be equal to 0.

1.3.5. It is a requirement of bitstream conformance that, when vps_base_layer_internal_flag is equal to 0, base_layer_parameter_set_compatibility_flag[ i ] shall be equal to 0.

1.4. Optimization of sub-DPB for the externally provided base layer.

In the current sub-DPB design for the externally provided base layer, the sub-DPB for the base layer is set to be empty after all pictures in the same access unit are decoded. However, in order to reduce the total size of DPB at decoders instantly, the sub-DPB of the base layer can be emptied when all pictures directly referencing the decoded picture of the base layer are decoded.

Proposed text changes:

When the access unit has at least one picture with nuh_layer_id greater than 0, after all pictures directly referencing the decoded picture with nuh_layer_id equal to 0 are decoded, the sub-DPB for the layer with nuh_layer_id equal to 0 is set to be empty.

1.5. Generalization of external layers.

The external base layer can be straightforwardly generalized to the external layers, which allow that any layer can be externally provided, not contained in the bitstream. This generalization brings more extensibility of hybrid codec scalability, e.g. HEVC(base) + other future codec (enhancement).

Proposed text changes:

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| vps_layer_internal_flag[ i ] | u(1) |
| … |  |
| } |  |

vps_layer_internal_flag[i] equal to 0 specifies that the i-th layer is provided by an external means not specified in this Specification. vps_layer_internal_flag[i] equal to 1 specifies that the i-th layer is provided in the bitstream.

When vps_layer_internal_flag[i] is equal to 0, the following applies:

- The value of vps_sub_layer_ordering_info_present_flag shall be equal to 0.

- The values of vps_max_dec_pic_buffering_minus1[ i ], vps_max_num_reorder_pics[ i ], and vps_max_latency_increase_plus1[ i ] shall all be equal to 0.

- Decoders shall ignore the values of vps_sub_layer_ordering_info_present_flag, vps_max_dec_pic_buffering_minus1[ i ], vps_max_num_reorder_pics[ i ], and vps_max_latency_increase_plus1[ i ].

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).