| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0125 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0034 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Non-base layer subtree and auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Several corrections and restrictions on non-base layer subtree and auxiliary pictures are proposed, as follows:

- In order to keep the compatibility of slice headers with nuh_layer_id equal to 0 after non-base layer subtree extraction, poc_lsb_not_present_flag[ i ] shall be equal to 1, when layer_id_in_nuh[ i ] is equal to AssignedBaseLayerId[ j ].

- After non-base layer subtree extraction, VPS shall have vps_base_layer_internal_flag equal to 1, when a VPS is present in outBitstream.

- A bug fix in highest_layer_idx_plus1 derivation is reported.

- The base layer pictures shall not be auxiliary pictures.

- AssignedBaseLayerId[i] is set equal to the minimum nuh_layer_id among the layers belonging to the i-th layer set, when AuxId[nuh_layer_id] is equal to 0.

- base_layer_parameter_set_compatibility_flag[ i ] shall be equal to 0, when AuxId[layer_id_in_nuh[ i ]] is greater than 0.

# Proposals

1.1. Compatibility of slice headers after non-base layer subtree extraction.

In order to keep the compatibility of slice headers with nuh_layer_id equal to 0 after non-base layer subtree extraction, the following restriction is proposed.

Proposed text:

It is a requirement of bitstream conformance that poc_lsb_not_present_flag[ i ] shall be equal to 1, when layer_id_in_nuh[ i ] is equal to AssignedBaseLayerId[ j ].

1.2. Restriction of the vps_base_layer_internal_flag after non-base layer subtree extraction.

After non-base layer subtree extraction, VPS shall have vps_base_layer_internal_flag equal to 1, when a VPS is present in outBitstream.

Proposed text:

- 5. Non-base layer subtree extraction process This process may be applied when num_add_layer_sets is greater than 0 in the active VPSs of the input bitstream. … the following applies: – If an access unit delimiter NAL unit is present in the same access unit that contains the SEI NAL unit, the nal_unit( ) in the payload of the VPS rewriting SEI message is inserted into outBitstream as the first NAL unit following the access unit delimiter NAL unit in decoding order. – Otherwise, the nal_unit( ) in the payload of the VPS rewriting SEI message is inserted into outBitstream as the first NAL unit, in decoding order, of the access unit. – The SEI NAL unit is removed. Note - vps_base_layer_internal_flag of VPS in outBitstream shall be equal to 1, when the VPS is present in outBitstream.

1.3. Bug fix(?) in highest_layer_idx_plus1 derivation

Proposed text:

highest_layer_idx_plus1[ i ][ j ] specifies the values of NumLayersInIdList[ vps_num_layer_sets_minus1 + 1 + i ] and LayerSetLayerIdList[ vps_num_layer_sets_minus1 + 1 + i ][ layerNum ] and is used to infer layer_id_included_flag[ vps_num_layer_sets_minus1 + 1 + i ][ layerId ] as follows: layerNum = 0 lsIdx = vps_num_layer_sets_minus1 + 1 + i for( layerId = 0; layerId <= 62; layerId++ ) layer_id_included_flag[ lsIdx ][ layerId ] = 0 for( treeIdx = 1; treeIdx < NumIndependentLayers; treeIdx++ ) for( layerCnt = 0; layerCnt <highest_layer_idx_plus1[ i ][ jtreeIdx ]; layerCnt++ ) { LayerSetLayerIdList[ lsIdx ][ layerNum ] = TreePartitionLayerIdList[ treeIdx ][ layerCnt ] layer_id_included_flag[ lsIdx ][ TreePartitionLayerIdList[ treeIdx ][ layerCnt ] ] = 1 layerNum++ } NumLayersInIdList[ lsIdx ] = layerNum The value of highest_layer_idx_plus1[ i ][ j ] shall be in the range of 0 to NumLayersInTreePartition[ j ], inclusive. The length of highest_layer_idx_plus1[ i ][ j ] is equal to Ceil( Log2( NumLayersInTreePartition[ j ] + 1 ) ).

1.4. Restriction of the AuxId[0] value

It is proposed to impose a restriction that the base layer pictures shall not be auxiliary pictures. Auxiliary pictures will not affect the normative decoding process of primary pictures. The auxiliary pictures may be used for post-processing of the associated primary pictures, after independently decoded. Note that the depth map and alpha pictures are basically monochrome with chroma_format_idc equal to 0, which is not compatible with HEVC main profile, even if compatible with other future profiles.

Even if this restriction is not agreed, at least AuxId[0] shall not be equal to AUX_ALPHA. Alpha map pictures are useless without the associated primary pictures.

If the above restriction is not agreed at all, we recommend to change AuxId to PicTypeId, and remove the name auxiliary picture.

Proposed text:

It is a requirement of bitstream conformance that AuxId[0] shall be equal to 0.

  - Assignment of AssignedBaseLayerId

For non-base layer subtree extraction, AssignedBaseLayerId is set equal to the minimum nuh_layer_id among the layers belonging to the layer set. However, it shall be prevented that the layer with auxiliary pictures is set to the assigned base layer for non-base layer subtree extraction.

Proposed text:

highest_layer_idx_plus1[ i ][ j ] specifies the values of NumLayersInIdList[ vps_num_layer_sets_minus1 + 1 + i ] and LayerSetLayerIdList[ vps_num_layer_sets_minus1 + 1 + i ][ layerNum ] and is used to infer layer_id_included_flag[ vps_num_layer_sets_minus1 + 1 + i ][ layerId ] as follows: layerNum = 0 lsIdx = vps_num_layer_sets_minus1 + 1 + i for( layerId = 0; layerId <= 62; layerId++ ) layer_id_included_flag[ lsIdx ][ layerId ] = 0 for( treeIdx = 1; treeIdx < NumIndependentLayers; treeIdx++ ) for( layerCnt = 0; layerCnt <highest_layer_idx_plus1[ i ][ j ]; layerCnt++ ) { LayerSetLayerIdList[ lsIdx ][ layerNum ] = TreePartitionLayerIdList[ treeIdx ][ layerCnt ] layer_id_included_flag[ lsIdx ][ TreePartitionLayerIdList[ treeIdx ][ layerCnt ] ] = 1 layerNum++ } NumLayersInIdList[ lsIdx ] = layerNum The value of highest_layer_idx_plus1[ i ][ j ] shall be in the range of 0 to NumLayersInTreePartition[ j ], inclusive. The length of highest_layer_idx_plus1[ i ][ j ] is equal to Ceil( Log2( NumLayersInTreePartition[ j ] + 1 ) ). It is a requirement of bitstream conformance that NumLayersInIdList[ vps_num_layer_sets_minus1 + 1 + i ] shall be greater than 0. AssignedBaseLayerId[ vps_num_layer_sets_minus1 + 1 + i ] is set equal to the smallest nuh_layer_id value in NumLayersInIdList[ vps_num_layer_sets_minus1 + 1 + i ], when AuxId[nuh_layer_id] is equal to 0.

1.6. On base_layer_parameter_set_compatibility_flag

The parameter sets for auxiliary pictures may not be compatible with the parameter sets of the base layer.

Proposed text:

base_layer_parameter_set_compatibility_flag[ i ] equal to 1 specifies that the following constraints apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ].base_layer_parameter_set_compatibility_flag[ i ] equal to 0 specifies that the following constraints may or may not apply to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a PPS with nuh_layer_id value equal to 0. - Each coded slice segment NAL unit with nuh_layer_id value equal to layer_id_in_nuh[ i ] referring to the VPS shall refer to a SPS with nuh_layer_id value equal to 0. - The values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the active SPS for the layer with nuh_layer_id equal to layer_id_in_nuh[ i ] shall be the same as the values of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8, respectively, of the vps_rep_format_idx[ i ]-th rep_format( ) syntax structure in the active VPS. It is a requirement of bitstream conformance that base_layer_parameter_set_compatibility_flag[ i ] shall be equal to 0, when AuxId[layer_id_in_nuh[ i ]] is greater than 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).