| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0127 |
| --- | --- |

| Title: | Cross check of ‘SCCE3: Test B.12 – Binarization of Escape Sample and Palette Index’ (JCTVC-R0065) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne Building A, The Park Estate, 5 Talavera Road, Macquarie Park NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check result of SCCE3 Test B.12 on escape pixel coding for palette mode. The cross-check confirms the simulation results reported by the proponent.

# Cross check procedure

The source code for test B.12 of Screen Content Core Experiment 3 (SCCE3), as defined in [1], was provided by Qualcomm. Canon compiled the source code using GCC 4.7.1 and the simulation was performed on a Linux cluster under lossy and lossless conditions, as defined in [2]. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was provided by Qualcomm and configured for test B.12.  This configuration is as follows:

- Truncated binary coding for the index and escape pixel was enabled.

- Check of above and left for truncated binary coding to reduce range of indices by 1 was enabled.

- CU-level escape pixel existence flag was introduced.

- The escape flag was disabled; instead the palette index was used to signal escape instead.

- Truncated binary code introduced to code each component of escape values.

- Palette index check introduced:

  - Truncated binarisation for run, exclude copy-from-above signaling if the upper pixel is escape or left and above palette indices are equal.

# Coding performance measurement

Table 1 and Table 2 show the lossy and lossless simulation result respectively for test B.12 of SCCE3 with a full-frame intra-block copy search area. The anchor is the Palette base software distributed to participants in SCCE3.

Table 1. Lossy simulation result with full-frame intra-block copy search.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.6% | -1.6% |
| RGB, text & graphics with motion,720p | -1.1% | -1.1% | -1.0% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.1% | -2.0% | -1.9% |
| YUV, text & graphics with motion,720p | -0.8% | -1.3% | -1.6% |
| YUV, mixed content, 1440p | -0.2% | -0.4% | -0.5% |
| YUV, mixed content, 1080p | -0.5% | -0.8% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.0% | -0.9% |
| RGB, text & graphics with motion,720p | -1.0% | -0.9% | -0.9% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.2% | -1.2% |
| YUV, text & graphics with motion,720p | -0.7% | -1.1% | -1.6% |
| YUV, mixed content, 1440p | -0.2% | -0.5% | -0.7% |
| YUV, mixed content, 1080p | -0.3% | -0.7% | -0.7% |
| YUV, Animation, 720p | 0.1% | 0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.8% |
| RGB, mixed content, 1440p | -0.4% | -0.3% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.7% | -1.0% | -1.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.6% | -1.0% |
| YUV, mixed content, 1440p | -0.4% | -0.7% | -1.0% |
| YUV, mixed content, 1080p | -0.2% | -1.5% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2. Lossless simulation result with full-frame intra-block copy search.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.9% | 1.0% | 0.2% | 1.6% |
| RGB, text & graphics with motion,720p | 0.3% | 0.4% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.2% | 0.2% | 2.0% |
| YUV, text & graphics with motion,720p | 0.5% | 0.6% | 0.0% | 1.1% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.7% | 0.2% | 1.5% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.6% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.7% | 0.2% | 1.7% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.7% | 0.2% | 1.5% |
| RGB, text & graphics with motion,720p | 0.0% | 0.2% | 0.0% | 0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.6% | 0.2% | 1.4% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.6% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 3 and Table 4 show the lossy and lossless simulation result for test B.11 of SCCE3 with a 2 CTU intra-block copy search area.

Table 3. Lossy simulation result with 2 CTU intra-block copy search area.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.8% | -2.8% | -2.7% |
| RGB, text & graphics with motion,720p | -1.6% | -1.5% | -1.5% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.3% | -3.1% | -3.1% |
| YUV, text & graphics with motion,720p | -1.3% | -1.8% | -2.1% |
| YUV, mixed content, 1440p | -0.2% | -0.5% | -0.6% |
| YUV, mixed content, 1080p | -0.7% | -1.0% | -1.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.4% | -1.5% | -1.5% |
| RGB, text & graphics with motion,720p | -1.3% | -1.2% | -1.3% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.7% | -1.6% |
| YUV, text & graphics with motion,720p | -1.1% | -1.4% | -2.0% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.7% |
| YUV, mixed content, 1080p | -0.4% | -1.0% | -0.9% |
| YUV, Animation, 720p | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.1% | -1.1% |
| RGB, text & graphics with motion,720p | -1.0% | -0.9% | -1.0% |
| RGB, mixed content, 1440p | -0.3% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.5% | -0.6% | -0.5% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.1% | -1.1% |
| YUV, text & graphics with motion,720p | -0.7% | -1.0% | -1.2% |
| YUV, mixed content, 1440p | -0.2% | -0.6% | -0.7% |
| YUV, mixed content, 1080p | -0.3% | -1.6% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4. Lossless simulation result with 2 CTU intra-block copy search area.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.6% | 1.8% | 0.2% | 2.8% |
| RGB, text & graphics with motion,720p | 0.4% | 0.6% | 0.1% | 1.7% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.8% | 1.9% | 0.2% | 2.8% |
| YUV, text & graphics with motion,720p | 0.7% | 0.9% | 0.1% | 2.2% |
| YUV, mixed content, 1440p | 0.2% | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 1.1% | 0.2% | 2.4% |
| RGB, text & graphics with motion,720p | 0.1% | 0.4% | 0.0% | 1.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 1.1% | 0.2% | 2.5% |
| YUV, text & graphics with motion,720p | 0.1% | 0.6% | 0.0% | 1.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 0.3% | 1.0% | 0.2% | 2.2% |
| RGB, text & graphics with motion,720p | 0.1% | 0.2% | 0.0% | 0.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.9% | 0.2% | 2.1% |
| YUV, text & graphics with motion,720p | 0.1% | 0.3% | 0.0% | 0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

# Conclusion

The performance of palette mode with truncated binary coding for palette indices, escape pixels, codeword redundancy reduction in the palette index signalling and the mode signalling, and a CU-level escape existence flag were tested in test B.12 of SCCE3.  The results provided by the proponents are verified in this cross-check report.

# References

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “Description of Screen Content Core Experiment 3 (SCCE3): Palette mode”, JCTVC-Q1123, Valencia, ES, March 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, Joint Collaborative Team on Video Coding, JCTVC-Q1015, Valencia, ES, March, 2014.

- “SCCE3: Test B.12 – Binarization of Escape Sample and Palette Index”, JCTVC-R0065, Sapporo, JP, June, 2014.