| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0128 |
| --- | --- |

| Title: | High 4:4:4 16 Intra Profile Specification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders, J. Gamei Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355 011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
|  | T. Suzuki 2-10-1, Osaki, Shinagawa-ku, Tokyo, 141-8610 JAPAN | Tel: Email: | +81 50 3750 2740 teruhikos@jp.sony.com |
|  | A. Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | +1 408 352 4715 ali.tabatabai@am.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution provides a specification for the High 4:4:4 16 intra profile that was agreed at the previous JCT-VC meeting to be planned for inclusion in one of the amendments resulting from the Sapporo meeting.

# Introduction

At the previous JCT-VC meeting in Valencia, it was agreed that a new “High 4:4:4 16 Intra” profile would be planned for inclusion in one of the amendments resulting from the Sapporo meeting. This profile would allow the maximum bit rate to be increased to the point where compression ratios suitable for high end professional content creation can be supported. This contribution provides a proposed specification for that profile, broadly reiterating the previous document (JCTVC-Q0186).

By modifying the definition of HbrFactor, the maximum bit rate and MinCr can be modified without affecting the coded picture buffer size (which would otherwise be unnecessarily large).

As there are no suitable constraint flags that would be interpreted by a version 2 decoder (the unused flags are all 0, and therefore are unconstrained), the proposed mechanism by which the new profile can be signalled is to use a new profile_idc.

The definition of the new profile would be identical to the definition of “Main 4:4:4 16 Intra”, except that:

- HbrFactor is increased by a factor of 12 (i.e. resulting in scaling by 12 or 24, instead of 1 or 2).

- The value of cabac_bypass_alignment_enabled_flag would be specified to be 1.

- Decoders supporting this profile would not automatically be required to support any other profiles.

- Wavefront-parallel processing and tiles may be used simultaneously, in order to reduce the system delay and to increase throughput.

# Bit Rate Discussion

During RExt development, studies were conducted under the AHG18 test conditions up to 2:1 compression for 16-bit source video. For the test material used, higher compression ratios did not lend themselves well to the study of high bit depths – as the ratios increased, the least significant bits in the output video became noise. This 2:1 operating point is also common for high end content creation, and has been used in lower bit-depth systems such as HDCAM-SR (which uses MPEG4-SStP (see JCTVC-L0182)).

It is proposed to use an HbrFactor 12 times larger than in Main. This would (when used in combination with general_lower_bit_rate_constraint_flag = 0, and the CpbVclFactor of 4000) result in bit rates of:

|  | Profile | Tier | Level |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 | 4.1 | 5 | 5.1 | 5.2 | 6 | 6.1 | 6.2 |
| Maximum bit rate (Mbps) | Main 4:4:4 16 Intra | Main | 96 | 160 | 200 | 320 | 480 | 480 | 960 | 1920 |
|  |  | High | 240 | 400 | 800 | 1280 | 1920 | 1920 | 3840 | 6400 |
|  | High 4:4:4 16 Intra | Main | 1152 | 1920 | 2400 | 3840 | 5760 | 5760 | 11520 | 23040 |
|  |  | High | 2880 | 4800 | 9600 | 15360 | 23040 | 23040 | 46080 | 76800 |
| Compression ratio of 4:4:4 16-bit video at level-targeted sample rate at maximum bit rate | Main 4:4:4 16 Intra | Main | 33.4 | 40.1 | 64.2 | 80.2 | 107.0 | 107.0 | 107.0 | 107.0 |
|  |  | High | 13.4 | 16.0 | 16.0 | 20.1 | 26.7 | 26.7 | 26.7 | 32.1 |
|  | High 4:4:4 16 Intra | Main | 2.8 | 3.3 | 5.3 | 6.7 | 8.9 | 8.9 | 8.9 | 8.9 |
|  |  | High | 1.1 | 1.3 | 1.3 | 1.7 | 2.2 | 2.2 | 2.2 | 2.7 |

This combination of Main and High profiles with Main and High tiers provides a wide range of bit rates and compression ratios.

The factor-of-12 increase is particularly important at level 5.2 (4K 120P); there is no room to create a higher bit rate level 5.3 as the bit-rate must be less than that of level 6. Of all levels, level 5.2 is expected to be of greatest immediate interest to the content creation industry. For 8K, the option remains to add further levels beyond 6.2 in the future.

# Text Changes

(Relative to JCTVC-Q1005_v9)

The text will require the following sections to be changed:

- Section 7.3.3 would be changed as follows:

7.3.3	Profile, tier and level syntax

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| general_profile_compatibility_flag[ j ] | u(1) |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| if( (general_profile_idc = = 4) ¦ ¦ (general_profile_idc = = 5) ¦ ¦ general_profile_compatibility_flag[ 4 ] ¦ ¦ general_profile_compatibility_flag[ 5 ]) { /* The number of bits in this syntax structure is not affected by this condition */ |  |
| general_max_12bit_constraint_flag | u(1) |
| general_max_10bit_constraint_flag | u(1) |
| general_max_8bit_constraint_flag | u(1) |
| general_max_422chroma_constraint_flag | u(1) |
| general_max_420chroma_constraint_flag | u(1) |
| general_max_monochrome_constraint_flag | u(1) |
| general_intra_constraint_flag | u(1) |
| general_one_picture_only_constraint_flag | u(1) |
| general_lower_bit_rate_constraint_flag | u(1) |
| general_reserved_zero_35bits | u(35) |
| } else |  |
| general_reserved_zero_44bits | u(44) |
| general_level_idc | u(8) |
| … |  |

- Section 7.4.4 would be changed as follows:

- 7.4.4	Profile, tier and level semantics

…

general_frame_only_constraint_flag equal to 1 specifies that field_seq_flag is equal to 0. general_frame_only_constraint_flag equal to 0 indicates that field_seq_flag may or may not be equal to 0.

NOTE 1 – Decoders may ignore the value of general_frame_only_constraint_flag, as there are no decoding process requirements associated with the value of field_seq_flag.

NOTE 2 – When general_progressive_source_flag is equal to 1, general_frame_only_constraint_flag may or may not be equal to 1.

general_max_12bit_‌constraint_flag, general_max_10bit_‌constraint_flag, general_max_8bit_‌constraint_flag, general_max_422chroma_‌constraint_flag, general_max_420chroma_‌constraint_flag, general_max_monochrome_‌constraint_flag, general_intra_‌constraint_flag, general_one_picture_only_‌constraint_flag, and general_lower_bit_rate_‌constraint_flag, when present, have semantics specified in Annex Error: Reference source not found. When general_profile_idc is not equal to 4 or 5 and general_profile_compatibility_flag[ 4 ] is not equal to 1 and general_profile_compatibility_flag[ 5 ] is not equal to 1, these syntax elements are not present and are inferred to be equal to 0.

general_reserved_zero_35bits, when present, shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for general_reserved_zero_35bits are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of general_reserved_zero_35bits.

…

A new section A.3.6 would be added as follows:

- A.3.6	Format range extensions high profiles

The following profiles, collectively referred to as the format range extensions high profiles, are specified in this subclause:

- The High 4:4:4 16 Intra profile.

Bitstreams conforming to the format range extensions high profiles shall obey the following constraints:

- SPSs shall have separate_colour_plane_flag, when present, equal to 0 only.

- SPSs shall have cabac_bypass_alignment_enabled_flag equal to 1 only.

- CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

- When a PPS has tiles_enabled_flag equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- In bitstreams conforming to the High 4:4:4 16 Intra profile, all pictures shall be IRAP pictures and the output order indicated in the bitstream shall be the same as the decoding order.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- In bitstreams conforming to the High 4:4:4 16 Intra profile, general_level_idc shall not be equal to 255 (which indicates level 8.5).

- The level constraints specified for the High 4:4:4 16 Intra profile in subclause Error: Reference source not found, as applicable, shall be fulfilled.

Conformance of a bitstream to the format range extensions high profiles is indicated by general_profile_idc being equal to 5 or general_profile_compatibility_flag[ 5 ] being equal to 1 with the additional indications specified in Table A-X.

All other combinations of general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_8bit_constraint_flag, general_max_422chroma_constraint_flag, general_max_420chroma_constraint_flag, general_max_monochrome_constraint_flag, general_intra_constraint_flag, general_one_picture_only_constraint_flag, and general_lower_bit_rate_constraint_flag with general_profile_idc equal to 5 or general_profile_compatibility_flag[ 5 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this Specification.

Table A-X – Bitstream indications for conformance to format range extensions high profiles

| Profile for which the bitstream indicates conformance | general_max_12bit_constraint_flag | general_max_10bit_constraint_flag | general_max_8bit_constraint_flag | general_max_422chroma_constraint_flag | general_max_420chroma_constraint_flag | general_max_monochrome_constraint_flag | general_intra_constraint_flag | general_one_picture_only_constraint_flag | general_lower_bit_rate_constraint_flag |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High 4:4:4 16 Intra | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 or 1 |

Section A.4.2 would be changed as follows:

- A.4.2	Profile-specific level limits for the video profiles

…

The variable HbrFactor is defined as follows:

- If general_profile_idc is equal to 1 or 2, or general_profile_compatibility_flag[ 1 ] is equal to 1, or general_profile_compatibility_flag[ 2 ] equal to 1, HbrFactor is set equal to 1.

- Otherwise, if general_profile_idc is equal to 5, or general_profile_compatibility_flag[ 5 ] is equal to 1, HbrFactor is set equal to 24 – (12 * general_lower_bit_rate_constraint_flag).

- Otherwise, HbrFactor is set equal to 2 − general_lower_bit_rate_constraint_flag.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

Bitstreams conforming to the Monochrome 12, Monochrome 16, Main, Main 10, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10, Main 4:4:4 12, Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 10 Intra, Main 4:4:4 12 Intra, or Main 4:4:4 16 Intra, or High 4:4:4 16 Intra profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

…

Table A-5 would be changed as follows:

Table A-5 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor, and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Monochrome 12 | 1000 | 1100 | 1.500 | 1.0 |
| Monochrome 16 | 1333 | 1467 | 2.000 | 1.0 |
| Main | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 | 1500 | 1650 | 2.250 | 1.0 |
| Main 4:2:2 10 | 1667 | 1833 | 2.500 | 0.5 |
| Main 4:2:2 12 | 2000 | 2200 | 3.000 | 0.5 |
| Main 4:4:4 10 | 2500 | 2750 | 3.750 | 0.5 |
| Main 4:4:4 12 | 3000 | 3300 | 4.500 | 0.5 |
| Main 4:4:4 | 2000 | 2200 | 3.000 | 0.5 |
| Main Intra | 1000 | 1100 | 1.500 | 1.0 |
| Main 10 Intra | 1000 | 1100 | 1.875 | 1.0 |
| Main 12 Intra | 1500 | 1650 | 2.250 | 1.0 |
| Main 4:2:2 10 Intra | 1667 | 1833 | 2.500 | 0.5 |
| Main 4:2:2 12 Intra | 2000 | 2200 | 3.000 | 0.5 |
| Main 4:4:4 Intra | 2000 | 2200 | 3.000 | 0.5 |
| Main 4:4:4 10 Intra | 2500 | 2750 | 3.750 | 0.5 |
| Main 4:4:4 12 Intra | 3000 | 3300 | 4.500 | 0.5 |
| Main 4:4:4 16 Intra | 4000 | 4400 | 6.000 | 0.5 |
| High 4:4:4 16 Intra | 4000 | 4400 | 6.000 | 0.5 |

Section A.4.3 would be changed as follows:

- A.4.3	Effect of level limits on picture rate for the video profiles (informative)

This subclause does not form an integral part of this Specification.

Informative Tables Error: Reference source not found and Error: Reference source not found provide examples of maximum picture rates for the Monochrome 12, Monochrome 16, Main, Main 10, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4, Main 4:4:4 10, Main 4:4:4 12, Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 Intra, Main 4:4:4 10 Intra, Main 4:4:4 12 Intra, and Main 4:4:4 16 Intra, and High 4:4:4 16 Intra profiles for various picture formats when MinCbSizeY is equal to 64.

…

# Conclusion

A proposed specification has been presented for the agreed High 4:4:4 16 Intra profile. It is indicated that this profile would allow for high-quality compression at professional operating points.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).