| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0130 |
| --- | --- |

| Title: | Crosscheck of SCCE4 Test 3.1 on Combined Normal 1-D Dictionary Mode and Reconstruction Based 1-D Dictionary Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, 1Chun-Chi Chen, Tsui-Shan Chang, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of SCCE4 Test 3.1 on combined normal 1-D dictionary mode and reconstruction based 1-D dictionary mode. The software package provided by the proponent was tested under the screen content common test conditions specified in JCTVC-Q1015 [1]. The BD-rate results perfectly match the data provided by the proponent while decrease of encoding time (-20% and -40% for All-Intra lossy and lossless configuration, respectively) relative to the proponent’s results is observed. Table 1 and Table 2 show the simulation results for lossy and lossless coding, respectively.

# Test results

Table 1. Simulation results of SCCE4 Test 3.1 for lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -21.3% | -22.1% | -22.0% |
| RGB, text & graphics with motion,720p | -13.0% | -13.4% | -13.2% |
| RGB, mixed content, 1440p | -2.5% | -2.5% | -2.5% |
| RGB, mixed content, 1080p | -6.0% | -6.1% | -6.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -20.7% | -21.3% | -20.6% |
| YUV, text & graphics with motion,720p | -10.4% | -12.7% | -14.3% |
| YUV, mixed content, 1440p | -2.0% | -2.2% | -2.1% |
| YUV, mixed content, 1080p | -5.1% | -5.2% | -4.8% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 135% |  |  |
| Dec Time[%] | 96% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -13.3% | -14.3% | -14.4% |
| RGB, text & graphics with motion,720p | -12.3% | -12.5% | -12.6% |
| RGB, mixed content, 1440p | -2.8% | -2.6% | -2.6% |
| RGB, mixed content, 1080p | -4.5% | -4.8% | -4.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -12.9% | -14.6% | -14.3% |
| YUV, text & graphics with motion,720p | -10.2% | -13.3% | -14.5% |
| YUV, mixed content, 1440p | -1.5% | -2.5% | -2.3% |
| YUV, mixed content, 1080p | -4.3% | -4.7% | -4.1% |
| YUV, Animation, 720p | 0.2% | -0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -8.7% | -9.6% | -9.6% |
| RGB, text & graphics with motion,720p | -7.3% | -7.5% | -7.4% |
| RGB, mixed content, 1440p | -1.9% | -2.1% | -2.0% |
| RGB, mixed content, 1080p | -2.5% | -2.7% | -2.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -7.4% | -9.5% | -9.2% |
| YUV, text & graphics with motion,720p | -5.9% | -8.2% | -9.1% |
| YUV, mixed content, 1440p | -1.2% | -2.2% | -2.3% |
| YUV, mixed content, 1080p | -1.4% | -3.2% | -1.6% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.2% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Simulation results of SCCE4 Test 3.1 for lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 27.9% | 28.0% | 19.9% | 39.1% |
| RGB, text & graphics with motion,720p | 15.0% | 13.1% | 0.7% | 24.9% |
| RGB, mixed content, 1440p | 2.9% | 3.1% | 2.0% | 4.3% |
| RGB, mixed content, 1080p | 4.1% | 4.1% | 4.1% | 4.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 31.2% | 31.2% | 18.3% | 43.6% |
| YUV, text & graphics with motion,720p | 14.0% | 13.8% | 0.6% | 23.4% |
| YUV, mixed content, 1440p | 3.1% | 3.3% | 1.8% | 4.8% |
| YUV, mixed content, 1080p | 4.2% | 4.2% | 4.2% | 4.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 155% |  |  |  |
| Dec Time[%] | 94% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 15.8% | 18.9% | 13.0% | 27.9% |
| RGB, text & graphics with motion,720p | 2.3% | 6.4% | 0.3% | 14.3% |
| RGB, mixed content, 1440p | 0.5% | 0.5% | 0.3% | 0.6% |
| RGB, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 15.7% | 21.5% | 14.6% | 31.6% |
| YUV, text & graphics with motion,720p | 2.1% | 7.5% | 0.3% | 19.8% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.4% | 0.5% |
| YUV, mixed content, 1080p | 0.7% | 0.7% | 0.7% | 0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 14.9% | 15.0% | 11.1% | 18.7% |
| RGB, text & graphics with motion,720p | 1.5% | 4.3% | 0.3% | 9.3% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 14.4% | 17.2% | 14.0% | 21.8% |
| YUV, text & graphics with motion,720p | 1.2% | 5.0% | 0.3% | 12.8% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

# Reference

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, Mar. 2014.