| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0131 |
| --- | --- |

| Title: | SCCE2: Line-based Intra Copy with 2-D BV and PU-level Splitting (Test B), and Its Variants with Constrained PU-level Splitting (Test C) and BV Merging (Test D) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report/Proposal |  |  |
| Author(s) or Contact(s): | Tsui-Shan Chang, 1Chun-Chi Chen, Ru-Ling Liao, Che-Wei Kuo, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
|  | 3Tao Lin, Liping Zhao 1239 Siping Ave, Shanghai 200092, China |  | 3lintao@tongji.edu.cn |
|  | Chun-Lung Lin, 4Fan-Di Jou 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan |  | 4Fandi@itri.org.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International, Tongji University |  |  |

_____________________________

# Abstract

In the context of the Screen Content Core Experiment 2 (JCTVC-Q1122), this contribution provides test results for the technique of the line-based intra copy. This mode extends the basic processing unit for intra copy from blocks to lines, which divides a prediction block horizontally or vertically into 1-pixel-wide lines and then operates intra copying line-by-line from part of the reconstructed pixels, to the left or on the top of the lines, in the current frame. Three different settings for the line-based intra copy are tested and their coding results are reported as follows:

- (Test B) line-based intra copy with 2-D BV and PU-level splitting:

  - RGB, text & graphics with motion, 1080p	: (AI) -7.1%,	(RA) -4.1%,	(LB) -2.6%;

  - RGB, text & graphics with motion, 720p	: (AI) -3.2%, 	(RA) -2.4%, 	(LB) -1.5%;

  - RGB, mixed content, 1440p			: (AI) -2.9%, 	(RA) -2.2%, 	(LB) -1.3%;

  - RGB, mixed content, 1080p			: (AI) -3.6%, 	(RA) -2.6%, 	(LB) -1.2%;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.7%, 	(RA) -3.7%, 	(LB) -2.2%;

  - YUV, text & graphics with motion, 720p	: (AI) -2.6%, 	(RA) -2.0%, 	(LB) -0.7%;

  - YUV, mixed content, 1440p			: (AI) -2.3%, 	(RA) -1.4%, 	(LB) -0.6%;

  - YUV, mixed content, 1080p			: (AI) -3.2%, 	(RA) -2.4%, 	(LB) -0.7%;

  - Encoding time				: (AI) 115%, 	(RA) 103%, 	(LB) 103%;

  - Decoding time				: (AI) 101%, 	(RA) 102%, 	(LB) 102%;

- (Test C) Test B + constrained PU-level splitting:

  - RGB, text & graphics with motion, 1080p	: (AI) -7.0%, 	(RA) -4.1%, 	(LB) -2.5%;

  - RGB, text & graphics with motion, 720p	: (AI) -3.2%, 	(RA) -2.3%, 	(LB) -1.5%;

  - RGB, mixed content, 1440p			: (AI) -2.8%, 	(RA) -2.1%, 	(LB) -1.1%;

  - RGB, mixed content, 1080p			: (AI) -3.7%, 	(RA) -2.6%, 	(LB) -1.3%;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.6%, 	(RA) -3.6%, 	(LB) -2.2%;

  - YUV, text & graphics with motion, 720p 	: (AI) -2.5%, 	(RA) -2.1%, 	(LB) -0.9%;

  - YUV, mixed content, 1440p			: (AI) -2.2%, 	(RA) -1.3%, 	(LB) -0.7%;

  - YUV, mixed content, 1080p			: (AI) -3.2%, 	(RA) -2.4%, 	(LB) -0.5%;

  - Encoding time				: (AI) 113%, 	(RA) 103%, 	(LB) 103%;

  - Decoding time				: (AI) 101%, 	(RA) 104%, 	(LB) 103%;

- (Test D) Test C + block vector merging:

  - RGB, text & graphics with motion, 1080p	: (AI) -6.9%,	(RA) -3.9%,	(LB) -2.3%;

  - RGB, text & graphics with motion, 720p	: (AI) -3.2%,	(RA) -2.4%,	(LB) -1.6%;

  - RGB, mixed content, 1440p			: (AI) -2.7%,	(RA) -1.9%,	(LB) -1.1%;

  - RGB, mixed content, 1080p			: (AI) -3.6%,	(RA) -2.6%,	(LB) -1.2%;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.6%,	(RA) -3.6%,	(LB) -2.1%;

  - YUV, text & graphics with motion, 720p	: (AI) -2.7%,	(RA) -2.0%,	(LB) -0.8%;

  - YUV, mixed content, 1440p			: (AI) -2.1%,	(RA) -1.3%,	(LB) -0.6%;

  - YUV, mixed content, 1080p			: (AI) -3.0%,	(RA) -2.3%,	(LB) -0.6%;

  - Encoding time				: (AI) 115%,	(RA) 104%,	(LB) 102%;

  - Decoding time				: (AI) 103%,	(RA) 101%,	(LB) 101%.

# Introduction

After the completion of the Call-for-proposals on Screen Content Coding [1] at the 17th JCT-VC meeting, the very first test model of screen content coding (SCM-1.0) was initiated based on the technologies specified in a draft HEVC Range Extension standard [2]. Among them is the intra block copy (IntraBC), which performs block copying from the reconstructed pixels inside the current frame. Figure 1 illustrates the concept of its operation. A flag is present at CU level to signal its use.

The Screen Content Core Experiments 2 (SCCE2) [3] was set up to study the technique of the line-based intra copy [4][5]. This scheme relaxes the restriction that the basic unit for intra copy must be a PU and performs the intra copy operation in a line-based manner with the intention to find a better trade-off between the prediction accuracy and the required BV overhead. Three tests are conducted to evaluate the line-based intra copy with different settings. The first (Test B) divides a prediction unit horizontally or vertically into multiple 1-pixel-wide lines and then operates intra copying line-by-line from part of the reconstructed pixels in the current frame. The second (Test C) puts an additional constraint to Test B that the splitting manner should be aligned over all PUs within a CU. The last (Test D) enables Test C the capability to reuse the most-recently decoded BV, and therefore the BV difference need not be signaled.

Figure 1. A demonstration of the IntraBC mode, where the gray area represents the search range of a PU.

# Tools Description

## Test B: Line-based Intra Copy with 2-D BV and PU-level Splitting

The notion of the line-based intra block copy is to provide an estimate of pixel intensities based on the intra block copy specified in a draft HEVC Range Extension standard [2]. It divides a prediction unit equally into 1-pixel-wide rows or columns, termed as lines in general. A CU-level flag, line_bc_flag, is additionally sent to allow an adaptive switching between this mode and the usual intra block copy mode. For each CU coded by using this mode, another flag, row_splitting_flag, is sent at PU level to indicate whether the row-wise splitting or the column-wise splitting is in use. Pixels within each line are predicted in the same way as the intra block copy except that the basic processing unit is a line (Figure 2) instead of a block, and the search range is restricted to part of the reconstructed region, to the left or on the top of the lines, of the current frame. Pixels within the current CU are not allowed for reference.

Similar to IntraBC, the line-based intra copy also refers to the most-recently decoded BV for prediction. The only difference is that either the x or the y component of the predictor will be used for prediction, because there is always a zero component existing in its 2-D BV. A flag, bv_x_zero_flag, is thus present for each line to indicate its use, and only the BV difference of the non-zero component need be signaled.

Besides, a fast encoding option is enabled for this mode to be evaluated only at the smallest CU level, and the search range for the line-based intra copy is restricted within 2 CTU.

Figure 2. An example of BV search for line-based intra copy.

## Test C: Test B + Constrained PU-level Splitting

This test imposes an additional constraint on Test B, which requires all PUs within a CU to share the same form of line splitting, row-wise or column-wise (Figure 3). Thus, the syntax, row_splitting_flag, only need be signaled once per CU to indicate which mode is in use.

Figure 3. An example of 2-D BV search for lines with constrained PU-level splitting.

## Test D: Test C + Block Vector Merging

To reduce BV overhead for Test C, this test reuses the most-recently decoded BV for representing the BV of current line, and no BV difference needs to be signaled. A flag, bv_merge_flag, is present for each line to switch adaptively between BV merging and BV difference coding.

# Experimental Results

The following experiments were conducted using the HM-14.0+RExt-7.0+SCM-1.0 software and the screen content common test conditions [6] to collect the BD-rate, encoding and decoding time of the proposed techniques in the All-Intra, Random Access, Low Delay B configurations. Results are summarized in Table 1 and Table 2 for Test B, in Table 3 and Table 4 for Test C, and in Table 5 and Table 6 for Test D.

Table 1. BD-rate reduction and processing time ratio of Test B in lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.1% | -7.1% | -7.1% |
| RGB, text & graphics with motion,720p | -3.2% | -3.1% | -3.1% |
| RGB, mixed content, 1440p | -2.9% | -2.9% | -2.9% |
| RGB, mixed content, 1080p | -3.6% | -3.8% | -3.9% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.7% | -6.6% | -6.6% |
| YUV, text & graphics with motion,720p | -2.6% | -2.9% | -3.4% |
| YUV, mixed content, 1440p | -2.3% | -2.7% | -2.9% |
| YUV, mixed content, 1080p | -3.2% | -3.9% | -3.7% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.1% | -4.1% | -4.2% |
| RGB, text & graphics with motion,720p | -2.4% | -2.2% | -2.4% |
| RGB, mixed content, 1440p | -2.2% | -2.0% | -2.1% |
| RGB, mixed content, 1080p | -2.6% | -2.8% | -2.7% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.7% | -4.0% | -3.9% |
| YUV, text & graphics with motion,720p | -2.0% | -2.3% | -2.5% |
| YUV, mixed content, 1440p | -1.4% | -2.1% | -2.3% |
| YUV, mixed content, 1080p | -2.4% | -3.4% | -3.1% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.6% | -2.6% | -2.5% |
| RGB, text & graphics with motion,720p | -1.5% | -1.4% | -1.4% |
| RGB, mixed content, 1440p | -1.3% | -1.4% | -1.2% |
| RGB, mixed content, 1080p | -1.2% | -1.3% | -1.3% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.2% | -2.3% | -2.4% |
| YUV, text & graphics with motion,720p | -0.7% | -1.3% | -1.5% |
| YUV, mixed content, 1440p | -0.6% | -1.6% | -1.9% |
| YUV, mixed content, 1080p | -0.7% | -2.4% | -0.8% |
| YUV, Animation, 720p | 0.0% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. BD-rate reduction and processing time ratio of Test B in lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 6.6% | 6.6% | 4.3% | 10.4% |
| RGB, text & graphics with motion,720p | 1.6% | 1.8% | 0.5% | 3.3% |
| RGB, mixed content, 1440p | 0.9% | 1.0% | 0.6% | 1.4% |
| RGB, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.7% | 6.6% | 4.3% | 10.3% |
| YUV, text & graphics with motion,720p | 1.6% | 2.1% | 0.4% | 4.8% |
| YUV, mixed content, 1440p | 0.9% | 1.0% | 0.5% | 1.4% |
| YUV, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 1.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.0% | 4.1% | 1.7% | 7.6% |
| RGB, text & graphics with motion,720p | 0.4% | 1.0% | 0.3% | 2.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.9% | 4.2% | 2.0% | 7.7% |
| YUV, text & graphics with motion,720p | 0.4% | 1.3% | 0.3% | 4.1% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.7% | 3.0% | 1.3% | 5.0% |
| RGB, text & graphics with motion,720p | 0.3% | 0.7% | 0.2% | 1.9% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.5% | 3.1% | 1.5% | 5.3% |
| YUV, text & graphics with motion,720p | 0.3% | 0.9% | 0.2% | 2.7% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 3. BD-rate reduction and processing time ratio of Test C in lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.0% | -7.0% | -6.9% |
| RGB, text & graphics with motion,720p | -3.2% | -3.2% | -3.2% |
| RGB, mixed content, 1440p | -2.8% | -2.7% | -2.8% |
| RGB, mixed content, 1080p | -3.7% | -3.8% | -3.8% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.6% | -6.5% | -6.5% |
| YUV, text & graphics with motion,720p | -2.5% | -3.0% | -3.3% |
| YUV, mixed content, 1440p | -2.2% | -2.7% | -2.9% |
| YUV, mixed content, 1080p | -3.2% | -3.9% | -3.7% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.1% | -4.0% | -4.0% |
| RGB, text & graphics with motion,720p | -2.3% | -2.2% | -2.4% |
| RGB, mixed content, 1440p | -2.1% | -1.9% | -1.9% |
| RGB, mixed content, 1080p | -2.6% | -2.7% | -2.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.6% | -3.9% | -3.9% |
| YUV, text & graphics with motion,720p | -2.1% | -2.6% | -2.6% |
| YUV, mixed content, 1440p | -1.3% | -2.4% | -2.1% |
| YUV, mixed content, 1080p | -2.4% | -3.1% | -3.0% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 104% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.5% | -2.4% | -2.3% |
| RGB, text & graphics with motion,720p | -1.5% | -1.4% | -1.5% |
| RGB, mixed content, 1440p | -1.1% | -1.3% | -1.1% |
| RGB, mixed content, 1080p | -1.3% | -1.3% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.2% | -2.4% | -2.5% |
| YUV, text & graphics with motion,720p | -0.9% | -1.1% | -1.8% |
| YUV, mixed content, 1440p | -0.7% | -1.7% | -1.8% |
| YUV, mixed content, 1080p | -0.5% | -2.0% | -0.4% |
| YUV, Animation, 720p | -0.1% | -0.5% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 103% |  |  |

Table 4. BD-rate reduction and processing time ratio of Test C in lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 6.6% | 6.5% | 4.3% | 10.2% |
| RGB, text & graphics with motion,720p | 1.5% | 1.7% | 0.5% | 3.2% |
| RGB, mixed content, 1440p | 0.9% | 1.0% | 0.6% | 1.4% |
| RGB, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.6% | 6.5% | 4.3% | 9.9% |
| YUV, text & graphics with motion,720p | 1.6% | 2.1% | 0.4% | 4.6% |
| YUV, mixed content, 1440p | 0.9% | 1.0% | 0.5% | 1.4% |
| YUV, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 1.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 121% |  |  |  |
| Dec Time[%] | 99% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.9% | 4.0% | 1.7% | 7.4% |
| RGB, text & graphics with motion,720p | 0.4% | 1.0% | 0.3% | 2.7% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.9% | 4.1% | 2.0% | 7.5% |
| YUV, text & graphics with motion,720p | 0.4% | 1.3% | 0.3% | 3.9% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 104% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.6% | 3.0% | 1.3% | 4.9% |
| RGB, text & graphics with motion,720p | 0.3% | 0.7% | 0.2% | 1.8% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.5% | 3.1% | 1.6% | 5.1% |
| YUV, text & graphics with motion,720p | 0.3% | 0.9% | 0.2% | 2.6% |
| YUV, mixed content, 1440p | 0.1% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

Table 5. BD-rate reduction and processing time ratio of Test D in lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.9% | -6.9% | -6.9% |
| RGB, text & graphics with motion,720p | -3.2% | -3.2% | -3.3% |
| RGB, mixed content, 1440p | -2.7% | -2.6% | -2.7% |
| RGB, mixed content, 1080p | -3.6% | -3.6% | -3.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.6% | -6.6% | -6.5% |
| YUV, text & graphics with motion,720p | -2.7% | -3.0% | -3.3% |
| YUV, mixed content, 1440p | -2.1% | -2.6% | -2.8% |
| YUV, mixed content, 1080p | -3.0% | -3.7% | -3.6% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  |
| Dec Time[%] | 103% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.9% | -3.9% | -3.9% |
| RGB, text & graphics with motion,720p | -2.4% | -2.3% | -2.3% |
| RGB, mixed content, 1440p | -1.9% | -1.8% | -1.9% |
| RGB, mixed content, 1080p | -2.6% | -2.6% | -2.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.6% | -3.8% | -3.8% |
| YUV, text & graphics with motion,720p | -2.0% | -2.4% | -2.5% |
| YUV, mixed content, 1440p | -1.3% | -2.1% | -2.2% |
| YUV, mixed content, 1080p | -2.3% | -3.1% | -3.1% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.3% | -2.1% |
| RGB, text & graphics with motion,720p | -1.6% | -1.4% | -1.5% |
| RGB, mixed content, 1440p | -1.1% | -1.1% | -1.0% |
| RGB, mixed content, 1080p | -1.2% | -1.3% | -1.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.1% | -2.3% | -2.3% |
| YUV, text & graphics with motion,720p | -0.8% | -1.2% | -1.8% |
| YUV, mixed content, 1440p | -0.6% | -1.4% | -1.5% |
| YUV, mixed content, 1080p | -0.6% | -2.2% | -0.7% |
| YUV, Animation, 720p | -0.1% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |

Table 6. BD-rate reduction and processing time ratio of Test D in lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 7.3% | 7.3% | 4.9% | 11.2% |
| RGB, text & graphics with motion,720p | 1.8% | 2.0% | 0.5% | 3.8% |
| RGB, mixed content, 1440p | 1.0% | 1.1% | 0.6% | 1.6% |
| RGB, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 1.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.5% | 7.4% | 5.0% | 11.1% |
| YUV, text & graphics with motion,720p | 1.9% | 2.4% | 0.5% | 5.5% |
| YUV, mixed content, 1440p | 1.0% | 1.1% | 0.5% | 1.6% |
| YUV, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 1.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 117% |  |  |  |
| Dec Time[%] | 98% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.4% | 4.6% | 2.0% | 8.3% |
| RGB, text & graphics with motion,720p | 0.4% | 1.2% | 0.3% | 3.2% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.3% | 4.7% | 2.3% | 8.5% |
| YUV, text & graphics with motion,720p | 0.5% | 1.5% | 0.3% | 4.7% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.1% | 3.4% | 1.4% | 5.6% |
| RGB, text & graphics with motion,720p | 0.3% | 0.8% | 0.3% | 2.1% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.0% | 3.6% | 1.8% | 5.9% |
| YUV, text & graphics with motion,720p | 0.3% | 1.0% | 0.2% | 2.9% |
| YUV, mixed content, 1440p | 0.1% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Conclusion

In this contribution, the line-based intra copy is proposed and tested. Three tests of line-based intra copy achieve similar coding gains which are up to 7.1% in the All-Intra configuration and introduce roughly 15% encoding time increase. According to the interesting results, we would recommend the group to consider further studying this technique in a CE.

# References

- “Joint Call for Proposals for Coding of Screen Content,” ISO/IEC JTC1/SC29/WG11, MPEG2014/N14175, San Jose, Jan. 2014

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan and T. Suzuki, “HEVC Range Extensions Draft 6,” JCTVC-P1005, San Jose, Jan. 2014.

- C. C. Chen, X. Xu, and L. Zhang, “HEVC Screen Content Coding Core Experiment 2 (SCCE2): Line-based Intra Copy,” JCTVC-Q1122, Valencia, Mar. 2014.

- C. C. Chen, T. S. Chang, R. L. Liao, W. H. Peng, H. M. Hang, C. L. Lin, and F. D. Jou, "AHG8: Line-based Intra Block Copy," JCTVC-O0205, Geneva, Oct. 2013.

- C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, Y.-J. Chang, C.-H. Hung, C.-C. Lin, J.-S. Tu, E.-C. Ke, J.-Y. Kao, C.-L. Lin, F.-D. Jou, and F.-C. Chen, "Description of screen content coding technology proposal by NCTU and ITRI International," JCTVC-Q0032, Valencia, Mar. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common Conditions for Screen Content Coding Tests,” JCTVC-Q1015, Valencia, Mar. 2014.

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).