| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0132 |
| --- | --- |

| Title: | Non-SCCE2: Line-based Intra Copy with Unconstrained 2-D BV Coding for Test C |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, Tsui-Shan Chang, Ru-Ling Liao, Che-Wei Kuo, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
|  | Chun-Lung Lin, 3Fan-Di Jou 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan |  | +886-3-582-0100 3Fandi@itri.org.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

In SCCE2 Test C, a line-based intra copy with 2-D block vector (BV) and constrained PU-level splitting is proposed (JCTVC-R0131). Reportedly, its syntax only supports BVs of each line pointing to a portion of the reconstructed pixels, to the left or on the top of the lines, of the current picture. In this contribution, the constraint on 2-D BV is relaxed by aligning the BV syntax with that of the intra block copy. Experimental results with two different encoding-only options (2-CTU or full-frame search) are reported respectively as follows:

- (Subtest 1) 2-CTU search:

  - RGB, text & graphics with motion, 1080p	: (AI) -9.4%,	(RA) -5.2%,	(LB) -2.8%;

  - RGB, text & graphics with motion, 720p	: (AI) -4.4%, 	(RA) -3.5%, 	(LB) -2.1%;

  - RGB, mixed content, 1440p			: (AI) -2.7%, 	(RA) -2.1%, 	(LB) -1.2%;

  - RGB, mixed content, 1080p			: (AI) -3.6%, 	(RA) -2.7%, 	(LB) -1.2%;

  - YUV, text & graphics with motion, 1080p	: (AI) -8.8%, 	(RA) -4.9%, 	(LB) -2.5%;

  - YUV, text & graphics with motion, 720p	: (AI) -3.3%, 	(RA) -2.7%, 	(LB) -1.1%;

  - YUV, mixed content, 1440p			: (AI) -2.1%, 	(RA) -1.3%, 	(LB) -0.7%;

  - YUV, mixed content, 1080p			: (AI) -3.0%, 	(RA) -2.2%, 	(LB) -0.4%;

  - Encoding time				: (AI) 127%, 	(RA) 103%, 	(LB) 102%;

  - Decoding time				: (AI) 99%, 	(RA) 101%, 	(LB) 102%;

- (Subtest 2) full-frame search:

  - RGB, text & graphics with motion, 1080p	: (AI) -12.9%,	(RA) -7.0%, 	(LB) -3.8%;

  - RGB, text & graphics with motion, 720p	: (AI) -6.0%, 	(RA) -4.6%, 	(LB) -2.5%;

  - RGB, mixed content, 1440p			: (AI) -3.3%, 	(RA) -2.3%, 	(LB) -1.5%;

  - RGB, mixed content, 1080p			: (AI) -6.0%, 	(RA) -4.2%, 	(LB) -2.0%;

  - YUV, text & graphics with motion, 1080p	: (AI) -12.0%,	(RA) -6.3%, 	(LB) -3.2%;

  - YUV, text & graphics with motion, 720p 	: (AI) -4.7%, 	(RA) -3.8%, 	(LB) -1.6%;

  - YUV, mixed content, 1440p			: (AI) -2.7%, 	(RA) -1.7%, 	(LB) -0.6%;

  - YUV, mixed content, 1080p			: (AI) -5.2%, 	(RA) -4.0%, 	(LB) -1.4%;

  - Encoding time				: (AI) 126%, 	(RA) 104%, 	(LB) 103%;

  - Decoding time				: (AI) 99%, 	(RA) 102%, 	(LB) 102%.

# Introduction

The line-based intra copy, described in SCCE2 Test C [1], first splits a block equally into multiple row-wise or column-wise lines with 1 pixel in width, and then performs the intra copying operation line-by-line from the reconstructed region, to the left or on the top of the lines but not full frame (Figure 1), inside the current frame. Due to this additional constraint, one component of its 2-D BV is always equal to zero. Accordingly, additional logics for BV coding instead of that of the intra block copy are required in Test C to support the coding of such constrained 2-D BV. Modifications will be made in this contribution to report additional results with removing such constraint on BV coding.

Figure 1. An example of line-based intra copy with constrained BV signaling.

# Tool Description

In this contribution, the constraint on 2-D BV is relaxed by modifying the BV syntax of the line-based intra copy to align with that of the intra block copy. Therefore, the BV prediction and BV difference coding of the intra block copy can be fully re-useable for the BV coding of the line-based intra copy. With this modification, the BVs can now be searched over the entire reconstructed region within the current frame for complete 2-D searching (Figure 2). As compared with Test C, no extra hardware overhead is required for supporting this new mode.

Figure 2. An example of line-based intra copy with relaxed BV signaling.

# Experimental Results

To strike a better trade-off between complexity and performance, two fast encoding strategies (2-CTU or full-frame search) are applied independently for the line-based intra copy to prevent the encoding from exhaustive search over the entire frame. Experimental results were conducted using the HM-14.0+RExt-7.0+SCM-1.0 software [3] and the screen content common test conditions [4] to obtain the coding results of the proposed techniques.

## Subtest 1: Test C with Unconstrained 2-D BV and 2-CTU Search

In this subtest, a fast encoding option is enabled for the proposed mode to be evaluated only at the smallest CU level, and fast searching process is described in the following steps:

- searching horizontally and vertically within the reconstructed region using the 2-CTU setting;

- continuing the search at pixel positions within a 16x16 region which is centered at the one pointed by the BV and is inside the 2 CTUs;

- searching the pixel positions indicated by using a hash-based technique [2] with the modification that only the most significant 4 bits of each sample are used for computing hash keys.

## Subtest 2: Test C with Unconstrained 2-D BV and Full-frame Search

In this subtest, the other fast encoding option is enabled for the proposed mode to be evaluated only at the smallest CU level, and fast searching process is described in the following steps:

- searching horizontally and vertically within the reconstructed region using the 2-CTU setting;

- searching the pixel positions indicated by using a hash-based technique [2].

Table 1. BD-rate reduction and processing time ratio of Subtest 1 in lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -9.4% | -9.3% | -9.2% |
| RGB, text & graphics with motion,720p | -4.4% | -4.2% | -4.2% |
| RGB, mixed content, 1440p | -2.7% | -2.6% | -2.6% |
| RGB, mixed content, 1080p | -3.6% | -3.6% | -3.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -8.8% | -8.7% | -8.6% |
| YUV, text & graphics with motion,720p | -3.3% | -3.8% | -4.3% |
| YUV, mixed content, 1440p | -2.1% | -2.8% | -3.0% |
| YUV, mixed content, 1080p | -3.0% | -3.8% | -3.7% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.2% | -5.2% | -5.0% |
| RGB, text & graphics with motion,720p | -3.5% | -3.0% | -3.2% |
| RGB, mixed content, 1440p | -2.1% | -1.9% | -2.0% |
| RGB, mixed content, 1080p | -2.7% | -2.7% | -2.9% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.9% | -5.2% | -5.1% |
| YUV, text & graphics with motion,720p | -2.7% | -3.3% | -3.6% |
| YUV, mixed content, 1440p | -1.3% | -2.7% | -2.5% |
| YUV, mixed content, 1080p | -2.2% | -3.6% | -3.2% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.8% | -2.8% | -2.6% |
| RGB, text & graphics with motion,720p | -2.1% | -1.6% | -1.9% |
| RGB, mixed content, 1440p | -1.2% | -1.2% | -1.1% |
| RGB, mixed content, 1080p | -1.2% | -0.8% | -1.3% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -2.7% | -2.8% |
| YUV, text & graphics with motion,720p | -1.1% | -1.5% | -1.8% |
| YUV, mixed content, 1440p | -0.7% | -1.5% | -1.5% |
| YUV, mixed content, 1080p | -0.4% | -1.2% | -1.2% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. BD-rate reduction and processing time ratio of Subtest 1 in lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 11.0% | 10.9% | 7.8% | 16.1% |
| RGB, text & graphics with motion,720p | 5.3% | 4.8% | 0.9% | 8.1% |
| RGB, mixed content, 1440p | 1.0% | 1.2% | 0.5% | 1.8% |
| RGB, mixed content, 1080p | 1.6% | 1.6% | 1.6% | 1.6% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 13.2% | 13.2% | 8.2% | 19.0% |
| YUV, text & graphics with motion,720p | 5.4% | 5.6% | 0.8% | 10.1% |
| YUV, mixed content, 1440p | 1.2% | 1.3% | 0.5% | 2.1% |
| YUV, mixed content, 1080p | 1.7% | 1.7% | 1.7% | 1.7% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 129% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 5.6% | 7.0% | 3.6% | 11.6% |
| RGB, text & graphics with motion,720p | 1.1% | 2.5% | 0.5% | 5.4% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.2% | 0.3% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.1% | 8.6% | 5.8% | 14.1% |
| YUV, text & graphics with motion,720p | 1.1% | 3.3% | 0.5% | 8.7% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 5.3% | 5.6% | 3.0% | 8.2% |
| RGB, text & graphics with motion,720p | 0.8% | 1.8% | 0.4% | 4.0% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 5.5% | 6.9% | 5.3% | 9.9% |
| YUV, text & graphics with motion,720p | 0.7% | 2.3% | 0.4% | 6.0% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

Table 3. BD-rate reduction and processing time ratio of Subtest 2 in lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -12.9% | -12.8% | -12.8% |
| RGB, text & graphics with motion,720p | -6.0% | -6.0% | -6.0% |
| RGB, mixed content, 1440p | -3.3% | -3.3% | -3.4% |
| RGB, mixed content, 1080p | -6.0% | -6.1% | -6.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -12.0% | -11.8% | -11.7% |
| YUV, text & graphics with motion,720p | -4.7% | -5.0% | -5.5% |
| YUV, mixed content, 1440p | -2.7% | -3.1% | -3.3% |
| YUV, mixed content, 1080p | -5.2% | -5.8% | -5.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 126% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -7.0% | -6.9% | -6.9% |
| RGB, text & graphics with motion,720p | -4.6% | -4.4% | -4.6% |
| RGB, mixed content, 1440p | -2.3% | -2.1% | -2.2% |
| RGB, mixed content, 1080p | -4.2% | -4.2% | -4.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.3% | -6.4% | -6.4% |
| YUV, text & graphics with motion,720p | -3.8% | -4.1% | -4.4% |
| YUV, mixed content, 1440p | -1.7% | -2.4% | -2.6% |
| YUV, mixed content, 1080p | -4.0% | -4.5% | -4.5% |
| YUV, Animation, 720p | 0.0% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 102% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.8% | -3.7% | -3.6% |
| RGB, text & graphics with motion,720p | -2.5% | -2.1% | -2.3% |
| RGB, mixed content, 1440p | -1.5% | -1.5% | -1.3% |
| RGB, mixed content, 1080p | -2.0% | -1.7% | -1.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.2% | -3.5% | -3.3% |
| YUV, text & graphics with motion,720p | -1.6% | -1.7% | -2.1% |
| YUV, mixed content, 1440p | -0.6% | -1.4% | -1.4% |
| YUV, mixed content, 1080p | -1.4% | -2.0% | -1.4% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |

Table 4. BD-rate reduction and processing time ratio of Subtest 2 in lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 15.8% | 15.5% | 9.6% | 24.7% |
| RGB, text & graphics with motion,720p | 5.5% | 5.8% | 0.7% | 10.2% |
| RGB, mixed content, 1440p | 2.2% | 2.2% | 1.9% | 2.6% |
| RGB, mixed content, 1080p | 3.4% | 3.4% | 3.4% | 3.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 18.1% | 17.8% | 12.0% | 28.8% |
| YUV, text & graphics with motion,720p | 5.9% | 7.1% | 0.7% | 15.7% |
| YUV, mixed content, 1440p | 2.4% | 2.5% | 1.8% | 3.1% |
| YUV, mixed content, 1080p | 3.6% | 3.6% | 3.6% | 3.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 126% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 9.4% | 10.4% | 3.7% | 17.4% |
| RGB, text & graphics with motion,720p | 1.0% | 3.1% | 0.4% | 8.5% |
| RGB, mixed content, 1440p | 0.4% | 0.4% | 0.2% | 0.6% |
| RGB, mixed content, 1080p | 0.5% | 0.5% | 0.5% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 9.7% | 12.0% | 5.3% | 20.7% |
| YUV, text & graphics with motion,720p | 1.0% | 4.3% | 0.4% | 13.3% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.3% | 0.6% |
| YUV, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 9.0% | 8.2% | 3.0% | 11.7% |
| RGB, text & graphics with motion,720p | 0.6% | 2.1% | 0.3% | 5.8% |
| RGB, mixed content, 1440p | 0.2% | 0.3% | 0.1% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 9.1% | 9.5% | 4.5% | 14.2% |
| YUV, text & graphics with motion,720p | 0.6% | 2.8% | 0.3% | 8.8% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.4% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

# Conclusion

In this contribution, THE line-based intra copy with relaxed BV signaling is proposed and tested. With the encoder option of 2-CTU search area, the BD-rate saving is up to 9.4% in the All-Intra configuration. Furthermore, 12.9% coding gain is observed when extending 2-CTU search area to full frame. According to the interesting gains, we would recommend the group to consider further studying this mode in a CE.

# References

- C. C. Chen, X. Xu, and L. Zhang, “HEVC Screen Content Coding Core Experiment 2 (SCCE2): Line-based Intra Copy,” JCTVC-Q1122, Valencia, Mar. 2014.

- B. Li, J. Xu, F Wu, X. Guo, and G. J. Sullivan, “Description of screen content coding technology proposal by Microsoft,” JCTVC-Q0035, Valencia, Mar. 2014.

- JCT-VC, “HM-14.0+RExt-7.0+SCM-1.0,”  https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/ HM-14.0+RExt-7.0+SCM-1.0/

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, " Common conditions for screen content coding tests," JCTVC-Q1015, Valencia, Mar. 2014.

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).