The proposed modifications are based on HEVC Range Extensions text specification: Draft 6 (JCTVC-P1005). The inserted parts are marked in yellow. The deleted parts are marked in pink.

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| if (refList == 2) |  |
| bvd_coding(x0,y0) |  |
| else { |  |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |

#### 7.3.8.x Block vector difference syntax

| bvd_coding( x0, y0) { | Descriptor |
| --- | --- |
| abs_bvd_msb_idx_p1[ 0] | ae(v) |
| abs_ bvd_msb_idx_p1[1] | ae(v) |
| if(abs_bvd_msb_idx_p1 [0] > 0 ) { |  |
| if(abs_bvd_msb_idx_p1 [0] > 1) |  |
| abs_bvd_refinement_bits [0] | ae(v) |
| bvd_sign_flag [0] | ae(v) |
| } |  |
| if(abs_bvd_msb_idx_p1 [1]>0 ) { |  |
| if(abs_bvd_msb_idx_p1 [1 ] > 1) |  |
| abs_bvd_refinement_bits [1] | ae(v) |
| bvd_sign_flag [ 1] | ae(v) |
| } |  |
| } |  |

#### Motion vector difference semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus2[ compIdx ] plus 2 specifies the absolute value of a motion vector component difference.

When abs_mvd_minus2[ compIdx ] is not present, it is inferred to be equal to −1.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The motion vector difference lMvd[ compIdx ] for compIdx = 0..1 is derived as follows:

lMvd[ compIdx ] = abs_mvd_greater0_flag[ compIdx ] *
		( abs_mvd_minus2[ compIdx ] + 2 ) * 	( 1 − 2 * mvd_sign_flag[ compIdx ] ) 	(7-1)

The variable MvdLX[ x0 ][ y0 ][ compIdx ], with X being 0 or 1, specifies the difference between a list X vector component to be used and its prediction. The value of MvdLX[ x0 ][ y0 ][ compIdx ] shall be in the range of −215 to 215 − 1, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ][ compIdx ] shall be in the range of −128 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvdIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

#### 7.4.9.x Block vector difference semantics

abs_bvd_msb_idx_p1[ compIdx] minus 1 specifies the most significant bit index of the absolute value of a block vector component difference in a binary representation. abs_bvd_msb_idx_p1[ compIdx] equal to 0 indicates the absolute value of a block vector component difference is equal to 0.

abs_bvd_refinement_bits [compIdx] specifies the refinement bits of the absolute value of a block vector component difference in a binary representation. When abs_bvd_refinement_bits [compIdx] is not present, it is inferred to be equal to 0.

bvd_sign_flag [compIdx] specifies the sign of a block vector component difference as follows:

- If bvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

- Otherwise (bvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

The block vector difference Bvd[ compIdx ] for compIdx = 0..1 is derived as follows:

- If abs_bvd_msb_idx_p1[ compIdx] is equal to 0,

Bvd[ compIdx ] = 0.

- Otherwise (abs_bvd_msb_idx_p1[ compIdx] is greater than 0),

Bvd[ compIdx ] =
		(  ( 1 << ( abs_bvd_msb_idx_p1[compIdx] – 1 ) )   +  abs_bvd_refinement_bits[compIdx] ) * 	( 1 − 2 * bvd_sign_flag[ compIdx ] )

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ][ compIdx ] shall be in the range of  −215 to 215 − 1, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. BvdIntra[ x0 ][ y0 ][ compIdx ] is set equal to Bvd[ compIdx ] for compIdx = 0..1.

| Table 9-1 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| slice_segment_data( ) | end_of_slice_segment_flag | FL | cMax = 1 |
|  | end_of_sub_stream_one_bit | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | FL | cMax = 1 |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_bc_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| prediction_unit( ) | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | Error: Reference source not found | nPbW, nPbH |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cr[ ][ ][ ] | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| bvd_coding ( ) | abs_bvd_msb_idx_pl[ ] | TR | cMax = 16 |
|  | abs_bvd_refinement_bits [ ] | FL | cMax = abs_bvd_msb_idx_pl-1 |
|  | bvd_sign_flag [ ] | FL | cMax = 1 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
|  | cu_chroma_qp_adjustment_flag | FL | cMax = 1 |
|  | cu_chroma_qp_adjustment_idc | TR | cMax = chroma_qp_adjustment_table_size_minus1, cRiceParam = 0 |
| cross_comp_pred( ) | log2_res_scale_abs_plus1 | TR | cMax = 4, cRiceParam = 0 |
|  | res_scale_sign_flag | FL | cMax = 1 |
| residual_coding( ) | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | explicit_rdpcm_flag[ ][ ][ ] | FL | cMax = 1 |
|  | explicit_rdpcm_dir_flag[ ][ ][ ] | FL | cMax = 1 |
|  | last_sig_coeff_x_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater1_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater2_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_remaining[ ] | Error: Reference source not found | current sub-block scan index i, baseLevel |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |